3GPP TSG-RAN3 #111-e	R3-211095

25th January – 4th February 2021 Online

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0163 | rev | 3 | Current version: | 15.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | RRC Re-establishment and inter-vendor CU operation CR 38.401 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Verizon Wireless, CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2021-01-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Currently in TS 38.401 in RRC Connection reestablishment the following note is cited. NOTE: If UE accesses from a gNB-DU other than the original one, gNB-CU should trigger UE Context Setup procedure towards this new gNB-DU. Currently, the only case that is mentioned is when the UE accesses from a gNB-DU other than the original one. The question we would like to raise is what happens in the case that the UE access the same gNB-DU but still the gNB-DU was not able to retrieve the previous UE context, how other-vendor CU will operate in this case? Since the gNB-DU was not able to access the context associated to the old F1AP IDs, hence it has issued a UE context release for that context. At the same time, a new F1 signalling connection has been established for the new F1AP IDs. This signalling connection leads to a new UE context setup. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that if UE access the same gNB-DU, then other vendor CU will trigger UE Context Setup procedure towards the same gNB-DU. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclarities regarding inter-vendor operation |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev.3 - Resubsmission |  |  |  |  |  |  |  |  |

<<<<<< NEXT CHANGE >>>>>>

## 8.7	RRC connection reestablishment

This procedure is used for the case that UE tries to reestablish the RRC connection, as shown in Figure 8.7-1.

Figure 8.7-1: RRC connection reestablishment procedure

1.	UE sends preamble to the gNB-DU.

2.	The gNB-DU allocates new C-RNTI and responds UE with RAR.

3.	The UE sends RRCReestablishmentRequest message to the gNB-DU, which contains old C-RNTI and old PCI.

4.	The gNB-DU includes the RRC message and, if the UE is admitted, the corresponding low layer configuration for the UE in the INITIAL UL RRC MESSAGE TRANSFER message and transfers to the gNB-CU. The INITIAL UL RRC MESSAGE TRANSFER message should include C-RNTI.

5.	The gNB-CU includes RRCReestablishment message and transfers to the gNB-DU. If the UE requests to re-establish RRC connection in the last serving gNB-DU, the DL RRC MESSAGE TRANSFER message shall include old gNB-DU UE F1AP ID.

6.	The gNB-DU retrieves UE context based on old gNB-DU UE F1AP ID, replaces old C-RNTI/PCI with new C-RNTI/PCI. It sends RRCReestablishment message to UE.

7-8.	The UE sends RRCReestablishmentComplete message to the gNB-DU. The gNB-DU encapsulates the RRC message in UL RRC MESSAGE TRANSFER message and sends to gNB-CU.

9-10.	The gNB-CU triggers UE context modification procedure by sending UE CONTEXT MODIFICIATION REQUEST, which may include DRBs to be modified and released list. The gNB-DU responses with UE CONTEXT MODIFICATION RESPONSE message.

9'-10'.	The gNB-DU triggers UE context modification procedure by sending UE CONTEXT MODIFICIATION REQUIRED, which may include DRBs to be modified and released list. The gNB-CU responses with UE CONTEXT MODIFICATION CONFIRM message.

NOTE:	Here it is assumed that UE accesses from the original gNB-DU where the UE contexts are available for that UE, and either step 9-10 or step 9’ and 10’ may exist or both could be skipped.

NOTE:	If UE accesses from a gNB-DU other than the original one, or if the original gNB-DU is not capable of retrieving the UE context, gNB-CU should trigger UE Context Setup procedure towards this new gNB-DU or towards the original gNB-DU.

11-12.	The gNB-CU includes RRCReconfiguration message into DL RRC MESSAGE TRANSFER message and transfers to the gNB-DU. The gNB-DU forwards it to the UE.

13-14.	The UE sends RRCReconfigurationComplete message to the gNB-DU, and gNB-DU forwards it to the gNB-CU.

<<<<<< END OF CHANGES >>>>>>