3GPP TSG-RAN WG3 Meeting #111-e	R3-211214

E-meeting, 25 Jan -5 Feb 2021

Agenda item:	10.2.1.5

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP to TS 36.423, NR_ENDC_SON_MDT_enh-Core) Further clarification of the TNL load

Document for:	Endorsement

# 1	Introduction

This document proposes text for a BL CR to X2AP. The proposed clarification relates to the issue of the input for the TNL load information.

# 2	Text proposal

*** First change, omitted text not changed ***

#### 8.7.21.2	Successful Operation

Figure 8.7.21.2-1: EN-DC Resource Status Reporting Initiation, successful operation

The procedure is initiated with an EN-DC RESOURCE STATUS REQUEST message sent from the eNB to the en-gNB to start a measurement, stop a measurement, add cells to report for a measurement.

If the Report Characteristics IE is included in the EN-DC RESOURCE STATUS REQUEST message and indicates cell specific measurements, the Cell To Report EN-DC List IE shall be included.

Upon receipt of the EN-DC RESOURCE STATUS REQUEST message, the en-gNB:

-	shall initiate the requested measurement according to the parameters given in the request in case the Registration Request IE set to "start"; or

-	shall stop all cells measurements and terminate the reporting in case the Registration Request IE is set to "stop"; or

-	shall add cells indicated in the Cell To Report EN-DC List IE list to the measurements initiated before for the given measurement IDs, in case the Registration Request IE is set to "add". If measurements are already initiated for a cell indicated in the Cell To Report EN-DC List IE, this information shall be ignored.

The en-gNB shall send an EN-DC RESOURCE STATUS RESPONSE message to the eNB to indicate that all of the requested measurement objects the measurement can be initiated.

Interaction with other procedures

When starting a measurement, the Report Characteristics IE in the EN-DC RESOURCE STATUS REQUEST indicates the type of objects en-gNB shall perform measurements on. For each cell, the en-gNB shall include in the EN-DC RESOURCE STATUS UPDATE message:

-	the Radio Resource Status IE, if the first bit, "PRB Periodic" of the Report Characteristics IE included in the EN-DC RESOURCE STATUS REQUEST message is set to "1". If the cell for which Radio Resource Status IE is requested to be reported supports more than one SSB, the Radio Resource Status IE for such cell shall include the SSB Area Radio Resource Status Item IE for all SSB areas supported by the cell. If the SSB To Report List IE is included for a cell, the Radio Resource Status IE for such cell shall include the requested SSB Area Radio Resource Status List IE.

-	the TNL Capacity Indicator IE, if the second bit, "TNL Capacity Ind Periodic" of the Report Characteristics IE included in the EN-DC RESOURCE STATUS REQUEST message is set to "1". The received TNL Capacity Indicator IE represents the lowest TNL capacity available for the cell, only taking into account interfaces providing user plane transport.

-	the Composite Available Capacity Group IE, if the third bit, "Composite Available Capacity Periodic" of the Report Characteristics IE included in the EN-DC RESOURCE STATUS REQUEST message is set to "1". If Cell Capacity Class Value IE is included within the Composite Available Capacity Group IE, this IE is used to assign weights to the available capacity indicated in the Capacity Value IE. If the cell for which Composite Available Capacity Group IE is requested to be reported supports more than one SSB, and if the SSB To Report List IE is included for a cell, the Composite Available Capacity Group IE for such cell shall include the requested SSB Area Capacity Value List IE, providing the SSB area capacity with respect to the Cell Capacity Class Value.

If the Reporting Periodicity IE in the EN-DC RESOURCE STATUS REQUEST is present, this indicates the periodicity for the reporting of periodic measurements. The en-gNB shall only report more than once if the Reporting Periodicity IE is included.

*** End of changes, remaining text not changed ***