3GPP TSG-RAN3 Meeting #111-e	R3-211231

E-meeting, 25 Jan – 5 Feb 2021

Title: 	Evaluation of slicing solutions

Source: 	Huawei

Agenda item:	17.2

Document Type:	Discussion and Decision

1. TP for TR 38.832

6.3 Solution evaluation

The evaluation criteria are as follows:

- RAN impact

The point here is to analyze RAN impact of the solution (standardization and node behaviour), for example what signalling procedures may be affected and at what extent.

- Core impact

The point here is to analyze Core impact of the solution (standardization and node behaviour), for example what signalling procedures may be affected and at what extent. Such analysis needs to be carried out together with SA2 and CT groups.

- OAM impact

The point here is to analyze operator and maintenance effort, for example how many network elements (e,g. gNB, NF) should be configured and managed by OAM. Such analysis may need to involve SA5.

- UE Impact

This is to analyse the impact at NAS and AS level on the UE. Such analysis needs to be carried out together with RAN2, SA2 and CT groups.

- Effectiveness of solution

The point here is to analyse the effectiveness after applying the solution, for example the UE’s service experience after applying the solution.

- Applicable scenarios

The point here is to list the applicable scenarios which the corresponding solution targets.

Table 6.3-1: Evaluation of the solutions

| Criteria Solution |  | RAN impact | Core impact | OAM impact | UE impact | Effectiveness | Applicable scenarios |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 6.2.1: Re-mapping Policy in target NG-RAN node | Configuration in target NG-RAN node | RAN is configured with re-mapping policy from the OAM. FFS if RAN needs to signal the slice remapping decision to CN. | CN is configured with re-mapping policy from the OAM. FFS if the CN needs to be notified in case of any slice-remapping, e.g. for charging purpose. CN reconfigures UE with NAS signalling to associate an ongoing PDU Session to a new S-NSSAI. Pending SA2 decisions. How the slice remapping is done in 5GC is pending to SA2. | OAM configures slice re-mapping policy to the NG-RAN, CN (if verification is needed). | UE needs to be reconfigured at NAS level to associate an ongoing PDU Session to a new S-NSSAI. Pending SA2 decisions. | Solution at the cost of CN, OAM, RAN and UE impact | 1, 2, 3, 4, 5, 6 |
|  | Signalled from 5GC/source RAN node | RAN is signalled with the remapping policy from CN/the source RAN node. FFS if RAN needs to signal the slice remapping decision to CN. | CN is configured with remapping policy from the OAM, and signals the re-mapping policy to the NG-RAN. FFS if the CN needs to be notified in case of any slice-remapping, e.g. for charging purpose. CN reconfigures UE with NAS signalling to associate an ongoing PDU Session to a new S-NSSAI. Pending SA2 decisions. How the slice remapping is done in 5GC is pending to SA2. | OAM configures slice re-mapping policy to the CN. | UE needs to be reconfigured at NAS level to associate an ongoing PDU Session to a new S-NSSAI. Pending SA2 decisions. | Solution at the cost of CN, OAM, RAN and UE impact | 1, 2, 3, 4, 5, 6 |
| 6.2.3: Configuration based Solution |  | RAN is configured with re-mapping policy from the OAM. RAN may possibly signal the RAN-internal slice resource change to CN. | FFS if the CN needs to be notified in case of any RAN-internal slice resource change e.g., for charging purpose. | OAM configures slice resource policy to the NG-RAN. Pending SA5 feedback | No impact | Simple and effective solution at the main cost of the OAM impact. | 1, 3, 5, 6. |
| 6.2.4: Candidate solutions with/without CN involvement | Solution with CN involvement | Same as 6.2.1: Signalled from 5GC/source RAN node | Same as 6.2.1: Signalled from 5GC/source RAN node | Same as 6.2.1: : Signalled from 5GC/source RAN node | Same as 6.2.1: Signalled from 5GC/source RAN node | Same as 6.2.1: Signalled from 5GC/source RAN node | Same as 6.2.1: Signalled from 5GC/source RAN node |
|  | Solution without CN involvement | RAN is configured with re-mapping policy from the OAM. New functionality to support semi-handover case. New behaviour in new gNB (allow usage by non-supported slice). Requires Xn support from inside old RA to any node inside new RA (unless continuity is broken later) | New functionality to support the new handover case, where the UE is connected to target but source maintains UE signalling connection with CN. | OAM configures slice re-mapping policy to the NG-RAN. | New functionality to support the new handover case, where the UE is connected to target but source maintains UE signalling connection with CN. Pending SA2 decisions. | Solution at the cost of CN, OAM, RAN and UE impact. | 2, 4 |
| 6.2.5: Slice resource re-partitioning |  | RAN is configured with re-mapping policy from the OAM. RAN may possibly signal the RAN-internal slice resource change to CN | FFS if the CN needs to be notified in case of any RAN-internal slice resource change e.g., for charging purpose. | OAM configures slice resource re-mapping policy to the NG-RAN. Pending SA5 feedback | No impact | Simple and effective solution at the main cost of the OAM impact | 1, 3, 5, 6 |
| 6.2.6: Multi-carrier radio resource sharing |  | No impact. | No impact | No impact | No impact | Simple and effective solution. It requires the same slice coverage across different frequencies. | 1, 3, 5, 6 |
| 6.2.7: 5GC Solution based on SSC-mode 3 |  | RAN is signalled with the remapping policy from CN/the source RAN node if this option is used. It requires support of updated “SSC-mode 3”, e.g., the target node needs to temporarily accept the PDU session even if slice is not supported in the cell. | CN is configured with remapping policy from the OAM, and signals the re-mapping policy to the NG-RAN if this option is used. It requires modification of the “SSC mode 3” procedure in CN. Pending SA2 decisions. . | OAM configures slice re-mapping policy to the RAN if this option is used. | The update of “SSC-mode 3”. Pending SA2 decisions. | Solution with OAM, CN, RAN and UE impact | 2, 4 |
| 6.2.8: Slice Remapping decision in 5GC |  | No impact | CN is configured with remapping policy from the OAM. New intra-CN procedure is needed to change the slice for an ongoing PDU session. Pending SA2 decisions. | OAM configures slice re-mapping policy to the CN. | UE needs to be reconfigured at NAS level to associate an ongoing PDU Session to a new S-NSSAI. Pending SA2 decisions. | Solution with OAM, CN and UE impact Loss of efficiency due to target gNB may refuse the re-mapping decision of 5GC due to resource shortage. | 2, 4 |