3GPP TSG-RAN WG3 Meeting #111-e		R3-211297

E-meeting, 25 Jan – 4 Feb 2021

Title:	LS on using MOBIKE in Integrated Access and Backhaul system

Response to:

Release:	Rel-17

Work Item:	NR_IAB_enh-Core

Source:	RAN3

To:	SA3

Cc:

Contact person:	Steven Xu

Steven.1.xu@nokia-sbell.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

During the discussion on Integrated Access and Backhaul Enhancements for NR WI, RAN3 agreed the Working Assumption that RFC4555 IKEv2 Mobility and Multihoming Protocol (MOBIKE) can be used to reduce the service interruption during Intra-Donor-CU Inter-Donor-DU topology adaptation.

- When IPsec tunnel mode is used to protect the F1 traffic, the IAB-DU’s outer IP address is anchored in the Donor-DU. The IAB-DU’s inner IP address is used for the SCTP and F1 interface with IAB Donor CU.

- For the migrating IAB node migrating to another cell under another Donor-DU, the IAB node will be assigned with a new outer IP address anchored in this other Donor-DU.

- In Rel-16 topology adaptation, the IAB node will get a new inner IP address, and will use the new inner IP address to establish the SCTP association with IAB Donor.

- If using MOBIKE, the IAB node can update the outer IP address, while reuse the previous inner IP address and the previously established SCTP association after topology adaptation. The service interruption during the topology adaptation can be reduced, e.g. avoid the establishment new SCTP association.

RAN3 would like to ask SA3 to provide feedback, e.g. any issue prevent using MOBIKE in Integrated Access and Backhaul system.

# 2	Actions

To SA3:

ACTION: 	RAN3 respectfully ask SA3 to provide feedback, e.g. any issue prevent using MOBIKE in Integrated Access and Backhaul system.

# 3	Dates of next TSG RAN3 meetings

TSG-RAN3 Meeting #112-e	    			17th – 28th May 2021

TSG-RAN3 Meeting #113-e	    			23th – 27th Aug 2021