3GPP TSG-RAN WG3 Meeting #112-e	R3-211534

E-meeting, 17-28 May 2021

Title: 	Consideration on the Supported RAT of the CN

Source: 		Huawei

Agenda item:	9.2.1

Document Type:	Discussion

1. Introduction

The RAT Type Handling was discussed during RAN3#111 meeting [1]. About providing Supported RAT of the CN to RAN [2] [3], it was marked as to be continued.

In this contribution, we further discuss if and how to provide Supported RAT of the CN to RAN.

3. Discussion

- As we know, the supported RATs of the RAN node and CN nodes changes from release to release, for example:

- Since Rel-13, the eNB and EPC may be configured to support NB-IoT and/or WB-EUTRA separately.

- Since Rel-16, the ng-eNB and 5GC may be configured to support NB-IoT and/or WB-EUTRA separately.

- RAN nodes and CN nodes are probably updated separately, very big configuration efforts are needed to provide accurate supported RAT information of the CN nodes to the RAN nodes, misconfiguration may happen, especially in case the CN and RAN nodes come from different vendors.

- Observation 1: Configure and update the supported RATs of the CN nodes to all the connected RAN nodes, requires big configuration efforts, especially in case the CN nodes and RAN nodes come from different vendors.

- In last meeting, we have already agreed that if none of the RATs indicated by the RAN node in the NG/S1 SETUP REQUEST message is supported by the CN node, the CN node shall fail the NG/S1 Setup procedure with an appropriate cause value. Therefore in this contribution, we only need to discuss the scenario that at least one of the RATs of the RAN node is supported by the CN node. In such case, if the RAN node does not know the supported RATs of the CN node, or been configured with wrong information, wrong routing issue will happen.

- There are several scenarios: (take NG interface as example, also applies to S1 interface)

- Scenario 1: RAN node provides NB-IoT Cells and WB-EUTRA Cells, Rel-15 CN node1 does not support NB-IoT

- In this scenario, the AMF1 is a Rel-15 CN node which does not support NB-IoT, and cannot comprehend the RAT Information IE carried in NG SETUP REQUEST message, note that the RAT Information IE is designed as optional IE with assigned criticality reject.

- Even if the ng-eNB1 does not know that the AMF1 does not support NB-IoT before triggering NG setup, it will know that immediately, as the NG setup procedure will be failed by the AMF1.

- Therefore, in this scenario1, the ng-eNB1 is able to know that the AMF1 does not support NB-IoT based on current NGAP signalling design, no wrong routing issue happens.

- No wrong routing issue.

- Scenario 2: RAN node provides NB-IoT Cells and WB-EUTRA Cells, Rel-16 CN node2 does not support NB-IoT

- In this scenario, the AMF2 is a Rel-16 CN node which does not support NB-IoT, BUT it can comprehend the RAT Information IE carried in NG SETUP REQUEST message.

- Note: in current NGAP specification, it is said:

- IE or IE group: When one new or modified IE or IE group is implemented for one EP from a standard version, then other new or modified IEs or IE groups specified for that EP in that standard version shall be considered comprehended by a receiving entity (some may still remain unsupported).

- Based on current specification, when the ng-eNB1 provides RAT Information IE to indicate that the TAI is associated with NB-IoT cells, the AMF2 will not fail the NG Setup procedure, and will not provide any hint to the ng-eNB1 on that.

- Therefore, the ng-eNB1 may route a NB-IoT UE1 towards the AMF2, the AMF2 has to reject the UE1’s access or reroute it to another AMF, which is not expected and will lead to bad user experience.

- Wrong routing happens.

- Scenario 3: RAN node provides NB-IoT Cells and WB-EUTRA Cells, Rel-16 CN node3 is deployed to serve NB-IoT UEs only, i.e. not serve WB-E-UTRA UEs.

- The RAT Information IE is defined as ENUMERATED (unlicensed, NB-IoT, ...), the absence of this IE for a TAI in the NG SETUP REQUEST message from an ng-eNB, means that the TAI is associated with WB-EUTRA cells.

- Based on current signalling design, when the ng-eNB1 provides the information of both TAIs associated with NB-IoT cells (RAT Information IE set to “NB-IoT”)and TAIs associated with WB-EUTRA cells (absence of RAT Information IE), the AMF3 will not fail the NG Setup procedure, and will not provide any hint to the ng-eNB1 on that as well.

- Then the ng-eNB1 may route a WB-EUTRA UE3 towards the AMF3, the AMF3 has to reject the UE3’s access or reroute it to another AMF, which is not expected and will lead to bad user experience.

- Wrong routing happens.

- Based on the discussion above, we can see that the wrong routing issue will happen for scenario 2 and 3. Considering the extra rerouting efforts, the bad user experience, and the complexity to detect the reason of the wrong routing, it is proposed to provide the supported RAT information of the CN to the RAN nodes in NG/S1 SETUP RESPONSE and AMF/MME CONFIGURATION UPDATE messages.

- Proposal: Include the supported RATs of the CN to RAN in NG/S1 SETUP RESPONSE and AMF/MME CONFIGURATION UPDATE.

4. Proposals

- In this contribution, we further discussed the RAT type handling issue, and get the following proposal:

- Proposal: Include the supported RATs of the CN to RAN in NG/S1 SETUP RESPONSE and AMF/MME CONFIGURATION UPDATE.

- The corresponding CRs are provided in [4] and [5].

5. Reference

- R3-210959 Summary of Offline Discussion on RAT type Handling, Huawei (moderator)

- R3-211193 Supported RATs of the AMF, CR to TS 38.413, Rel-16, Huawei, CMCC, Vodafone, Telecom Italia, China Telecom, Deutsche Telekom, China Unicom, Verizon Wireless

- R3-21194 Supported RATs of the MME, CR to TS 36.413, Rel-16, Huawei, CMCC, Vodafone, Telecom Italia, China Telecom, Deutsche Telekom, China Unicom, Verizon Wireless

- R3-211535 Supported RATs of the AMF, CR to TS 38.413, Rel-16

- R3-211536 Supported RATs of the MME, CR to TS 36.413, Rel-16