3GPP TSG-RAN3 Meeting #112-e	R3-211696

# E-meeting, 17th – 27th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.463 | CR | 0589 | rev | 0 | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on E1AP handling for unmapped DL QoS flows |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, Intel Corporation, Huawei, China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CPUP_Split |  |  |  |  |  | Date: |  |  | 2021-05-06 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Procedures shown in Annex A.2, A.3 and A.6 of TS 38.300 are not fully supported in CP/UP split architecture. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new IE “PDU Session To Notify List” is added into the E1AP DL DATA NOTIFICATION message, reusing the same structure of the current “PDU Session To Notify List” included in the E1AP UL DATA NOTIFICATION message for the perspective of ASN.1. Impact assessment towards the previous version of the specification (same release): This CR is a clarification and has an isolated impact towards the previous version of the specification (same release). This CR only has an impact on QoS flow handling. This CR is backward compatible. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Procedures shown in Annex A.2, A.3 of TS 38.300 are not fully supported in CP/UP split architecture. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.7, 9.2.2.13, 9.3.3.x (new), 9.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR 38.460 CR 0049 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

///////////////////////////////////////////////////////////////////////skip unrelated text///////////////////////////////////////////////////////////////////////

8.3.7	DL Data Notification

8.3.7.1	General

This procedure is initiated by the gNB-CU-UP to indicate the detection of DL data arrival for the UE, or that a DL packet including a QFI value in the NG-U header not configured by the QoS Flows Information To Be Setup IE or the Flow Mapping Information IE is received for the first time. The procedure uses UE-associated signalling.

8.3.7.2	Successful Operation

Figure 8.3.7.2-1: DL Data Notification procedure: Successful Operation.

The gNB-CU-UP initiates the procedure by sending the DL DATA NOTIFICATION message to the gNB-CU-CP.

If the PPI IE is included in the DL DATA NOTIFICATION message, the gNB-CU-CP shall use it for paging policy differentiation.

If the PDU Session To Notify List IE is included in the DL DATA NOTIFICATION message, the gNB-CU-CP shall either map the flow(s) included in PDU Session To Notify List IE to the existing DRB or establish a new DRB for the flow(s).

8.3.7.3	Abnormal Conditions

Not applicable.

///////////////////////////////////////////////////////////////////////skip unrelated text///////////////////////////////////////////////////////////////////////

9.2.2.13	DL DATA NOTIFICATION

This message is sent by the gNB-CU-UP to provide information about the DL data detection to the gNB-CU-CP.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP UE E1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-CU-UP UE E1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Paging Priority Indicator (PPI) | O |  | 9.3.1.55 |  | YES | ignore |
| PDU Session To Notify List | O |  | 9.3.3.x |  | YES | ignore |

///////////////////////////////////////////////////////////////////////skip unrelated text///////////////////////////////////////////////////////////////////////

9.3.3.x	PDU Session To Notify List

This IE contains PDU session to notify related information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PDU Session To Notify Item |  | 1..<maxnoofPDUSessionResource> |  |  |
| >PDU Session ID | M |  | 9.3.1.21 |  |
| >QoS Flow List | M |  | 9.3.1.12 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessionResource | Maximum no. of PDU Sessions for a UE. Value is 256. |

///////////////////////////////////////////////////////////////////////skip unrelated text///////////////////////////////////////////////////////////////////////

9.4.4	PDU Definitions

////////////////////////////////////////////////////////////////skip unrelated codes////////////////////////////////////////////////////////////////

-- **************************************************************

--

#### -- DL DATA NOTIFICATION

--

-- **************************************************************

-- **************************************************************

--

-- DL Data Notification

--

-- **************************************************************

DLDataNotification ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { DLDataNotificationIEs } },

...

}

DLDataNotificationIEs E1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-CP-UE-E1AP-ID			CRITICALITY reject	TYPE GNB-CU-CP-UE-E1AP-ID			PRESENCE mandatory }|

{ ID id-gNB-CU-UP-UE-E1AP-ID			CRITICALITY reject	TYPE GNB-CU-UP-UE-E1AP-ID			PRESENCE mandatory }|

{ ID id-PPI							CRITICALITY ignore	TYPE PPI							PRESENCE optional }|

{ ID id-PDU-Session-To-Notify-List		CRITICALITY ignore	TYPE PDU-Session-To-Notify-List		PRESENCE optional },

...

}

////////////////////////////////////////////////////////////////end of change////////////////////////////////////