3GPP TSG-RAN WG3 #112-e	R3-211861

E-meeting, 17th – 27th May 2021	Revise of R3-210306

Source:	CATT

Title:	(TP on SON for 36.423) Discussion on PSCell MLB and SUL PRB usage

Agenda Item:	10.2.1.5

Document for:	Discussion and decision

- Introduction

During RAN3 #111-e meeting, there was some discussion on whether two new types of load metrics can be introduced: a new load metric for exchanging potential EN-DC PSCell load between eNBs, and a new load metric to indicate SUL load. We found that the two new types proposed are similar for some aspect. In this document we provide our understanding on why they can be beneficial.

- Discussion

Please consider the following scenario:

In one area, there were three E-UTRA cells at first: E-Cell #1, E-Cell #2 and E-Cell #3. These three E-UTRA cells belonged to three different eNBs: eNB 1, eNB 2 and eNB 3, respectively. These three eNBs are connected with any another with X2 interfaces.

And one day, the operator decided to enhance the network to support EN-DC. They deployed three en-gNBs: en-gNB 1, en-gNB 2 and en-gNB 3, each of which collocates with the corresponding eNB, and has only one X2 interface with the corresponding eNB. Each en-gNB controls some NR cells, including NR Cell #1, NR Cell #2 and NR Cell #3, each of which belongs to the corresponding en-gNB.

This EN-DC network is quite welcome by the consumers, and often works in full capacity. And then the operator wishes to introduce some MLB mechanism in order for a better efficiency.

But according to the current specification, one eNB can only get aware of the load status of the cells controlled by any RAN node which has a direct X2 connection with itself. E.g., eNB 1 is only aware of the load status of E-Cell #1, E-Cell #2, E-Cell #3 and NR Cell #1; it is unaware of the load status of NR Cell #2 or NR Cell #3.

Considering NR Cell tends to have a much higher capacity than E-UTRA cells, lacking of such information means that MLB is often impossible to work as supposed.

For example, one UE is receiving some service that is only suitable in NR, and is serviced by the E-Cell #1/NR Cell #1 but on the border between E-Cell #1/NR Cell #1 and E-Cell #2/NR Cell #2.

Now E-Cell #1/NR Cell #1 is experiencing a high load and eNB 1 wishes to offload it toward E-Cell #2/NR Cell #2. But eNB 1, according to the current specification, does not know the load status of NR Cell #2. It only observes that E-Cell #2 is now in medium load and triggers a handover for MLB. But NR Cell #2 may be experiencing an extremely high load at present. Such handover will clearly contradict the purpose of MLB.

During the meeting RAN3 #110e meeting, it was ever proposed that the legacy E-UTRA load metrics can be defined as taken the load status of potential MR-DC PSCell into account. However we find this solution will severely impact the legacy LTE MLB mechanism. Please consider the following scenarios:

| Load of E-Cell #2 with capacity = 10 | Load of NR Cell #2 with capacity = 100 | Ideal target of MLB for legacy LTE UE with required capacity = 2 | Ideal target of MLB for EN-DC UE with required capacity = 20 |
| --- | --- | --- | --- |
| Low, 2 used, 8 free | Low, 20 used, 80 free | Yes (8 > 2) | Yes (8 + 80 = 88 > 20) |
| Low, 2 used, 8 free | High, 90 used, 10 free | Yes (8 > 2) | No (8 + 10 = 18 < 20) |
| High, 9 used, 1 free | Low, 20 used, 80 free | No (1 < 2) | Yes (1 + 80 = 81 > 20) |
| High, 9 used, 1 free | High, 90 used, 10 free | No (1 < 2) | No (1 + 10 = 11 < 20) |

Figure 1: Reason for two separate load metrics in EN-DC for two layers of MLB with two types of UE.

It can be seen that there are some scenarios that the E-UTRA cell is suitable for legacy LTE UE but not suitable for EN-DC capable UE, while there are also some scenarios that it is suitable for EN-DC capable UE but not suitable for legacy UE. Only one load metric is obviously not enough to represent such two-dimentional load status.

Observation 1: There are cases that one E-UTRA cell / NR Cell is filled with legacy LTE UEs, in which state the E-UTRA’s load is high and the NR’s load is low, thus ideal to accept EN-DC UEs but not ideal for legacy LTE UEs.

Observation 2: One load metric is not enough to represent both the dimention of whether an E-UTRA cell is a suitable target for MLB-oriented handover of legacy LTE UEs and the dimention of whether it is a suitable target for MLB-oriented handover of EN-DC capable UEs.

Therefore, we proposed that one eNB working in EN-DC should also be possible to get aware of the load information of potential target PSCell from other eNBs.

Proposal 1: One eNB working in EN-DC should also be possible to get aware of the load information of potential target PSCell from other eNBs.

Nevertheless, we acknowledge the concern that such metric has too much impact on specification and/or network behaviour, and may exaggerate message size. A more “composite” solution, such as adding a metric of “composite EN-DC load information” for every E-UTRA cell instead of potential target EN-DC PSCell, is also acceptable for us.

The case on SUL load is similar. SUL bands tend to have wider coverage and thus accessible for more UEs (especially for indoor UEs) than NUL, but tend to have less radio resources, just like legacy E-UTRA cells tend to be accessible for more UEs than NSA NR cells, but tend to have less radio resources:

| Load of SUL with capacity = 10 | Load of NUL with capacity = 100 | Ideal target of MLB for indoor UEs (NUL not available) with required capacity = 2 | Ideal target of MLB for outdoor UEs with required capacity = 20 |
| --- | --- | --- | --- |
| Low, 2 used, 8 free | Low, 20 used, 80 free | Yes (8 > 2) | Yes (8 + 80 = 88 > 20) |
| Low, 2 used, 8 free | High, 90 used, 10 free | Yes (8 > 2) | No (8 + 10 = 18 < 20) |
| High, 9 used, 1 free | Low, 20 used, 80 free | No (1 < 2) | Yes (1 + 80 = 81 > 20) |
| High, 9 used, 1 free | High, 90 used, 10 free | No (1 < 2) | No (1 + 10 = 11 < 20) |

Figure 2: Reason for separate load metrics for SUL.

Observation 3: There are cases that one NR Cell configured with NUL and SUL is filled UEs in poor radio condition (e.g. indoor), in which state the SUL’s load is high and the NUL’s load is low, thus ideal to accept UEs in good radio condition (e.g. outdoor) but not ideal for UEs in poor radio condition.

Proposal 2: Load of SUL should also be possible to exchange between RAN entities.

- Conclusion

Observation 1: There are cases that one E-UTRA cell / NR Cell is filled with legacy LTE UEs, in which state the E-UTRA’s load is high and the NR’s load is low, thus ideal to accept EN-DC UEs but not ideal for legacy LTE UEs.

Observation 2: One load metric is not enough to represent both the dimention of whether an E-UTRA cell is a suitable target for MLB-oriented handover of legacy LTE UEs and the dimention of whether it is a suitable target for MLB-oriented handover of EN-DC capable UEs.

Proposal 1: One eNB working in EN-DC should also be possible to get aware of the load information of potential target PSCell from other eNBs.

Observation 3: There are cases that one NR Cell configured with NUL and SUL is filled UEs in poor radio condition (e.g. indoor), in which state the SUL’s load is high and the NUL’s load is low, thus ideal to accept UEs in good radio condition (e.g. outdoor) but not ideal for UEs in poor radio condition.

Proposal 2: Load of SUL should also be possible to exchange between RAN entities.

We provide one TP accordingly as below coveraging the two proposals above, and two separate TPs for Proposal 2 [1–2].

- Reference

[1] R3-211862; (TP on SON for 38.423) Support of SUL PRB usage; CATT.

[2] R3-211863; (TP on SON for 38.473) Support of SUL PRB usage; CATT.

- TP for TS 36.423

//////////////////////////////////////////////////////////////skip unchanged//////////////////////////////////////////////////////////////

8.3.6	Resource Status Reporting Initiation

8.3.6.1	General

This procedure is used by an eNB to request the reporting of load measurements to another eNB.

The procedure uses non UE-associated signalling.

8.3.6.2	Successful Operation

Figure 8.3.6.2-1: Resource Status Reporting Initiation, successful operation

The procedure is initiated with a RESOURCE STATUS REQUEST message sent from eNB1 to eNB2. Upon receipt, eNB2:

-	shall initiate the requested measurement according to the parameters given in the request in case the Registration Request IE set to "start"; or

-	shall stop all cells measurements and terminate the reporting in case the Registration Request IE is set to "stop"; or

-	if supported, stop cell measurements and terminate the reporting for cells indicated in the Cell To Report IE list, in case the Registration Request IE is set to "partial stop"; or

-	if supported, add cells indicated in the Cell To Report IE list to the measurements initiated before for the given measurement IDs, in case the Registration Request IE is set to "add".

If the eNB2 received a RESOURCE STATUS REQUEST message, which includes the Registration Request IE set to "stop", the Cell To Report IE list shall be ignored.

If the Registration Request IE is set to "start" then the Report Characteristics IE shall be included in RESOURCE STATUS REQUEST message. The eNB2 shall ignore the Report Characteristics IE, if the Registration Request IE is not set to "start".

The Report Characteristics IE indicates the type of objects eNB2 shall perform measurements on. For each cell, the eNB2 shall include in the RESOURCE STATUS UPDATE message:

-	the Radio Resource Status IE, if the first bit, "PRB Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1;

-	the S1 TNL Load Indicator IE, if the second bit, "TNL Load Ind Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1;

-	the Hardware Load Indicator IE, if the third bit, "HW Load Ind Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1;

-	the Composite Available Capacity Group IE, if the fourth bit, "Composite Available Capacity Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1. If Cell Capacity Class Value IE is included within the Composite Available Capacity Group IE, this IE is used to assign weights to the available capacity indicated in the Capacity Value IE;

-	the ABS Status IE, if the fifth bit, "ABS Status Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1 and eNB1 had indicated the ABS pattern to eNB2;

-	the RSRP Measurement Report List IE, if the sixth bit, "RSRP Measurement Report Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1;

-	the CSI Report IE, if the seventh bit, "CSI Report Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1;

-	the Radio Resource Status IE within the NR Neithbour Cell Measurement Result IE, if the eighth bit, "Neighbour Cell Load Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1.

NOTE:	In order to avoid duplication, the eNB2 may include only one copy of NR Neithbour Cell Measurement Result Item per RESOURCE STATUS UPDATE messages per NR neighbour cell, even if this NR cell neighbours to multiple E-UTRA cell served by eNB2. The eNB1 should deduce which NR cell is capable of performing EN-DC with the corresponding served E-UTRA cell by using the NR Neighbour Information IE received during the X2 Setup procedure or the eNB Configuration Update procedure.

If the Reporting Periodicity IE is included in the RESOURCE STATUS REQUEST message, eNB2 shall use its value as the time interval between two subsequent RESOURCE STATUS UPDATE messages that include the Radio Resource Status IE, S1 TNL Load Indicator IE, Hardware Load Indicator IE, Composite Available Capacity Group IE, or ABS Status IE.

If the Reporting Periodicity of RSRP Measurement Report IE is included in the RESOURCE STATUS REQUEST message, eNB2 shall use its value as the minimum time interval between two subsequent RESOURCE STATUS UPDATE messages that include the RSRP Measurement Report List IE.

If the Reporting Periodicity of CSI Report IE is included in the RESOURCE STATUS REQUEST message, eNB2 shall use its value as the minimum time interval between two subsequent RESOURCE STATUS UPDATE messages that include the CSI Report IE.

If eNB2 is capable to provide all requested resource status information, it shall initiate the measurement as requested by eNB1, and respond with the RESOURCE STATUS RESPONSE message.

If eNB2 is capable to provide some but not all of the requested resource status information and the Partial Success Indicator IE is present in the RESOURCE STATUS REQUEST message, it shall initiate the measurement for the admitted measurement objects and include the Measurement Initiation Result IE in the RESOURCE STATUS RESPONSE message.

//////////////////////////////////////////////////////////////skip unchanged//////////////////////////////////////////////////////////////

9.1.2.11	RESOURCE STATUS REQUEST

This message is sent by an eNB1 to neighbouring eNB2 to initiate the requested measurement according to the parameters given in the message.

Direction: eNB1  eNB2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| eNB1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by eNB1 | YES | reject |
| eNB2 Measurement ID | C-ifRegistrationRequestStoporPartialStoporAdd |  | INTEGER (1..4095,...) | Allocated by eNB2 | YES | ignore |
| Registration Request | M |  | ENUMERATED(start, stop, …, partial stop, add) | Type of request for which the resource status is required. | YES | reject |
| Report Characteristics | O |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates measurement object the eNB2 is requested to report. First Bit = PRB Periodic, Second Bit = TNL load Ind Periodic, Third Bit = HW Load Ind Periodic, Fourth Bit = Composite Available Capacity Periodic, this bit should be set to 1 if at least one of the First, Second or Third bits is set to 1, Fifth Bit = ABS Status Periodic, Sixth Bit = RSRP Measurement Report Periodic, Seventh Bit = CSI Report Periodic, Eighth Bit = Neighbour Cell Load Periodic. Other bits shall be ignored by the eNB2. | YES | reject |
| Cell To Report |  | 1 |  | Cell ID list to which the request applies. | YES | ignore |
| >Cell To Report Item |  | 1 .. <maxCellineNB> |  |  | EACH | ignore |
| >>Cell ID | M |  | ECGI 9.2.14 |  | – |  |
| Reporting Periodicity | O |  | ENUMERATED(1000ms, 2000ms, 5000ms,10000ms, …) | Periodicity that can be used for reporting of PRB Periodic, TNL Load Ind Periodic, HW Load Ind Periodic, Composite Available Capacity Periodic or ABS Status Periodic. | YES | ignore |
| Partial Success Indicator | O |  | ENUMERATED(partial success allowed, ...) | Included if partial success is allowed | YES | ignore |
| Reporting Periodicity of RSRP Measurement Report | O |  | ENUMERATED(120ms, 240ms, 480ms, 640ms, ...) | Periodicity that can be used for the reporting of RSRP Measurement Report Periodic. | YES | ignore |
| Reporting Periodicity of CSI Report | O |  | ENUMERATED(5ms, 10ms, 20ms, 40ms, 80ms, ...) | Periodicity that can be used for the reporting of CSI Report Periodic. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxCellineNB | Maximum no. cells that can be served by an eNB. Value is 256. |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestStoporPartialStoporAdd | This IE shall be present if the Registration Request IE is set to the value "stop", "partial stop" or "add". |

9.1.2.12	RESOURCE STATUS RESPONSE

This message is sent by the eNB2 to indicate that the requested measurement, for all or for a subset of the measurement objects included in the measurement is successfully initiated.

Direction: eNB2  eNB1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| eNB1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by eNB1 | YES | reject |
| eNB2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by eNB2 | YES | reject |
| Criticality Diagnostics | O |  | 9.2.7 |  | YES | ignore |
| Measurement Initiation Result |  | 0..1 |  | List of all cells in which measurement objects were requested, included when indicating partial success | YES | ignore |
| >Measurement Initiation Result Item |  | 1 .. <maxCellineNB> |  |  | EACH | ignore |
| >>Cell ID | M |  | ECGI 9.2.14 |  | – |  |
| >>Measurement Failure Cause List |  | 0..1 |  | Indicates that eNB2 could not initiate the measurement for at least one of the requested measurement objects in the cell | – |  |
| >>>Measurement Failure Cause Item |  | 1 .. <maxFailedMeasObjects> |  |  | EACH | ignore |
| >>>>Measurement Failed Report Characteristics | M |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates measurement object that failed to be initiated in the eNB2. First Bit = PRB Periodic, Second Bit = TNL load Ind Periodic, Third Bit = HW Load Ind Periodic, Fourth Bit = Composite Available Capacity Periodic, Fifth Bit = ABS Status Periodic, Sixth Bit = RSRP Measurement Report Periodic, Seventh Bit = CSI Report Periodic, Eighth Bit = Neighbour Cell Load Periodic. Other bits shall be ignored by the eNB1. | – |  |
| >>>>Cause | M |  | 9.2.6 | Failure cause for measurement objects for which the measurement cannot be initiated | – |  |

| Range bound | Explanation |
| --- | --- |
| maxFailedMeasObjects | Maximum number of measurement objects that can fail per measurement. Value is 32. |
| maxCellineNB | Maximum no. cells that can be served by an eNB. Value is 256. |

9.1.2.13	RESOURCE STATUS FAILURE

This message is sent by the eNB2 to indicate that for none of the requested measurement objects the measurement can be initiated.

Direction: eNB2  eNB1.

| IE/Group Name | Presence |  | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  |  | 9.2.13 |  | YES | reject |
| eNB1 Measurement ID | M |  |  | INTEGER (1..4095,...) | Allocated by eNB1 | YES | reject |
| eNB2 Measurement ID | M |  |  | INTEGER (1..4095,...) | Allocated by eNB2 | YES | reject |
| Cause | M |  |  | 9.2.6 | Ignored by the receiver when the Complete Failure Cause Information IE is included | YES | ignore |
| Criticality Diagnostics |  | O |  | 9.2.7 |  | YES | ignore |
| Complete Failure Cause Information |  |  | 0..1 |  | Complete list of failure causes for all requested cells | YES | ignore |
| >Complete Failure Cause Information Item |  |  | 1 .. <maxCellineNB> |  |  | EACH | ignore |
| >>Cell ID | M |  |  | ECGI 9.2.14 |  | – |  |
| >>Measurement Failure Cause List |  |  | 1 |  |  | – |  |
| >>>Measurement Failure Cause Item |  |  | 1 .. <maxFailedMeasObjects> |  |  | EACH | ignore |
| >>>>Measurement Failed Report Characteristics | M |  |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates measurement object that failed to be initiated in the eNB2. First Bit = PRB Periodic, Second Bit = TNL load Ind Periodic, Third Bit = HW Load Ind Periodic, Fourth Bit = Composite Available Capacity Periodic, Fifth Bit = ABS Status Periodic, Sixth Bit = RSRP Measurement Report Periodic, Seventh Bit = CSI Report Periodic, Eighth Bit = Neighbour Cell Load Periodic. Other bits shall be ignored by the eNB1. | – |  |
| >>>>Cause | M |  |  | 9.2.6 | Failure cause for measurements that cannot be initiated | – |  |

| Range bound | Explanation |
| --- | --- |
| maxCellineNB | Maximum no. cells that can be served by an eNB. Value is 256. |
| maxFailedMeasObjects | Max number of measurement objects that can fail per measurement. Value is 32. |

9.1.2.14	RESOURCE STATUS UPDATE

This message is sent by eNB2 to neighbouring eNB1 to report the results of the requested measurements.

Direction: eNB2  eNB1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | ignore |
| eNB1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by eNB1 | YES | reject |
| eNB2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by eNB2 | YES | reject |
| Cell Measurement Result |  | 1 |  |  | YES | ignore |
| >Cell Measurement Result Item |  | 1 .. <maxCellineNB> |  |  | EACH | ignore |
| >>Cell ID | M |  | ECGI 9.2.14 |  |  |  |
| >>Hardware Load Indicator | O |  | 9.2.34 |  |  |  |
| >>S1 TNL Load Indicator | O |  | 9.2.35 |  |  |  |
| >>Radio Resource Status | O |  | 9.2.37 |  |  |  |
| >>Composite Available Capacity Group | O |  | 9.2.44 |  | YES | ignore |
| >>ABS Status | O |  | 9.2.58 |  | YES | ignore |
| >>RSRP Measurement Report List | O |  | 9.2.76 |  | YES | ignore |
| >>CSI Report | O |  | 9.2.79 |  | YES | ignore |
| >>Cell Reporting Indicator | O |  | ENUMERATED(stop request, ...) |  | YES | ignore |
| >>NR Neithbour Cell Measurement Result |  | 0..1 |  |  | YES | ignore |
| >>>NR Neithbour Cell Measurement Result Item |  | 1 .. <maxnoofNRNeighbours> |  |  |  |  |
| >>>>Cell ID | M |  | NR CGI 9.2.111 |  |  |  |
| >>>>Radio Resource Status | O |  | NR Radio Resource Status 9.2.162 |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxCellineNB | Maximum no. cells that can be served by an eNB. Value is 256. |
| maxnoofNRNeighbours | Maximum no. of neighbour NR cells associated to a given served cell. Value is 1024. |

//////////////////////////////////////////////////////////////skip unchanged//////////////////////////////////////////////////////////////

9.2.162	NR Radio Resource Status

The NR Radio Resource Status IE indicates the usage of the PRBs per cell and per SSB area for all traffic in Downlink and Uplink and the usage of PDCCH CCEs for Downlink and Uplink scheduling.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| SSB Area Radio Resource Status List |  | 1 |  |  | – |  |
| >SSB Area Radio Resource Status Item |  | 1..<maxnoofSSBAreas> |  |  | – |  |
| >>SSB Index | M |  | 9.2.167 |  | – |  |
| >>SSB Area DL GBR PRB usage | M |  | INTEGER (0..100) | Per SSB area DL GBR PRB usage | – |  |
| >>SSB Area UL GBR PRB usage | M |  | INTEGER (0..100) | Per SSB area UL GBR PRB usage | – |  |
| >>SSB Area DL non-GBR PRB usage | M |  | INTEGER (0..100) | Per SSB area DL non-GBR PRB usage | – |  |
| >>SSB Area UL non-GBR PRB usage | M |  | INTEGER (0..100) | Per SSB area UL non-GBR PRB usage | – |  |
| >>SSB Area DL Total PRB usage | M |  | INTEGER (0..100) | Per SSB area DL Total PRB usage | – |  |
| >>SSB Area UL Total PRB usage | M |  | INTEGER (0..100) | Per SSB area UL Total PRB usage | – |  |
| >>DL scheduling PDCCH CCE usage | O |  | INTEGER (0..100) |  | – |  |
| >>UL scheduling PDCCH CCE usage | O |  | INTEGER (0..100) |  | – |  |
| >>SSB Area SUL GBR PRB usage | O |  | INTEGER (0..100) | Per SSB area SUL GBR PRB usage | YES | ignore |
| >>SSB Area SUL non-GBR PRB usage | O |  | INTEGER (0..100) | Per SSB area SUL non-GBR PRB usage | YES | ignore |
| >>SSB Area SUL Total PRB usage | O |  | INTEGER (0..100) | Per SSB area SUL Total PRB usage | YES | ignore |
| >>SUL scheduling PDCCH CCE usage | O |  | INTEGER (0..100) |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a NG-RAN node cell. Value is 64. |

//////////////////////////////////////////////////////////////end//////////////////////////////////////////////////////////////