3GPP TSG-RAN WG3 #112-e	R3-212131

17 May– 28 May 2021

Online

Title: 	(TP for SON BL CR for TS 38.413) Successful Handover support

Source: 	ZTE

Agenda item:	10.2.1.3

Document for:	Approval

1	Introduction and discussion

This contribution provided the TP for based on the following proposal.

Proposal: In order to support inter-RAT SHR, to define Choice IE type of“Successful HO Report”in XnAP, NGAP and F1AP.

2	TP to TS 38.413

*** Start changed ***

- 9.3.3.35	SON Information Report

This IE contains the configuration information to be transferred.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE SON Information Report | M |  |  |  |
| >Failure Indication Information |  |  |  |  |
| >>Failure Indication | M |  | 9.3.3.37 |  |
| >HO Report Information |  |  |  |  |
| >>HO Report | M |  | 9.3.3.39 |  |
| CHOICE SHR type | O |  |  |  |
| >NR |  |  |  |  |
| >>>NRSuccessful HO Report List |  | 0..1 |  |  |
| >>>> NRSuccessful HO Report List Item |  | 1 .. <maxnoofSuccessfulHOReports> |  |  |
| >>> Successful HO Report Container | O |  | OCTET STRING | FFS |
| >LTE |  |  |  |  |
| >>LTE Successful HO Report List |  | 0..1 |  |  |
| >>>LTE Successful HO Report List Item |  | 1 .. <maxnoofSuccessfulHOReports> |  |  |
| >>>> LTE Successful HO Report Container | O |  | OCTET STRING | FFS on the definition |

| Range bound | Explanation |
| --- | --- |
| maxnoofSuccessfulHOReports | Maximum no. of Successful HO Reports, the maximum value is 64. FFS |

*** End of changed ***