3GPP TSG-RAN3 Meeting # 112-e	   R3-212342

# 17th  - 28th  May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 0611 | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the specified maximum length of the Routing ID Octet String |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_POS-Core |  |  |  |  |  | Date: |  |  | 2021-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Contrary to the E-SMLC Routing ID in LTE coded as INTEGER (0..255), the LMF Routing ID in NR has an unspecified octet string. According to TS 23.273 sections 6.11.2: - “The AMF forwards the Network Positioning message to the serving NG-RAN node in an N2 Transport message. The AMF includes a Routing identifier, in the N2 Transport message, identifying the LMF (e.g. a global address of the LMF).” The general identification of the LMF ID is defined in TS 29.518 as the string NfInstanceId, which is defined in TS 29.571 (UUID, i.e. 16 octets) TS 29.518: nfId; NfInstanceId; C; 0..1; This IE shall be present if the subscription is for "NRPPa" N2 information class and/or "LPP" N1 information class. When present, this IE shall carry the value to be used for NGAP "Routing ID" IE, which identifies the Network Function (e.g. LMF) instance handling the NRPPa and/or LPP data.. And then TS 29.571: NfInstanceId; string; String uniquely identifying a NF instance. The format of the NF Instance ID shall be a Universally Unique Identifier (UUID) version 4, as described in IETF RFC 4122 [15]. (NOTE 3). Different AMF implementations cannot unfortunately guarantee that the Routing ID is coded in a unique way. A size limitation is required for a safe inter-vendor operable networking. This is important so that RAN can be aware of the maximum Octet String length to compare two Routing IDs when, for e.g., it receives NRPPa messages from different LMFs. It is proposed hence to clarify the octet string maximum size of the LMF Routing ID in NG-AP to be that of the NfInstanceId, set to UUID format, i.e. 16 octets. This CR has impact on ASN.1 Routing ID OCTET STRING size and can be considered non backwards compatible. |
| Summary of change: |  | Specify the Routing ID IE Type OCTET STRING’s maximum size to be that of the NfInstanceId string per TS 29.571, used to identify the LMF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There can be problem in multi-vendor deployments with different AMF implementations, if the limitation of the Routing ID maximum length is not known at RAN level, or supposed to be inferior to what can be used. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.3.13, 9.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

#### 9.3.3.13	Routing ID

This IE is used to identify an LMF within the 5GC.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Routing ID | M |  | OCTET STRING (SIZE(1..16)) | The maximum length corresponds to NfInstanceId defined in TS 29.571 [35] |

NEXT CHANGE

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

SKIPPED TEXT UNCHANGED

#### -- R

SKIPPED TEXT UNCHANGED

RoutingID ::= OCTET STRING (SIZE (1..16))

END OF CHANGES