3GPP TSG-RAN WG3 Meeting #112-e	R3-212437

E-meeting, 17–28 May 2021

Title: 	(TP to TS 38.470 BL CR and TS 38.460 BL CR) Bearer management over F1 and E1

Source: 	Huawei, CBN

Agenda item:	22.2.4

Document Type:	Discussion, Decision

# 1. Introduction

As discussed in [1], the following proposals can be found:

Proposal 1. Legacy UE-associated E1AP and F1AP procedures could be reused to setup the MRB for multicast; enhancing the DRB with MBS info or introducing a new MRB subjects to RAN2 progress.

Proposal 2. The setup of the MRB for multicast over F1 could be accomplished by the F1AP: UE Context Modification procedure.

Proposal 3. The setup of the MRB for multicast over E1 could be accomplished by the E1AP: Bearer Context Modification procedure.

Proposal 5. Support IP multicast method for the shared F1-U transport for multicast and broadcast services.

Proposal 6. To support Shared NG-U transport, it is needed to introduce a non UE associated Class1 E1AP procedure, e.g. named as Multicast Distribution Setup procedure.

Proposal 7. To support Shared F1-U transport, it is needed to introduce a non UE associated Class1 F1AP procedure, e.g. named as Multicast Distribution Setup procedure.

Proposal 8. To support Shared E1-U transport, it is needed to introduce a non UE associated Class1 E1AP procedure, it could be the same procedure to setup shared NG-U transport, or another new procedure.

Proposal 10. To support the UE dedicated F1 tunnel for MBS transmission, legacy UE-associated F1AP procures could be applied, i.e. UE Context Modification procedure.

Proposal 11. To support the UE dedicated E1 tunnel for MBS transmission, legacy UE-associated E1AP procures could be applied, i.e. Bearer Context Modification procedure.

Proposal 12. Support Per Cell Shared F1-U Tunnel as the baseline, and further discuss whether per DU shared F1-U tunnel can be used or not after SA3 progress.

Proposal 13. To setup of the MRB for broadcast over E1/F1 interface, non-UE associated F1/E1 procedures are needed.

Proposal 14. For broadcast session, the shared F1-U/NG-U tunnel shall be established using non-UE associated procedures, signalling is FFS.

In this contribution, we provide corresponding TPs to TS 38.470 and TS38.460 BL CRs.

# 2. Reference

- R3-212436 (TP to TS 38.401 BL CR) Bearer management over F1 and E1, Huawei

# 3. TP for TS38.470 BL CR

--------------------------------Start of the Change-----------------------------

### 5.2.x	NR MBS function

Eidtor Notes: TBD

The NR MBS function supports the establishment, modification and releasing the resources for the MBS shared F1-U transmission between the gNB-CU and gNB-DU.

If IP multicast based transmission is used, the gNB-CU allocates the Transport Layer Address used for the IP multicast and the DL TEID used for the shared F1-U Transport, and then the gNB-DU joins the IP multicast group of MBS traffic for the shared F1-U transport.

If IP multicast based transmission is not used, the gNB-DU assigns DL GTP-U Tunnel information used for the shared F1-U Transport, and then the gNB-DU transmit the NR MBS packets to the gNB-DU via the shared F1-U tunnel.

--------------------------------Start of the Next Change-----------------------------

### 6.1.x	NR MBS procedures

The F1 MBS procedures are listed below:

- [TBD];

- Multicast Distribution Setup procedure

- Multicast Distribution Modification procedure

- Multicast Distribution Release procedure

- Broadcast Context setup procedure

- Broadcast Context Release Request (gNB-DU initiated) procedure

- Broadcast Context Release (gNB-CU initiated) procedure

- Broadcast Context Modification (gNB-CU initiated) procedure

- Broadcast Context Modification Required (gNB-DU initiated) procedure

--------------------------------End of the Changes-----------------------------

# 4. TP for TS38.460 BL CR

--------------------------------Start of the Change-----------------------------

# 5	Functions of the E1 interface

## 5.1	General

The following clauses describe the functions supported over E1.

>>>>>>>>>>skip the unchanged part<<<<<<<<<<

### 5.1.x	NR MBS function

Editor Notes: TBD

#### 5.1.x.a	Multicast Distribution Management function

This function is used to establish, modify and release the resources for MBS session related shared F1-U transmission between the gNB-DU and the gNB-CU-UP, and shared NG-U between the gNB-CU-UP and the UPF.

If IP multicast based transmission is used for the shared F1-U Transport, the gNB-CU-UP allocates the Transport Layer Address used for the IP multicast and the DL TEID used for the shared F1-U Transport, provides such information to the gNB-DU via gNB-CU-CP, and then the gNB-DU joins the IP multicast group of MBS traffic for the shared F1-U transport.

If IP multicast based transmission is not used for the shared F1-U Transport, the gNB-DU assigns DL GTP-U Tunnel information used for the shared F1-U Transport, provides such information to the gNB-CU-UP via gNB-CU-CP, and then the gNB-DU-UP transmits the NR MBS packets to the gNB-DU via the shared F1-U tunnel.

If IP multicast based transmission is used for the shared NG-U Transport, the gNB-CU-CP forwards the Transport Layer Address used for the IP multicast and the DL TEID provided from core network used for the shared NG-U Transport to the gNB-CU-UP, and then the gNB-CU-UP joins the IP multicast group of MBS traffic for the shared NG-U transport.

If IP multicast based transmission is not used for the shared NG-U Transport, the gNB-CU-UP assigns DL GTP-U Tunnel information used for the shared NG-U Transport, provides such information to the core network via gNB-CU-CP, and then the core network transmits the NR MBS packets to the gNB-CU-UP via the shared NG-U tunnel.

#### 5.1.x.b	Broadcast Bearer Management function

The establishment of the broadcast bearer context is initiated by the gNB-CU-CP and accepted or rejected by the gNB-CU-UP based on admission control criteria (e.g., resource not available).

The modification of the broadcast bearer context can be initiated by either gNB-CU-CP or gNB-CU-UP. The receiving node can accept or reject the modification. The broadcast bearer context management function also supports the release of the broadcast context previously established in the gNB-CU-UP. The release of the broadcast context is triggered by the gNB-CU-CP either directly or following a request received from the gNB-CU-UP.

>>>>>>>>>>skip the unchanged part<<<<<<<<<<

## 5.2	TEIDs allocation

The gNB-CU-UP is responsible for the allocation of the F1-U UL GTP TEID for each data radio bearer.

The gNB-CU-UP is responsible for the allocation of the S1-U DL GTP TEID for each E-RAB and the NG-U DL GTP TEID for each PDU Session.

The gNB-CU-UP is responsible for the allocation of the X2-U DL/UL GTP TEID or the Xn-U DL/UL GTP TEID for each data radio bearer.

If IP multicast based transmission is not used for the shared NG-U transport for a MBS session, the gNB-CU-UP is responsible for the allocation of the NG-U DL GTP TEID for the MBS Session.

-------------------------- Start of the Next Change--------------------------

# 6	Procedures of the E1 interface

>>>>>>>>>>skip the unchanged part<<<<<<<<<<

## 6.x	NR MBS procedures

The following list of Multicast distribution management procedures are used to establish, modify or release F1-U Multicast Distribution resources and NG-U Multicast Distribution resources for MBS Session. The E1 MBS procedures are listed below:

-	[TBD]

-	Multicast Distribution Setup procedure

-	Multicast Distribution Modification procedure

-	Multicast Distribution Release procedure

-	Broadcast Bearer Context Setup procedure

-	Broadcast Bearer Context Modification (gNB-CU-CP initiated) procedure

-	Broadcast Bearer Context Modification Required (gNB-CU-UP initiated) procedure

-	Broadcast Bearer Context Release (gNB-CU-CP initiated) procedure

-	Broadcast Bearer Context Release Request (gNB-CU-UP initiated) procedure

--------------------------------End of the Changes-----------------------------