3GPP TSG-RAN WG3 Meeting #112-e	R3-212789

17-27 May 2021

Agenda item:	20.2.1

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP for BL CR for TS 38.300) Cell ID handling on NG/Xn interface

Document for:	Discussion & decision

Per the discussion of CB: # 78_NTN_NW_IDhandling, the following TP is proposed for BL CR for TS 38.300.

TP for TS38.300 BL CR:

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

…

[43]	3GPP TS 37.355: "LTE Positioning Protocol (LPP)".

[x]	3GPP TS 38.423: " NG-RAN; Xn Application Protocol (XnAP)".

### 16.x.5 Signalling [FFS]

The Cell Identity used in following cases corresponds to a fixed geographical area, irrespective of the orbit of the NTN payload.

-	The Cell Identity indicated by the gNB to the Core Network as part of the User Location Information (as defined in TS 38.413 [26])

-	The Cell Identity used for Paging Optimization in NG interface (as defined in TS 38.413 [26])

-	The Cell Identity used for Area of Interest (as defined in TS 38.413 [26])

-	The Cell Identity used for PWS (as defined in TS 38.413 [26])

Editor’s note:	it is FFS whether the Cell Identity indicated by the gNB to the Core Network should be specified per function/procedute/feature as above.

The Cell Identity included within the target identification of the handover messages (as defined in TS 38.413 [26] and TS 38.423 [x]) allows identifying the correct target cell.

Editor’s note:	it is FFS whether the Cell Identity used in other cases is only allowed to corresponds to a Uu Cell Identity or is allowed to correspond to both, a Uu and a fixed geographical area.

Editor’s note:	It is FFS whether the RAN Paging Area as signalled over Xn includes a mapped cell ID, or a Uu cell ID.

The mapping between Cell Identities and geographical areas is configured in the RAN and Core Network.

Editor’s note:	It is assumed that the method how the gNB derive the Cell Identity provided to the CN is a matter of implementation and configuration, and it is FFS whether this fact requires normative or informative specification text.