3GPP TSG-RAN WG3 Meeting #112-e	R3-212841

# Online meeting, 17th  – 27th  May 2021

Agenda Item:	18.2

Source:	Ericsson

Title:	Framework for RAN intelligence

Document for:	Discussion

# TP to TR 37.817

## TP proposal #1

-----------------------------------Start of Changes-----------------------------------

4.1	High-level Principles

The following high level principles should be applied for AI-enabled RAN intelligence:

- The detailed AI/ML algorithms and models for use cases are out of RAN3 scope.

- The study focuses on AI/ML functionality and corresponding types of inputs/outputs.

- The input/output and the location of AI inference should be studied case by case.

- RAN3 should focus on the analysis of data needed at the training function from external functions, while the aspects of how the training function uses inputs to train a model are out of RAN3 scope

- The inference and training functions should be able to request, if needed, specific information to be used to execute or train the AI/ML algorithm and to avoid reception of unnecessary information. The nature of such information depends on the use case and may depend on the specific ML model.

- The inference function should signal the outputs of the model only to nodes that have explicitly requested them (e.g. via subscription), or nodes that are subject to actions based on the output from model inference.

- NG-RAN is prioritized; EN-DC is included in the scope. FFS on whether MR-DC should be down-prioritized.

- A general framework and workflow for AI/ML optimization should be defined and captured in the TR. The generalized workflow should not prevent to “think beyond” the workflow if the use case requires so.

## 4.2	Functional Framework

Figure 4.2-1: Functional Framework for RAN Intelligence

This section introduces the common terminologies related to the functional framework for RAN intelligence illustrated in Figure 4.2-1.

- Data Collection is a function that provide input data to Model training and Model inference functions. ML model specific pre-processing of data is not carried out in the Data Collection function.  
Examples of input data may include measurements from UEs or different network entities, performance feedback, ML model feedback/output.

  - Training Data: information needed for the ML model training/validation.

  - Inference Data: information needed as an input for the Model inference function to provide a corresponding output.

- Model Training is a function that performs the training of the ML model. The Model training function is also responsible for data preparation (e.g. data pre-processing and cleaning, formatting, and transformation of raw data).

- Model Inference is a function that provides ML model output (e.g. predictions or decisions). The Model inference function is also responsible for data preparation (e.g. data pre-processing and cleaning, formatting, and transformation of raw data).

- Actor is a function that receives the output from the Model inference function and triggers of performs corresponding actions. The Actor may trigger actions directed to other entities or to itself.

- Feedback: Information that may be needed to derive training or inference data or performance feedback.

-----------------------------------End of Changes-----------------------------------