3GPP TSG-RAN WG3 Meeting #113-e	R3-213165

# 16-26 Aug 2021, E-meeting

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.463 | CR | 0606 | rev | 2 | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.463 on the support of CP-UP separation for eNB and ng-eNB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_arch_evo_enh |  |  |  |  |  | Date: |  |  | 2021-08-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To introduce the support of CP-UP separation for eNB and ng-eNB |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the support of CP-UP separation for eNB and ng-eNB, including: - Capture the new logical entities to each occurrence of gNB-CU-CP and gNB-CU-UP for ng-eNB-CU and eNB CP-UP separation from section 1 to section 7. - all the exisiting E1 AP procedures could be also applied to both eNB and ng-eNB CP/UP separation, except for the IAB UP TNL Address Update procedure. - to add notes in sections 8 and 9 clarifying that each occurrence of gNB-CU-CP/gNB-CU-UP pair could also be applied to CP/UP separation for LTE eNB and NG-RAN eNB deployment scenario. - to reuse existing IEs with necessary updates to support ng-eNB/eNB CP/UP separation, e.g. to Update the semantics description and values (if needed) of the related IEs and values, to support the eNB CP-UP separation, including: - PDCP related parameters: - DRB ID, PDCP SN UL Size, PDCP SN DL Size, RLC mode, ROHC Parameters, max CID, ROHC Profiles, T-Reordering Timer, Discard Timer, UL Data Split Threshold, PDCP SN Size, EHC Parameters (EHC-CID-Length, drb-ContinueEHC-DL, drb-ContinueEHC-UL), PDCP Re-establishment, PDCP Data Recovery, Out Of Order Delivery, PDCP Status Report Indication - Security related parameters: Security Algorithm and User Plane Security Keys - DRB ID, PDCP SN UL Size, PDCP SN DL Size, RLC mode, ROHC Parameters, max CID, ROHC Profiles, T-Reordering Timer, Discard Timer, UL Data Split Threshold, PDCP SN Size, EHC-CID-Length, drb-ContinueEHC-DL, drb-ContinueEHC-UL, should refer to TS 36.331. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | eNB and ng-eNB CP-UP separation is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 1, 2, 3, 5, 8, 9.3.1.16, 9.3.1.20, 9.3.1.31, 9.3.1.32, 9.3.1.38, 9.3.1.40, 9.3.1.41,9.3.1.42, 9.3.1.61, 9.3.1.90 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1: Capture the agreements of RAN3 #112-e Rev 2: BL CR to RAN3 #113-e |  |  |  |  |  |  |  |  |

1	Scope

The present document specifies the 5G radio network layer signalling protocol for the E1 interface. The E1 interface provides means for interconnecting a gNB-CU-CP and a gNB-CU-UP of a gNB within an NG-RAN, or for interconnecting a gNB-CU-CP and a gNB-CU-UP of an en-gNB within an E-UTRAN, or for interconnecting an eNB-CP and an eNB-UP of an eNB within an E-UTRAN, or for interconnecting an ng-eNB-CU-CP and an ng-eNB-CU-UP of an ng-eNB within an NG-RAN. The E1 Application Protocol (E1AP) supports the functions of E1 interface by signalling procedures defined in the present document. E1AP is developed in accordance to the general principles stated in TS 38.401 [2] and TS 38.460 [3].

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.401: "NG-RAN; Architecture Description".

[3]	3GPP TS 38.460: "NG-RAN; E1 general aspects and principles".

[4]	3GPP TS 38.300: "NR; Overall description; Stage-2".

[5]	3GPP TR 25.921 (version.7.0.0): "Guidelines and principles for protocol description and error".

[6]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[7]	ITU-T Recommendation X.691 (2002-07): "Information technology - ASN.1 encoding rules - Specification of Packed Encoding Rules (PER)".

[8]	ITU-T Recommendation X.680 (07/2002): "Information technology – Abstract Syntax Notation One (ASN.1): Specification of basic notation".

[9]	ITU-T Recommendation X.681 (07/2002): "Information technology – Abstract Syntax Notation One (ASN.1): Information object specification".

[10]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol Specificaiton”.

[11]	3GPP TS 23.401: “General Packet Radio Service (GPRS) Enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access”.

[12]	3GPP TS 23.203: “Policy and Charging Control Architecture”.

[13]	3GPP TS 33.501: “Security Architecture and Procedures for 5G System”.

[14]	IETF RFC 5905: “Network Time Protocol Version 4: Protocol and Algorithms Specification”.

[15]	3GPP TS 29.281: “General Packet Radio System (GPRS) Tunnelling Protocol User Plane (GTPv1-U)”.

[16]	3GPP TS 38.414: “NG-RAN; NG Data Transport”.

[17]	3GPP TS 38.323: "NR; Packet Data Convergence Protocol (PDCP) specification".

[18]	3GPP TS 38.462: "NG-RAN; E1 Signalling Transport".

[19]	3GPP TS 37.340: "NR; Multi-connectivity; Overall description; Stage-2".

[20]	3GPP TS 23.501: "System Architecture for the 5G System".

[21]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC) protocol specification".

[22]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[23]	3GPP TS 23.003: "Numbering, addressing and identification".

[24]	3GPP TS 32.422: "Trace control and configuration management".

[25]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[26]	3GPP TS 32.425: "Performance measurements; Evolved Universal Terrestrial Radio Access Network (E-UTRAN)".

[27]	3GPP TS 37.320: "Universal Terrestrial Radio Access (UTRA) and Evolved Universal Terrestrial Radio Access (E-UTRA); Radio measurement collection for Minimization of Drive Tests (MDT);Overall description; Stage 2".

[28]	3GPP TS 38.474: "NG-RAN; F1 data transport".

[x1]	3GPP TS 37.470: "W1 interface; General aspects and principles".

[x2]	3GPP TS 36.401: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Architecture description".

[x4]	3GPP TS 33.401: "3GPP System Architecture Evolution (SAE); Security architecture".

[x5]	3GPP TS 36.331: "Radio Resource Control (RRC); Protocol specification".

[x6]	3GPP TS 36.323: " Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Packet Data Convergence Protocol (PDCP) specification".

3	Definitions and abbreviations

3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Elementary Procedure: E1AP consists of Elementary Procedures (EPs). An Elementary Procedure is a unit of interaction between gNB-CU-CP and gNB-CU-UP, or between eNB-CP and eNB-UP, or between ng-eNB-CU-CP and ng-eNB-CU-UP. These Elementary Procedures are defined separately and are intended to be used to build up complete sequences in a flexible manner. If the independence between some EPs is restricted, it is described under the relevant EP description. Unless otherwise stated by the restrictions, the EPs may be invoked independently of each other as standalone procedures, which can be active in parallel. The usage of several E1AP EPs together is specified in stage 2 specifications (e.g., TS 38.460 [3]).

An EP consists of an initiating message and possibly a response message. Two kinds of EPs are used:

-	Class 1: Elementary Procedures with response (success and/or failure).

-	Class 2: Elementary Procedures without response.

For Class 1 EPs, the types of responses can be as follows:

Successful:

-	A signalling message explicitly indicates that the elementary procedure successfully completed with the receipt of the response.

Unsuccessful:

-	A signalling message explicitly indicates that the EP failed.

-	On time supervision expiry (i.e., absence of expected response).

Successful and Unsuccessful:

-	One signalling message reports both successful and unsuccessful outcome for the different included requests. The response message used is the one defined for successful outcome.

Class 2 EPs are considered always successful.

Conditional handover: as defined in TS 38.300 [4].

Conditional PSCell Change: as defined in TS 37.340 [19].

DAPS Handover: as defined in TS 38.300 [4].

eNB-CP: as defined in TS 36.401 [x2].

eNB-UP: as defined in TS 36.401 [x2].

gNB: as defined in TS 38.300 [4].

gNB-CU: as defined in TS 38.401 [2].

gNB-DU: as defined in TS 38.401 [2].

gNB-CU-CP: as defined in TS 38.401 [2].

gNB-CU-UP: as defined in TS 38.401 [2].

ng-eNB-CU: as defined in TS 37.470 [x1].

ng-eNB-CU-CP: as defined in TS 38.401 [2].

ng-eNB-CU-UP: as defined in TS 38.401 [2].

ng-eNB-DU: as defined in TS 37.470 [x1].

PDU Session Resource: as defined in TS 38.401 [2].

UE-associated signalling: When E1AP messages associated to one UE uses the UE-associated logical E1-connection for association of the message to the UE in gNB-CU-UP and gNB-CU-CP, or in eNB-CP and eNB-UP, or in ng-eNB-CU-CP and ng-eNB-CU-UP.

UE-associated logical E1-connection: The UE-associated logical E1-connection uses the identities GNB-CU-CP UE E1AP ID and GNB-CU-UP UE E1AP ID according to the definition in TS 38.401 [2]. For a received UE associated E1AP message the gNB-CU-CP or eNB-CP or ng-eNB-CU-CP identifies the associated UE based on the GNB-CU-CP UE E1AP ID IE and the gNB-CU-UP identifies the associated UE based on the GNB-CU-UP UE E1AP ID IE.

Public Network Integrated NPN: as defined in TS 23.501 [20].

Stand-alone Non-Public Network: as defined in TS 23.501 [20].

Next change

5	E1AP services

E1AP provides the signalling service between the gNB-CU-CP and the gNB-CU-UP, or between the eNB-CP and the eNB-UP, or between the ng-eNB-CU-CP and the ng-eNB-CU-UP that is required to fulfil the E1AP functions described in clause 7. E1AP services are divided into two groups:

Non UE-associated services:	They are related to the whole E1 interface instance between the gNB-CU-CP and gNB-CU-UP, or between the eNB-CP and eNB-UP, or between the ng-eNB-CU-CP and ng-eNB-CU-UP utilising a non UE-associated signalling connection.

UE-associated services:	They are related to one UE. E1AP functions that provide these services are associated with a UE-associated signalling connection that is maintained for the UE in question.

Unless explicitly indicated in the procedure specification, at any instance in time one protocol endpoint shall have a maximum of one ongoing E1AP procedure related to a certain UE.

Next change

8	E1AP procedures

NOTE 1:	The  procedures listed in this section should also be applied to CP/UP separation for eNB and ng-eNB, except for the IAB UP TNL Address Update procedure, if not stated otherwise. With this understanding, in this section each instance of gNB-CU-CP could be treated as eNB-CP or ng-eNB-CU-CP, and each gNB-CU-UP could be treated as eNB-UP or ng-eNB-CU-UP, for eNB or ng-eNB CP/UP separation respectively.

Editor’s note: It is FFS whether to introduce ECGI.

Next change

8.3.11	MR-DC Data Usage Report

8.3.11.1	General

This procedure is initiated by the gNB-CU-UP to report data volume served at the gNB-CU-UP, where the UE is connected to the 5GC or EPC. The procedure uses UE-associated signalling.

Next change

9	Elements for E1AP communication

NOTE 1:	In this section, each occurance of gNB-CU-CP could be replaced by eNB-CP or ng-eNB-CU-CP, and each occurance of gNB-CU-UP could be replaced by eNB-UP or ng-eNB-CU-UP, for eNB CP-UP separation and ng-eNB CP-UP separation respectively.

Editor’s note: It is FFS whether to introduce ECGI. If ECGI has to be introduced, ECGI Support List IE should be added to the related messages.

Editor’s note: Updates in other parameters are FFS.

Next change

9.3.1.16	DRB ID

This IE uniquely identifies a DRB for a UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| DRB ID | M |  | INTEGER (1.. 32, ...) | Corresponds to the DRB-Identity defined in TS 38.331 [10] for the gNB/ ng-eNB CP-UP separation, or in TS 36.331 [x5] for the eNB CP-UP separation. |

Next change

9.3.1.20	Bit Rate

This IE indicates the number of bits delivered by NG-RAN/E-UTRAN in UL or to NG-RAN/E-UTRAN in DL within a period of time, divided by the duration of the period. It is used, for example, to indicate the maximum or guaranteed bit rate for a GBR QoS flow or a GBR bear, or an aggregated maximum bit rate.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Bit Rate | M |  | INTEGER (0.. 4,000,000,000,000,…) | The unit is: bit/s |

Next change

9.3.1.31	Security Algorithm

This IE defines the type of ciphering algorithm and/or integrity protection used for the DRBs.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Ciphering Algorithm | M |  | ENUMERATED (NEA0, 128-NEA1, 128-NEA2, 128-NEA3) | As defined in TS 33.501 [13] for NG-RAN or TS 33.401 [x4] for E-UTRAN where the corresponding enumerated value is EEA0, 128-EEA1, 128-EEA2, 128-EEA3. |
| Integrity Protection Algorithm | O |  | ENUMERATED (NIA0, 128-NIA1, 128-NIA2, 128-NIA3) | As defined in TS 33.501 [13] for NG-RAN for NG-RAN or TS 33.401 [x4] for E-UTRAN where the corresponding enumerated value is EEA0, 128-EEA1, 128-EEA2, 128-EEA3. |

9.3.1.32	User Plane Security Keys

This IE contains the ciphering and/or integrity protection keys generated by the gNB-CU-CP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Encryption Key | M |  | OCTET STRING | As defined in TS 33.501 [13] for gNB or ng-eNB CP-UP separation, or in TS 33.401 [x4] for eNB CP-UP separation. |
| Integrity Protection Key | O |  | OCTET STRING | As defined in TS 33.501 [13] for NG-RAN. |

Next change

9.3.1.38	PDCP Configuration

This IE carries the PDCP configuration.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDCP SN UL Size | M |  | PDCP SN Size 9.3.1.61 | Indicates the PDCP SN UL size in bits. For more information see PDCP-Config IE in TS 38.331 [10] for gNB or ng-eNB CP-UP separation, or in TS 36.331 [x5] for eNB CP-UP separation. Is ignored if received through DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message. | - | - |
| PDCP SN DL Size | M |  | PDCP SN Size 9.3.1.61 | Indicates the PDCP SN DL size in bits. For more information see PDCP-Config IE in TS 38.331 [10] for gNB or ng-eNB CP-UP separation, or in TS 36.331 [x5] for eNB CP-UP separation. Is ignored if received through DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message. | - | - |
| RLC mode | M |  | ENUMERATED (RLC-TM, RLC-AM, RLC-UM-Bidirectional, RLC-UM-Unidirectional-UL, RLC-UM-Unidirectional-DL, …) | Indicates the RLC mode for the DRB. For more information see PDCP-Config IE in TS 38.331 [10] for gNB or ng-eNB CP-UP separation, or in TS 36.331 [x5] for eNB CP-UP separation. Is ignored if received through DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message. | - | - |
| ROHC Parameters | O |  | 9.3.1.40 |  | - | - |
| T-Reordering Timer | O |  | 9.3.1.41 |  | - | - |
| Discard Timer | O |  | 9.3.1.42 |  | - | - |
| UL Data Split Threshold | O |  | 9.3.1.43 |  | - | - |
| PDCP Duplication | O |  | ENUMERATED (True, …) | Indicates whether PDCP duplication is to be configured for the DRB. This IE is ignored when the “Additional PDCP duplication Information” IE is present. | - | - |
| PDCP Re-establishment | O |  | ENUMERATED (true,…) | Indicates PDCP entity re-establishment to be triggered as defined in TS 38.323 [17] for gNB or ng-eNB CP-UP separation, or in TS 36.323 [x6] for eNB CP-UP separation. | - | - |
| PDCP Data Recovery | O |  | ENUMERATED (true,…) | Indicates PDCP data recovery to be triggered as defined in TS 38.323 [17] for gNB or ng-eNB CP-UP separation, or in TS 36.323 [x6] for eNB CP-UP separation. | - | - |
| Duplication Activation | O |  | ENUMERATED ( Active, Inactive, …) | Information on the initial state of DL PDCP duplication | - | - |
| Out Of Order Delivery | O |  | ENUMERATED (true,…) | Indicates whether or not outOfOrderDelivery specified in TS 38.323 [17] is configured for gNB or ng-eNB CP-UP separation, or indicates whether or not rlc-OutOfOrderDelivery in TS 36.323 [x6] is configured for eNB CP-UP separation. Out of order delivery is configured only when the radio bearer is established. | - | - |
| PDCP Status Report Indication | O |  | ENUMERATED (downlink, uplink, both, …) | For AM DRB, “downlink” indicates that the PDCP entity is configured to send PDCP status report(s) to the UE, and “uplink” indicates that the UE is configured to send PDCP status report(s), as specified in TS 38.323 [17]. for gNB or ng-eNB CP-UP separation, or in TS 36.323 [x6] for eNB CP-UP separation “both” indicates that both “downlink” and “uplink” should be applied. | YES | ignore |
| Additional PDCP duplication Information | O |  | ENUMERATED (three, four, …) | Indicates the number of PDCP duplication configured when it is more than 2 for the DRB | YES | ignore |
| EHC Parameters | O |  | 9.3.1.90 |  | YES | ignore |

Next change

9.3.1.40	ROHC Parameters

This IE carries the ROCH parameters for header compressions.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Choice ROHC Parameters | M |  |  | For more information see PDCP-Config IE in TS 38.331 [10] for gNB or ng-eNB CP-UP separation, or in TS 36.331 [x5] for eNB CP-UP separation. |
| >ROHC |  |  |  |  |
| >>max CID | M |  | INTEGER (0..16383) | See description of maxCID inTS 38.331 [10] for gNB or ng-eNB CP-UP separation, or in TS 36.331 [x5] for eNB CP-UP separation. |
| >>ROHC Profiles | M |  | INTEGER (0..511) | Bitmap with supported UE profiles, bit 0 (LSB 0) = profile0x0001, bit 1 = profile0x0002, bit 2 = profile0x0003, bit 3 = profile0x0004, bit 4 = profile0x0006, bit 5 = profile0x0101, bit 6 = profile0x0102, bit 7 = profile0x0103, bit 8 = profile0x0104. See description of supportedROHC-Profiles in PDCP-Parameters in TS 38.331 [10] for gNB or ng-eNB CP-UP separation, or in TS 36.331 [x5] for eNB CP-UP separation. |
| >>Continue ROHC | O |  | ENUMERATED (true, …) | See description of drb-ContinueROHC inTS 38.331 [10] |
| >uplinkOnlyROHC |  |  |  |  |
| >>max CID | M |  | INTEGER (0..16383) | See description of maxCID inTS 38.331 [10] for gNB or ng-eNB CP-UP separation, or in TS 36.331 [x5] for eNB CP-UP separation. |
| >>ROHC Profiles | M |  | INTEGER (0..511) | Bitmap with supported UE profiles, bit 4 = profile0x0006. See description of supportedROHC-Profiles in PDCP-Parameters in TS 38.331 [10] for gNB or ng-eNB CP-UP separation, or in TS 36.331 [x5] for eNB CP-UP separation. |
| >>Continue ROHC | O |  | ENUMERATED (true, …) | See description of drb-ContinueROHC inTS 38.331 [10] |

9.3.1.41	T-Reordering Timer

This IE indicates the t-Reordering timer.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| T-Reordering Timer | M |  | ENUMERATED (0, 1, 2, 4, 5, 8, 10, 15, 20, 30, 40, 50, 60, 80, 100, 120, 140, 160, 180, 200, 220, 240, 260, 280, 300, 500, 750, 1000, 1250, 1500, 1750, 2000, 2250, 2500, 2750, 3000, …) | Indicates the t-Reordering UL timer. The values are expressed in ms. For more information see PDCP-Config IE in TS 38.331 [10] for gNB or ng-eNB CP-UP separation, or in TS 36.331 [x5] for eNB CP-UP separation. |

9.3.1.42	Discard Timer

This IE indicates PDCP discard timer.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Discard Timer |  |  | ENUMERATED (10, 20, 30, 40, 50, 60, 75, 100, 150, 200, 250, 300, 500, 750, 1500, Infinity) | Indicates the PDCP discard timer. The values are expressed in ms. For more information see PDCP-Config IE in TS 38.331 [10] for gNB or ng-eNB CP-UP separation, or in TS 36.331 [x5] for eNB CP-UP separation. |

9.3.1.43	UL Data Split Threshold

This IE indicates UL data split threshold.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UL Data Split Threshold |  |  | ENUMERATED (0, 100, 200, 400, 800, 1600, 3200, 6400, 12800, 25600, 51200, 102400, 204800, 409600, 819200, 1228800, 1638400, 2457600, 3276800, 4096000, 4915200, 5734400, 6553600, Infinity, …) | Indicates the UL data split threshold. The values are expressed in bytes. For more information see PDCP-Config IE in TS 38.331 [10] for gNB or ng-eNB CP-UP separation, or in TS 36.331 [x5] for eNB CP-UP separation. |

Next change

9.3.1.61	PDCP SN Size

This IE carries the PDCP SN Size.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PDCP SN Size | M |  | ENUMERATED (s-12, s-18, …, s-7) | Indicates the PDCP SN size in bits. For more information see PDCP-Config IE in TS 38.331 [10] for gNB or ng-eNB CP-UP separation, or in TS 36.331 [x5] for eNB CP-UP separation. |

Next change

9.3.1.90	EHC Parameters

This IE carries the EHC parameters for ethernet header compression.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| EHC Common | M |  |  |  |
| >EHC-CID-Length | M |  | ENUMERATED { bits7, bits15, … } | See description of ehc-CID-Length in TS 38.331 [10] for gNB or ng-eNB CP-UP separation, or in TS 36.331 [x5] for eNB CP-UP separation. |
| EHC Downlink | O |  |  |  |
| >drb-ContinueEHC-DL | M |  | ENUMERATED { true, … } | See description of drb-ContinueEHC-DL in TS 38.331 [10] for gNB or ng-eNB CP-UP separation, or in TS 36.331 [x5] for eNB CP-UP separation. |
| EHC Uplink | O |  |  |  |
| >drb-ContinueEHC-UL | M |  | ENUMERATED { true, … } | See description of drb-ContinueEHC-UL in TS 38.331 [10] for gNB or ng-eNB CP-UP separation, or in TS 36.331 [x5] for eNB CP-UP separation. |

End of changes