3GPP TSG-RAN WG3 #113-e						R3-213283

E-meeting, 16– 26 August 2021

Agenda Item:  14.2

Source:       NEC

Title:		SCG Activation / deactivation discussion on E1

Document for:	dicision

- Introduction

This contribution discuss the RAN3 signalling impact on support for activation/deactivation for one SCG, mainly focus on E1 impact.

RAN3#112e: RAN3 does not enhance Activity Notification for the sake of supporting SCG (de)activation for the MN initiated SCG (de)activation. WA: RAN3 does not enhance Activity Notification for the sake of supporting SCG (de)activation for the SN initiated SCG (de)activation. F1 interface enhancement to support SCG (de)activation reuses the principle in Xn interface regarding: - Codepoint design for SCG (de)activation for UE context setup - Whether/how DU can reject the SCG (de)activation during UE context setup procedure - Whether/how DU can reject the SCG (de)activation during UE context modification procedure WA:E1 interface enhancement to support SCG (de)activation is needed to let CU-UP be aware of the SCG state. RAN3 supports SCG (de)activation during SN initiated SN modification. In the SN addition request message, to set SCG (de)activated, two codepoints are supported (i.e. one for SCG activation, another for SCG deactivation). A new cause value will be introduced to indicate the reason to reject SCG (de)activation. FFS what exactly value. WA: For SCG (de)activation during MN initiated SN modification, SN can reject the SCG (de)activation when accepting SN modification request. Open issue: Which node is exactly responsible for the SCG activity detection and if any enhancement to E1/F1/Xn interface is needed. To be continued…

- 2. Discussion

Last RAN3#112e meeting reached an Working Assumption for E1 that:

WA:E1 interface enhancement to support SCG (de)activation is needed to let CU-UP be aware of the SCG state.

In this contribution we continue to discuss how E1 work to support SCG activation/deactivation function.

- 2.1 how to let SN-CU-UP know the SCG activated / deactivated status

Today E1AP Bearer Context Setup and Bearer Context Modification procedures, it has already a mean to let the CU-UP know the bearer context is “suspend”, “resume” from the Bearer Context Status Change IE.

It can be easily to reuse this existing Bearer Context Status Change IE to let the SN-CU-UP to know the status of bearer context whether it is “SCG-deactivated”.

Proposal 1: to reuse the existing Bearer Context Status Change IE to let the SN-CU-UP to know the status of bearer context by adding a new code point “SCG-deactivated”.

| Bearer Context Status Change | O |  | ENUMERATED (Suspend, Resume, …, SCG-deactivated) | Indicates the status of the Bearer Context | YES | reject |
| --- | --- | --- | --- | --- | --- | --- |

Whether need also the code-point “SCG-activated” in Bearer Context Status Change IE?

Even if the status of the bearer context is “SCG-deactivated”, the bearer context itself is always “activated” as the gNB-CU-UP needs to prepare to receive data whenever after the bearer context and tunnel are established. Indeed this is by default the existing CU-UP behaviour. This will mean that there is no need for the SN-CU-CP to ask the SN-CU-UP to “activate” the bearer. Therefore it is proposed:

Proposal 2: there is no need to add code point “SCG-activated” as the CU-UP needs to receive data whenever after bearer context and tunnels are established.

- 2.2 How to let SN-CU-UP to assist the SN-CU-CP

The SN-CU-UP is informed the status of bearer context “SCG-deactivated”. Then if any DL data is received in the SN-CU-UP, the E1AP existing DL Data Notification message can be reused, for the SN-CU-UP to inform the DL data is arrived, so the SN-CU-CP can activate the SCG.

Proposal 3: for the DL data arrival during SCG-deactivated state, the E1AP existing DL Data Notification procedure as it is can be reused, for the SN-CU-UP to inform the DL data is arrived, so the SN-CU-CP can activate the SCG.

For the UL data arrival during SCG-inactivated state, it can be the gNB-DU to detect first therefore no need to have E1 enhancement for the UL data detection during SCG-inactivated state.

Proposal 4: for the UL data arrival during SCG-inactivated state, there is no need to have E1 enhancement as it can rely on gNB-DU in F1 procedure.

- 2.4 Failure handling in SN-CU-UP

If the SN-CU-CP inform the SN-CU-UP that the bearer context is “SCG-deactivated” but the SN-CU-UP for any reason does not accept e.g. does not support the function, then it needs to let the SN-CU-CP know because the SN-CU-CP behaviour will be different. Therefore it is proposed:

Proposal 5: If any failure happens in SN-CU-UP during Bearer Context Setup procedure or gNB-CU-CP initiated Bearer Context Modification procedure, simply just send back the failure message with an appropriate cause value.

- 3. proposal

Proposal 1: to reuse the existing Bearer Context Status Change IE to let the SN-CU-UP to know the status of bearer context by adding a new code point “SCG-deactivated”.

Proposal 2: there is no need to add code point “SCG-activated” as the CU-UP needs to receive data whenever after bearer context and tunnels are established.

Proposal 3: for the DL data arrival during SCG-deactivated state, the E1AP existing DL Data Notification procedure as it is can be reused, for the SN-CU-UP to inform the DL data is arrived, so the SN-CU-CP can activate the SCG.

Proposal 4: for the UL data arrival during SCG-inactivated state, there is no need to have E1 enhancement as it can rely on gNB-DU in F1 procedure.

Proposal 5: If any failure happens in SN-CU-UP during Bearer Context Setup procedure or gNB-CU-CP initiated Bearer Context Modification procedure, simply just send back the failure message with an appropriate cause value.

Proposal 6: the proposed change in the Annex to be the base for baseline CR of 38.463.

References:

RP-201040 WID on Further Multi-RAT Dual-Connectivity enhancements [LTE_NR_DC_enh2-Core]

R3-207003 Summary of Offline Discussion on SCG (de)activation

R3-211132 Summary of Offline Discussion on MRDC2 - SCG (de)activation

R3-212784 Summary of offline discussion on SCG activation and deactivation

Annex: base content for baseline CR for 38.463

- 9.2.2.1	BEARER CONTEXT SETUP REQUEST

This message is sent by the gNB-CU-CP to request the gNB-CU-UP to setup a bearer context.

Direction: gNB-CU-CP  gNB-CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP UE E1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| Security Information | M |  | 9.3.1.10 |  | YES | reject |
| UE DL Aggregate Maximum Bit Rate | M |  | Bit Rate 9.3.1.20 |  | YES | reject |
| UE DL Maximum Integrity Protected Data Rate | O |  | Bit Rate 9.3.1.20 | The Bit Rate is a portion of the UE’s Maximum Integrity Protected Data Rate, and is enforced by the gNB-CU-UP node. | YES | reject |
| Serving PLMN | M |  | PLMN Identity 9.3.1.7 |  | YES | ignore |
| Activity Notification Level | M |  | 9.3.1.67 |  | YES | reject |
| UE Inactivity Timer | O |  | Inactivity Timer 9.3.1.54 | Included if the Activity Notification Level is set to UE. | - | - |
| Bearer Context Status Change | O |  | ENUMERATED (Suspend, Resume, …, SCG-deactived) | Indicates the status of the Bearer Context | YES | reject |
| CHOICE System | M |  |  |  | YES | reject |
| >E-UTRAN |  |  |  |  |  |  |
| >>DRB To Setup List | M |  | DRB To Setup List E-UTRAN 9.3.3.1 |  | YES | reject |
| >>Subscriber Profile ID for RAT/Frequency priority | O |  | 9.3.1.69 |  | YES | ignore |
| >>Additional RRM Policy Index | O |  | 9.3.1.70 |  | YES | Ignore |
| >NG-RAN |  |  |  |  |  |  |
| >>PDU Session Resource To Setup List | M |  | 9.3.3.2 |  | YES | reject |
| RAN UE ID | O |  | OCTET STRING (SIZE(8)) |  | YES | ignore |
| gNB-DU ID | O |  | 9.3.1.65 | Included whenever it is known by the gNB-CU-CP | YES | ignore |
| Trace Activation | O |  | 9.3.1.68 |  | YES | ignore |
| NPN Context Information | O |  | 9.3.1.84 |  | YES | reject |
| Management Based MDT PLMN List | O |  | MDT PLMN List 9.3.1.89 |  | YES | ignore |
| CHO Initiation | O |  | ENUMERATED (True, …) |  | YES | reject |
| Additional Handover Information | O |  | ENUMERATED(Discard PDCP SN, …) | If set to “Discard PDCP SN”, indicates that the forwarded PDCP SNs have to be removed | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs for a UE. Value is 32. |
| maxnoofPDUSessionResource | Maximum no. of PDU Sessions for a UE. Value is 256. |

- 9.2.2.4	BEARER CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU-CP to request the gNB-CU-UP to modify a bearer context.

Direction: gNB-CU-CP  gNB-CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP UE E1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-CU-UP UE E1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Security Information | O |  | 9.3.1.10 |  | YES | reject |
| UE DL Aggregate Maximum Bit Rate | O |  | Bit Rate 9.3.1.20 |  | YES | reject |
| UE DL Maximum Integrity Protected Data Rate | O |  | Bit Rate 9.3.1.20 | The Bit Rate is a portion of the UE’s Maximum Integrity Protected Data Rate, and is enforced by the gNB-CU-UP node. | YES | reject |
| Bearer Context Status Change | O |  | ENUMERATED (Suspend, Resume, …, SCG-deactivaed) | Indicates the status of the Bearer Context | YES | reject |
| New UL TNL Information Required | O |  | ENUMERATED (required, …) | Indicates that new UL TNL information has been requested to be provided. | YES | reject |
| UE Inactivity Timer | O |  | Inactivity Timer 9.3.1.54 | Included if the Activity Notification Level is set to UE. | - | - |
| Data Discard Required | O |  | ENUMERATED (required, …) | Indicate to discard the DL user data in case of RAN paging failure. | YES | ignore |
| CHOICE System | O |  |  |  | YES | reject |
| >E-UTRAN |  |  |  |  |  |  |
| >>DRB To Setup List | O |  | DRB To Setup Modification List E-UTRAN 9.3.3.7 |  | YES | reject |
| >>DRB To Modify List | O |  | DRB To Modify List E-UTRAN 9.3.3.8 |  | YES | reject |
| >>DRB To Remove List | O |  | DRB To Remove List E-UTRAN 9.3.3.9 |  | YES | reject |
| >>Subscriber Profile ID for RAT/Frequency priority | O |  | 9.3.1.69 |  | YES | ignore |
| >>Additional RRM Policy Index | O |  | 9.3.1.70 |  | YES | ignore |
| >NG-RAN |  |  |  |  |  |  |
| >>PDU Session Resource To Setup List | O |  | PDU Session Resource To Setup Modification List 9.3.3.10 |  | YES | reject |
| >>PDU Session Resource To Modify List | O |  | 9.3.3.11 |  | YES | reject |
| >>PDU Session Resource To Remove List | O |  | 9.3.3.12 |  | YES | reject |
| RAN UE ID | O |  | OCTET STRING (SIZE(8)) |  | YES | ignore |
| gNB-DU ID | O |  | 9.3.1.65 |  | YES | ignore |
| Activity Notification Level | O |  | 9.3.1.67 |  | YES | ignore |

#### -- B

BearerContextStatusChange	::= 	ENUMERATED {

suspend,

resume,

...,

sCG-deactivated

}