3GPP TSG-RAN WG3 #113-e						R3-213285

E-meeting, 16 – 26 August 2021

Agenda Item:  14.3.

Source:       NEC

Title:		(TP for CPAC BL CR to TS 38.423 ) Discussion on CPAC (Conditional PSCell Addition and PSCell Change)

Document for:	dicision

- Introduction

This contribution give TP (Text Proposal) for CPAC BL CR to TS 38.423, as discusses in R3-213284.

(as sometime it may be difficult to see which tiny parts have been modified, the TP from this paper are highlighted in green color)

(TP for CPAC BL CR to TS 38.423 )

- 8.3.5	S-NG-RAN node initiated S-NG-RAN node Change

- 8.3.5.1	General

This procedure is used by the S-NG-RAN node to trigger the change of the S-NG-RAN node.

The procedure uses UE-associated signalling.

- 8.3.5.2	Successful Operation

Figure 8.3.5.2-1: S-NG-RAN node initiated S-NG-RAN node Change, successful operation.

The S-NG-RAN node initiates the procedure by sending the S-NODE CHANGE REQUIRED message to the M-NG-RAN node including the Target S-NG-RAN node ID IE. When the S-NG-RAN node sends the S-NODE CHANGE REQUIRED message, it shall start the timer TXnDCoverall.

The S-NODE CHANGE REQUIRED message may contain

-	the S-NG-RAN node to S-NG-RAN node Container IE.

If the M-NG-RAN node is able to perform the change requested by the S-NG-RAN node, the M-NG-RAN node shall send the S-NODE CHANGE CONFIRM message to the S-NG-RAN node. For DRBs configured with the PDCP entity in the S-NG-RAN node, the M-NG-RAN node may include data forwarding related information in the Data Forwarding Info from target NG-RAN node IE.

If the S-NODE CHANGE CONFIRM message includes the DRB IDs taken into use IE, the S-NG-RAN node shall, if applicable, act as specified in TS 37.340 [8].

The S-NG-RAN node may start data forwarding and stop providing user data to the UE and shall stop the timer TXnDCoverall upon reception of the S-NODE CHANGE CONFIRM message.

If the Conditional PSCell Change Information Required IE is included in the S-NODE CHANGE REQUIRED message, the M-NG-RAN node shall consider that the requirement concerns CPAC, as described in TS 37.340 [8]. Accordingly, the M-NG-RAN node shall include the Conditional PSCell Change Information Confirm IE in the S-NODE CHANGE CONFIRM message.

*** Skip unchanged ***

- 9.1.2.2	S-NODE ADDITION REQUEST ACKNOWLEDGE

This message is sent by the S-NG-RAN node to confirm the M-NG-RAN node about the S-NG-RAN node addition preparation.

Direction: S-NG-RAN node  M-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| PDU Session Resources Admitted To Be Added List |  | 1 |  |  | YES | ignore |
| >PDU Session Resources Admitted To Be Added Item |  | 1 .. <maxnoofPDUSessions> |  | NOTE: If neither the PDU Session Resource Setup Response Info – SN terminated IE nor the PDU Session Resource Setup Response Info – MN terminated IE is present in a PDU Session Resources Admitted to be Added Item IE, abnormal conditions as specified in clause 8.3.1.4 apply. | – |  |
| >>PDU Session ID | M |  | 9.2.3.18 |  | – |  |
| >>PDU Session Resource Setup Response Info – SN terminated | O |  | 9.2.1.6 |  | – |  |
| >>PDU Session Resource Setup Response Info – MN terminated | O |  | 9.2.1.8 |  | – |  |
| PDU Session Resources Not Admitted List | O |  |  |  | YES | ignore |
| >PDU Session Resources Not Admitted List – SN terminated | O |  | PDU Session Resources Not Admitted List 9.2.1.3 |  | – |  |
| >PDU Session Resources Not Admitted List – MN terminated | O |  | PDU Session Resources Not Admitted List 9.2.1.3 |  | – |  |
| S-NG-RAN node to M-NG-RAN node Container | M |  | OCTET STRING | Includes the CG-Config message as defined in subclause 11.2.2 of TS 38.331 [10]. | YES | reject |
| Admitted Split SRBs | O |  | ENUMERATED (srb1, srb2, srb1&2, ...) | Indicates admitted SRBs | YES | reject |
| RRC Config Indication | O |  | 9.2.3.72 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| Location Information at S-NODE | O |  | Target Cell Global ID 9.2.3.25 | Contains information to support localisation of the UE | YES | ignore |
| MR-DC Resource Coordination Information | O |  | 9.2.2.33 | Information used to coordinate resource utilisation between M-NG-RAN node and S-NG-RAN node. | YES | ignore |
| Available fast MCG recovery via SRB3 | O |  | ENUMERATED (true, ...) | Indicates the fast MCG recovery via SRB3 isenabled. | YES | ignore |
| Conditional PSCell Addition Information Acknowledge | O |  |  |  | YES | ignore |
| >CPAC Candidate PSCells Ack List |  | 1 |  |  |  |  |
| >>CPAC Candidate PSCells Ack Item |  | 1 .. <maxnoofPSCellCandidate> |  |  |  |  |
| >>>PSCell ID | M |  | Target Cell Global ID 9.2.3.25 |  |  |  |
| >>>RRC Container | M |  | OCTET STRING | Includes RRCReconfiguration message as defined in subclause 6.2.2 of TS 38.331[10]. FFS whether single or multiple RRC containers. |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions. Value is 256 |
| maxnoofPSCellCandidate | Maximum no, of PSCell candidate. Value is FFS |

Skip unchanged

- 9.1.2.11	S-NODE CHANGE REQUIRED

This message is sent by the S-NG-RAN node to the M-NG-RAN node to trigger the change of the S-NG-RAN node.

Direction: S-NG-RAN node  M-NG-RAN node.

| IE/Group Name |  |  |  | Presence |  |  |  | Range |  |  |  | IE type and reference |  |  |  | Semantics description |  |  |  | Criticality |  |  |  | Assigned Criticality |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Message Type |  |  |  | M |  |  |  |  |  |  |  | 9.2.3.1 |  |  |  |  |  |  |  | YES |  |  |  | reject |  |  |  |
| M-NG-RAN node UE XnAP ID |  |  |  | M |  |  |  |  |  |  |  | NG-RAN node UE XnAP ID 9.2.3.16 |  |  |  | Allocated at the M-NG-RAN node |  |  |  | YES |  |  |  | reject |  |  |  |
| S-NG-RAN node UE XnAP ID |  |  |  | M |  |  |  |  |  |  |  | NG-RAN node UE XnAP ID 9.2.3.16 |  |  |  | Allocated at the S-NG-RAN node |  |  |  | YES |  |  |  | reject |  |  |  |
| Target S-NG-RAN node ID |  |  |  | M |  |  |  |  |  |  |  | Global NG-RAN Node ID 9.2.2.3 |  |  |  |  |  |  |  | YES |  |  |  | reject |  |  |  |
| Cause |  |  |  | M |  |  |  |  |  |  |  | 9.2.3.2 |  |  |  |  |  |  |  | YES |  |  |  | ignore |  |  |  |
| PDU Session SN Change Required List |  |  |  |  |  |  |  | 0..1 |  |  |  |  |  |  |  |  |  |  |  | YES |  |  |  | ignore |  |  |  |
| >PDU Session SN Change Required Item |  |  |  |  |  |  |  | 1 .. <maxnoofPDUsessions> |  |  |  |  |  |  |  | NOTE: If the PDU Session Resource Change Required Info – SN terminated IE is not present in a PDU Session SN Change Required Item IE, abnormal conditions as specified in clause 8.3.5.4 apply. |  |  |  | – |  |  |  |  |  |  |  |
| >>PDU Session ID |  |  |  | M |  |  |  |  |  |  |  | 9.2.3.18 |  |  |  |  |  |  |  | – |  |  |  |  |  |  |  |
| >>PDU Session Resource Change Required Info – SN terminated |  |  |  | O |  |  |  |  |  |  |  | 9.2.1.18 |  |  |  |  |  |  |  | – |  |  |  |  |  |  |  |
| S-NG-RAN node to M-NG-RAN node Container |  |  |  | M |  |  |  |  |  |  |  | OCTET STRING |  |  |  | Includes the CG-Config message as defined in subclause 11.2.2 of TS 38.331 [10]. |  |  |  | YES |  |  |  | reject |  |  |  |
| Conditional PSCell Change Information Required |  |  |  | O |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | YES |  |  |  | reject |  |  |  |
| >CPAC Indicator |  |  |  | M |  |  |  |  |  |  |  | ENUMERATED (CPAC-initiation, ...) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| >Maximum Number of PSCells To Prepare |  |  |  | O |  |  |  |  |  |  |  | INTEGER (1..FFS, …) |  |  |  | Indicates the maximum number of PSCells that the target SN may prepare. |  |  |  |  |  |  |  |  |  |  |  |
| >CPAC Candidate PSCells REQD List |  |  |  |  |  |  |  | 0..1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| >>CPAC Candidate PSCells REQD Item |  | 1 .. <maxnoofPSCellCandidate> |  |  |  |  |
| >>>PSCell ID |  |  |  | M |  |  |  |  |  |  |  | NR CGI 9.2.2.7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| >>>RRC Container |  |  |  | M |  |  |  |  |  |  |  | OCTET STRING |  |  |  | Includes CPCExecutionCondition as defined in subclause 6.2.x of TS 38.331 [10]. |  |  |  |  |  |  |  |  |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPSCellCandidate | Maximum no, of PSCell candidate. Value is FFS |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUsessions | Maximum no. of PDU sessions. Value is 256 |

- 9.1.2.12	S-NODE CHANGE CONFIRM

This message is sent by the M-NG-RAN node to inform the S-NG-RAN node that the preparation of the S-NG-RAN node initiated S-NG-RAN node change was successful.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name |  |  |  | Presence |  |  |  | Range |  |  |  | IE type and reference |  |  |  | Semantics description |  |  |  | Criticality |  |  |  | Assigned Criticality |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Message Type |  |  |  | M |  |  |  |  |  |  |  | 9.2.3.1 |  |  |  |  |  |  |  | YES |  |  |  | reject |  |  |
| M-NG-RAN node UE XnAP ID |  |  |  | M |  |  |  |  |  |  |  | NG-RAN node UE XnAP ID 9.2.3.16 |  |  |  | Allocated at the M-NG-RAN node |  |  |  | YES |  |  |  | ignore |  |  |
| S-NG-RAN node UE XnAP ID |  |  |  | M |  |  |  |  |  |  |  | NG-RAN node UE XnAP ID 9.2.3.16 |  |  |  | Allocated at the S-NG-RAN node |  |  |  | YES |  |  |  | ignore |  |  |
| PDU Session SN Change Confirm List |  |  |  |  |  |  |  | 0..1 |  |  |  |  |  |  |  |  |  |  |  | YES |  |  |  | ignore |  |  |
| >PDU Session SN Change Confirm Item |  |  |  |  |  |  |  | 1 .. <maxnoof PDUsessions> |  |  |  |  |  |  |  | NOTE: If the PDU Session Resource Change Confirm Info – SN terminated IE is not present in a PDU Session SN Change Confirm Item IE, abnormal conditions as specified in clause 8.3.5.4 apply. |  |  |  | – |  |  |  |  |  |  |
| >>PDU Session ID |  |  |  | M |  |  |  |  |  |  |  | 9.2.3.18 |  |  |  |  |  |  |  | – |  |  |  |  |  |  |
| >>PDU Session Resource Change Confirm Info – SN terminated |  |  |  | O |  |  |  |  |  |  |  | 9.2.1.19 |  |  |  |  |  |  |  | – |  |  |  |  |  |  |
| Criticality Diagnostics |  |  |  | O |  |  |  |  |  |  |  | 9.2.3.3 |  |  |  |  |  |  |  | YES |  |  |  | ignore |  |  |
| Conditional PSCell Change Information Confirm |  |  |  | O |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | YES |  |  |  | ignore |  |  |
| >CPAC Candidate PSCells CONF List |  |  |  |  |  |  |  | 0..1 |  |  |  |  |  |  |  | the SN that initiated CPC is informed of the candidate PSCell IDs. |  |  |  |  |  |  |  |  |  |  |
| >>CPAC Candidate PSCells CONF Item |  | 1 .. <maxnoofPSCellCandidate> |  |  |  |  |
| >>>PSCell ID |  |  |  | M |  |  |  |  |  |  |  | NR CGI 9.2.2.7 |  |  |  |  |  |  | – |  |  |  |  |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPSCellCandidate | Maximum no, of PSCell candidate. Value is FFS |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUsessions | Maximum no. of PDU sessions. Value is 256 |

*** Skip unchanged ***

CPACcandidatePSCellsAck-list ::= SEQUENCE (SIZE(1..maxnoofPSCellCandidates)) OF CPACcandidatePSCellsAck-item

CPACcandidatePSCellsAck-item ::= SEQUENCE {

pscell-id						NR-CGI,

rrc-container					RRCContainer (FFS),

iE-Extensions					ProtocolExtensionContainer { {CPACcandidatePSCellsAck-item-ExtIEs}}	OPTIONAL,

...

}

CPACcandidatePSCellsAck-item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

CPACcandidatePSCellsREQD-list ::= SEQUENCE (SIZE(1..maxnoofPSCellCandidates)) OF CPACcandidatePSCellsREQD-item

CPACcandidatePSCellsREQD-item ::= SEQUENCE {

pscell-id						NR-CGI,

rrc-container					RRCContainer,

iE-Extensions					ProtocolExtensionContainer { {CPACcandidatePSCellsREQD-item-ExtIEs}}	OPTIONAL,

...

}

CPACcandidatePSCellsREQD-item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

CPACindicator ::= ENUMERATED {cpac-initiation, ...}

CPCindicator ::= ENUMERATED {cpc-triggered, ...}

CPAInformationRequest ::= SEQUENCE {

cpac-indicator					CPACindicator,

max-no-of-pscells				INTEGER (1..maxnoofPSCellCandidates)	OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { CPAInformationRequest-ExtIEs} } OPTIONAL,

...

}

CPAInformationRequest-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

CPAInformationAck ::= SEQUENCE {

candidate-pscells					CPACcandidatePSCellsAck-list,

iE-Extensions		ProtocolExtensionContainer { { CPAInformationAck-ExtIEs} } OPTIONAL,

...

}

CPAInformationAck-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

CPCInformationRequired::= SEQUENCE {

cpac-indicator					CPACindicator,

max-no-of-pscells				INTEGER (1..maxnoofPSCellCandidates)	OPTIONAL,

candidate-pscells				CPACcandidatePSCellsREQD-list,

iE-Extensions		ProtocolExtensionContainer { {CPCInformationRequired-ExtIEs} } OPTIONAL,

...

}

CPCInformationRequired-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

CPCInformationConfirm ::= SEQUENCE {

candidate-pscells				CPACcandidatePSCellsCOFN-list,

iE-Extensions		ProtocolExtensionContainer { { CPCInformationConfirm-ExtIEs} } OPTIONAL,

...

}

CPCInformationConfirm-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

CPACcandidatePSCellsCOFN -list ::= SEQUENCE (SIZE(1..maxnoofPSCellCandidates)) OF CPACcandidatePSCellsCOFN-item

CPACcandidatePSCellsCOFN-item ::= SEQUENCE {

pscell-id						NR-CGI,

rrc-container					RRCContainer,

iE-Extensions					ProtocolExtensionContainer { {CPACcandidatePSCellsCOFN-item-ExtIEs}}	OPTIONAL,

...

}

CPACcandidatePSCellsCOFN-item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}