3GPP TSG-RAN WG3 Meeting #113-e															R3-213399

Electronic Meeting, August 16 – 27, 2021

Agenda item:	19.2.1

Source: 	Qualcomm Incorporated

Title: 	Signalling and Procedures for supporting Positioning Reference Units

Document for:		Discussion and Decision

# 1	Introduction

At RAN1#104-e the following agreement has been made [1]:

Agreement: - Study specification impact for enabling a reference device with known location to support the following functionalities: - Measure DL PRS and report associated measurements (e.g., RSTD, Rx-Tx time difference, RSRP) to the LMF; - Transmit SRS and enable TRPs to measure and report measurements (e.g., RTOA, Rx-Tx time difference, AOA) associated with the reference device to the LMF; - FFS: The details of the signalling, the measurements, the parameters related to the Rx and Tx timing delays, AoD and AOA enhancements and measurement calibrations; - FFS: The report of device location coordinate information to the LMF if the LMF does not have the information - FFS: The device with the known location being a UE and/or a gNB - FFS: Precision to which location of reference device is known - Note: RAN1 assumes using these enhancements for the purpose of network synchronization is NOT within the scope of the WI

RAN1 observed benefits by using positioning reference units for enhancing the positioning performance. An LS was sent to RAN2/3 with the following agreement [2]:

Agreement: Send an LS to RAN2/RAN3 (cc SA2), including the following content: - RAN1 has evaluated the use of positioning reference units (PRUs) with known locations for positioning and observes improvements in using PRUs for enhancing the positioning performance. But, RAN1 has not identified specification enhancements needed in RAN1 specifications. RAN1 kindly requests RAN2/RAN3 (cc SA2) to determine if and what specification enhancements are adopted for PRUs for positioning. - Notes: - The term "positioning reference unit (PRU)" is only used as a terminology in this discussion. PRU does not necessarily mean an introduction of a new network node. - PRU may support, at least, some of the Rel-16 positioning functionalities of UE, if agreed, which is up to RAN2. The positioning functionalities may include, but not limited to, the following: - Provide the positioning measurements (e.g., RSTD, RSRP, Rx-Tx time differences) - Transmit the UL SRS signals for positioning - PRU may be requested by the LMF to provide its own known location coordinate information to the LMF. If the antenna orientation information of the PRU is known, the information may also be requested by the LMF.

The actions for RAN2/3 are as follows [2]:

"RAN1 kindly requests RAN2/RAN3 to determine if and what specification enhancements are adopted for the use of PRUs for positioning."

In this contribution, we study the specification impacts to support "Positioning Reference Units (PRUs)" with known location in a network.

# 2	Positioning Reference Units (PRUs)

A Positioning Reference Unit (PRU) at a known location can perform DL positioning measurements (e.g., RSTD, RSRP, UE Rx-Tx Time Difference measurements, etc.) and report these measurements to a location server. In addition, the PRU can transmit UL-PRS (SRS) to enable TRPs to measure and report UL positioning measurements from devices at known location (e.g., RTOA, UL-AoA, gNB Rx-Tx Time Difference, etc.). The PRU measurements can be compared with the "true" measurements expected at the known device position to determine correction terms for other nearby UEs. The DL- and/or UL location measurements for other UEs can then be corrected based on the previously determined correction terms. This principle is well known from e.g., Differential-GNSS operation.

The PRUs perform positioning measurements just like a normal UE, but at an a-priori known location. Therefore, the PRU-terminated positioning protocols could be the same protocols as used for normal UE positioning.

Observation 1:	The PRU-terminated positioning protocols could be the same protocols as used for normal UE positioning.

# 3.	UE Positioning Operations

The general UE positioning operation is summarized in clause 5.2 of TS 38.305 [3] and clause 6.11 of TS 23.273 [4] and is shown in Figure 1 below. At Step 1, either some entity in the 5GC (e.g., a GMLC), or the UE, or the AMF itself (e.g. to locate the UE for an emergency call) requests some location services from an AMF. The AMF then transfers the location request to an LMF at Step 2 in Figure 1. The LMF may then invoke one or more LPP and/or NRPPa positioning procedures with the UE and/or NG-RAN at Step 3. Once the LMF has determined the UE location based on the measurement information received in Step 3, the LMF provides the location estimate to the AMF at Step 4, which then provides the UE location to the requesting client at Step 5.

A prerequisite for Step 3 is that a LCS Correlation identifier and the AMF identity has been passed to the LMF by the serving AMF, as described in TS 23.273 [4] (e.g., clause 6.11.1). The LCS Correlation ID and the AMF ID are provided by the AMF to the LMF in the Location Service Request at Step 2 in Figure 1 (Nlmf_Location_DetermineLocationRequest service operation [5] towards the LMF). When the LMF then instigates Step 3 using an (UE associated) Namf_Communication_N1N2MessageTransfer service operation [6] towards the AMF with a LPP or NRPPa PDU included, the LMF also includes the LCS Correlation ID for this location session, together with the AMF ID which indicates the AMF instance serving the UE.

The LMF uses the AMF instance to forward LPP/NRPPa messages to the UE/NG-RAN.

The LCS Correlation identifier is used to ensure that during a positioning session between the LMF and UE, positioning response messages from the UE are returned by the AMF to the correct LMF and carrying an indication (the LCS Correlation identifier) which can be recognized by the LMF.

Figure 1: Location Service Support by NG-RAN (according to TS 38.305 [3]).

The PRU location measurements are used by an LMF to correct the measurements for target UEs. That is, the consumer of the PRU location information is an LMF, and therefore, Steps 1 and 2 in Figure 1 would not happen for PRUs.

This essentially means, the LMF becomes a "LCS Client" for PRUs and need to be enabled to instigate location sessions with a PRU in absence of Steps 1/2 in Figure 1.

Observation 2:	For PRUs, an AMF/LMF would not receive a location request from an LCS client. Instead, the location client for PRU measurements would be the LMF itself.

# 4.	PRU Positioning Operations

As a consequence of Observation 2, Steps 1/2 in Figure 1 would have to be replaced by a "PRU Registration" procedure to make an LMF aware of PRUs in a network such that the LMF is enabled to instigate a LPP or NRPPa session with the desired PRU or NG-RAN serving the PRU, respectively, as illustrated in Figure 2.

The "PRU Registration" procedure at Step 1 in Figure 2 should enable an LMF to instigate the positioning procedures at Step 3 in the same way as currently specified for target UEs.

Figure 2: Location Service Support for PRUs.

As also mentioned in the RAN1 agreement in section 1, there are two principle architecture options for supporting PRUs in a network as shown in Figure 3:

Option 1: The PRU is considered as UE from LMF perspective;

Option 2: The PRU is considered as part of a gNB (e.g., gNB-DU, enhanced TRP, etc.) from LMF perspective.

Both options are discussed and evaluated in the following.

Figure 3: Positioning Architecture with PRUs.

# 5.	PRU Registration Procedure

## 5.1	Option 1: PRU operates as a UE from LMF perspective

If the PRU can be regarded as a "UE" from LMF perspective, two general PRU registration procedures seem possible, as described in section 5.1.1 and 5.1.2 below.

### 5.1.1	Solution 1: PRU Access Registration

In this solution, the PRU registers to the gNB and the 5G Core Network like a normal UE. As part of this registration procedure, the UE provides an indication to the serving AMF whether the device can function as a Position Reference Unit. The serving AMF then registers the PRU at an LMF using a new PRU Registration Request service operation towards an LMF. The PRU Registration operation enables an LMF to exchange LPP and NRPPa messages between LMF and PRU and NG-RAN, respectively, at a later time when the LMF requires location measurements from a (registered) PRU.

The procedure is shown in Figure 4 below.  Steps 1-11 are the same general procedure as used for UE registration with the gNB and the 5G Core Network with the addition of Steps 4 and 5.

Figure 4: PRU Positioning Operation (Solution 1).

1.	The PRU and NG-RAN Node perform the 5G-NR RRC Connection Setup procedure (like a normal UE). At the end of this procedure, the PRU sends the RRC Setup Complete message with a "Registration Request" in the dedicatedNAS-Message field to the NG-RAN Node.

The NAS Registration Request includes the 5GMM Capability IEI, as specified in TS 24.501 [7]. This IE currently includes an indication whether the UE supports LPP and the 5G-LCS notification mechanisms. An additional bit can be allocated to indicate whether the UE can operate as Positioning Reference Unit.

2.	The gNB selects the AMF and allocates a "RAN UE NGAP ID". The AMF will use this ID to address the PRU context on the gNB. 
The gNB then sends the NGAP Initial UE Message to the selected AMF. The message carries the "Registration Request" message that was received from the PRU in the RRC Setup Compete message in Step 1. The "RAN UE NGAP ID" and the "RRC Establishment Cause" are also included in the message.

3.	The PRU and 5GC perform the NAS Authentication and Security procedures.

4.	If the 5GMM Capability IEI received at Step 2 indicates support for "PRU functionality", the AMF invokes a "PRU Registration Request" service operation towards the selected LMF to register the PRU at the LMF. The service operation includes a LCS Correlation identifier and the AMF identity, together with the serving cell identity and may also include the SUPI and PEI of the PRU.

5.	If the LMF can handle the PRU Registration Request, the LMF returns an acknowledgement to the AMF in a PRU Registration Response service operation towards the AMF including the LCS Correlation identifier received at Step 4.

The LMF stores the LCS Correlation Identifier and the AMF ID for later use in Steps 13.

6.	The AMF, NG-RAN, and PRU continue the general registration procedure as for normal UEs. The AMF may initiate a session setup with the gNB. The message typically contains the Registration Accept NAS message. The NAS Registration Accept may include an indication that the PRU registration at an LMF was successful.

7.	The PRU and gNB perform the AS Security Procedures.

8.	A RRC Reconfiguration message is typically sent to the PRU for setting up radio bearers, etc. The message includes the 5GS Registration Result received in the Registration Accept message at Step 6. The 5GS Registration Result informs the PRU whether the registration at an LMF was successful or not.

9.-11. The remaining steps to complete a normal registration procedure.

12.	At some later time, the LMF determines that location measurements from a PRU previously registered at the LMF are needed and may instigate a LPP and/or NRPPa session with the PRU or the NG-RAN serving the PRU at Steps 13a and 13b, respectively.

13.	The procedures at Step 13a and 13b can now be executed as currently defined for normal UEs and specified in e.g., TS 23.273 [4], clause 6.11.1 and 6.11.2, and TS 38.305 [3], clause 6.4.2 and 6.5.2

The first step in these procedures comprise a Namf_Communication_N1N2MessageTransfer service operation as specified in TS 29.518 [6] towards the AMF which previously registered the PRU at the LMF for this PRU. This service operation may include a LPP PDU which should be transferred to the PRU, or a NRPPa PDU which should be transferred to the serving NG-RAN Node of the PRU.

This service operation includes the LCS Correlation ID and the AMF ID provided to the LMF previously at Step 4 and stored at the LMF. The AMF instance belonging to the AMF ID/LCS Correlation ID is used to forward the LPP/NRPPa messages to the PRU/NG-RAN.

As part of Step 13a, the PRU provides the requested reference measurements (e.g., RSTD, RSRP, Rx-Tx Time Difference) in a LPP Provide Location Information to the LMF.

As part of Step 13b, the NG-RAN Node may provide an UL-PRS (SRS) configuration to the PRU and/or activate a UL-PRS configuration in the PRU. The PRU then transmits the UL-PRS which may be received by multiple gNBs/TRPs which perform UL positioning reference measurements (e.g., UL-AoA, RTOA, gNB Rx-Tx Time Difference measurements).

14.	The LMF determines correction terms from the received PRU measurements at Step 13 using the known location of the PRU. The LMF stores the correction terms and uses them to correct location measurements for other UEs.

The LMF repeats steps 13-14 whenever new or updated PRU measurements are needed.

For this solution, a new service operation is needed to allow an AMF to register a PRU at the LMF, as shown by Steps 4 and 5 in Figure 4. This operation may be part of the Nlmf_Location service operation specified in TS 29.572 [5], or may be a new e.g. Nlmf_ReferenceUnit service operation. In addition to the PRU Registration, a PRU Deregistration and PRU Registration Update service operation may also be needed. E.g.,

-	Positioning Reference Unit Registration: It provides Positioning Reference Unit Information to an LMF, including

-	LCS Correlation ID

-	AMF ID

-	ECGI, NCGI

-	PRU SUPI, PEI

-	Positioning Reference Unit Registration Update: It provides updated Positioning Reference Unit Information to an LMF for a previously registered Positioning Reference Unit.

-	For example, a new ECGI or NCGI in case the serving NG-RAN node changes; or PRU location changes, etc.

-	Positioning Reference Unit Deregistration: It enables an AMF to deregister a previously registered Positioning Reference Unit at an LMF.

-	For example, in case the serving AMF for a PRU changes, or PRU is switched-off, etc.

As mentioned above, the location procedures between LMF and PRU can be performed as for normal UEs.

NRPPa can be used as currently specified for UL- (e.g., UL-AoA, UL-TDOA) and UL+DL (e.g., Multi-RTT) positioning methods in TS 38.305 [3] and no changes to the NRPPa protocol [10] seem required.

Also LPP [11] can be used as currently defined, however, some PRU specific additions seem required/useful.

##### LPP Capability Transfer:

The positioning method capabilities (e.g., for UL-TDOA, UL-AoA, DL-TDOA, DL-AoD, or Multi-RTT, etc.) would equally apply to PRUs. They provide positioning specific capabilities supported by the PRU to the LMF (e.g., supported measurements, etc.).

The LPP Provide Capabilities message may include in addition the location (latitude/longitude/altitude together with uncertainty shape) and antenna orientation information of the PRU, and additional attributes such as whether the PRU is static/fixed or mobile, or whether the PRU is battery powered or mains-operated, etc.

##### LPP Assistance Data Transfer

No PRU specific additions seem required for the assistance data delivery. The normal (UE-assisted) positioning method specific assistance data would equally apply to PRUs.

##### Location Information Transfer

The positioning method specific request/provide messages are equally applicable to PRUs for requesting and providing the UE positioning measurements (e.g., NR-Multi-RTT-ProvideLocationInformation, NR-DL-AoD-ProvideLocationInformation, NR-DL-TDOA-ProvideLocationInformation [11], etc.). However, some additions to the CommonIEsRequestLocationInformation seem useful/required.

The CommonIEsRequestLocationInformation carries common IEs for a Request Location Information LPP message Type incl. a (mandatory present) LocationInformationType, which indicates whether the server requires a location estimate or measurements, whether location measurements are preferred but location estimate is also allowed, or whether location estimate is preferred but location measurements are also allowed:

LocationInformationType ::= ENUMERATED {

locationEstimateRequired,

locationMeasurementsRequired,

locationEstimatePreferred,

locationMeasurementsPreferred,

...

}

A location estimate should not be applicable to PRUs, since the (precise) location of the reference unit need to be known and only location measurements should be reported. The Reference Unit Location and Reference Unit Measurements must be independent; i.e. Reference Unit Measurements are not used for determining the Reference Unit Location.

An additional entry in the LocationInformationType could avoid mixing 'location measurements' and 'location estimates' as used for normal UE positioning and would only be applicable to PRUs:

LocationInformationType ::= ENUMERATED {

locationEstimateRequired,

locationMeasurementsRequired,

locationEstimatePreferred,

locationMeasurementsPreferred,

...,

referenceMeasurementsRequired

}

### 5.1.2	Solution 2: PRU Registration using Supplementary Services

In this solution, the PRU registers with an LMF using a new Supplementary Services message pair. The LMF and PRU can then exchange LPP messages via the serving AMF.  This solution avoids any impacts to an AMF and gNB, and can also avoid any impacts to LPP. The general procedure is shown in Figure 5 below.

Figure 5: PRU Positioning Operation (Solution 2).

1.	The PRU performs a UE triggered service request as defined in clause 4.2.3.2 of TS 23.502 [8] if in CM-IDLE state in order to establish a signalling connection with the AMF.

2.	The PRU sends a Supplementary Services (SS) LCS Positioning Reference Unit Registration Request message inside a UL NAS Transport message as specified in TS 24.501 [7]  to the serving AMF.

The PRU sets the Payload Container Type to "Location Services Message Container" [7] and includes a Routing Identifier in the "Additional Information" IE [7].

The Routing Identifier in the UL NAS Transport message may be preconfigured in the PRU to indicate a specific LMF.

The LCS Positioning Reference Unit Registration Request message includes the location of the PRU (latitude, longitude, altitude, together with uncertainty shape), and other PRU capabilities, such as whether the device is fixed or mobile, battery powered, or mains-operated, etc. 

Alternatively, this information may also be provided in an LPP PDU embedded in the LCS Positioning Reference Unit Registration Request message.

However, providing this information as part of the SS LCS Positioning Reference Unit Registration Request message and specified in SS [13] no LPP changes may be required. The PRU and LMF can assume that all LPP messages exchanged during Steps 8 relate to the reference location of the PRU.

3.	The AMF determines the LMF from the Routing Identifier received in the Additional Information IE of the UL NAS Transport message in Step 2 and forwards the LCS Positioning Reference Unit Registration Request message to the LMF via triggering Namf_Communication_N1MessageNotify service operation towards the LMF. The AMF also includes the Payload Container Type and the LCS Correlation Identifier set to the Routing Identifier received at Step 2.

The AMF includes the SUPI or PEI of the PRU in the N1MessageNotification. 

The LMF stores the serving AMF ID included in the N1 Message Container for later use in Steps 8.

4.	The LMF may verify whether the PRU is allowed to act as a reference unit in the network from the SUPI or PEI.  The SUPI or PEI of allowed Reference Units in an LMF may be preconfigured.

5.	The LMF returns a supplementary services acknowledgement message to the PRU via triggering an Namf_Communication_N1N2MessageTransfer service operation to the serving AMF. The acknowledgement indicates to the PRU that it has been successfully registered at the LMF. The LMF also assigns and includes a LCS Correlation Identifier identifying the LMF.

6.	Upon receipt of the acknowledgement message from the LMF, the AMF sets the Payload Container Type in the DL NAS Transport to "Location Services Message Container" [7] and includes a LCS Correlation ID  in the "Additional Information" IE [7].

7.	At some later time, the LMF determines that location measurements from a PRU previously registered at the LMF are needed and may instigate a LPP and/or NRPPa session with the PRU or the NG-RAN serving the PRU at Steps 8a and 8b, respectively. Step 7 may be performed immediately following Registration. For example, the LMF could determine to initiate periodic reporting of location measurements from the PRU.

8.	The procedure at Step 8a and 8b can now be executed as Steps 13a/b in Solution 1 (Figure 4) with the following modifications:

The LCS Correlation Identifier indicating the LMF and positioning session is assigned by the LMF. The LCS Correlation identifier is used to ensure that during the positioning session between the LMF and PRU, positioning response messages from the PRU are returned by the AMF to the correct LMF and carrying an indication (the LCS Correlation identifier) which can be recognized by the LMF.

To enable an AMF to distinguish a LCS Correlation identifier assigned by an LMF (used in this procedure) from a LCS Correlation identifier assigned by the AMF, the two types of LCS Correlation identifier could be selected from different ranges.

NOTE:	This is similar as applied in the deferred 5GC-MT-LR for periodic, triggered and UE available location events, as specified in TS 23.273 [4], clause 6.3.1, Note 9.

9.	The LMF determines correction terms from the received PRU measurements using the known location of the PRU. The LMF stores the correction terms and uses them to correct location measurements from other UEs.

The LMF repeats steps 8 whenever new PRU measurements are needed.

If the PRU moves to a new AMF, the PRU can repeat the registration at Step 2.

Observation 3:	In the case the PRU is considered as a "UE" from LMF perspective, a "PRU Registration" procedure can enable an LMF to instigate the needed positioning procedures with a PRU in a similar way as currently specified for target UEs.

## 5.2	Option 2: PRU operates as part of a gNB from LMF perspective

If the PRU can be regarded as part of a gNB from LMF perspective (e.g., part of a gNB-DU, TRP), a PRU Registration Procedure as shown in Step 1 of Figure 2 is not strictly necessary. The "PRU Registration" would be performed by a deployment/operator, similar to the provisioning of gNB information to an LMF (typically via some Operation&Maintenance functionality). The PRUs could be part of the normal gNB/TRP data base in an LMF. Specific PRU information could be requested by an LMF using the Rel-16 TRP Information Exchange procedure [10] as for normal gNBs/TRPs.

The PRU could be defined as an "Enhanced TRP" (eTRP) which may be part of a gNB-DU or separate, as shown in Figure 3. The eTRP has essentially inverse functions compared to a Rel-16 TRP: The eTRP transmits UL-PRS (a Rel-16 TRP transmits DL-PRS), and receives DL-PRS (a Rel-16 TRP would receive UL-PRS).

One benefit of this solution is that Non UE associated NRPPa procedures can be used, which avoids the PRU Registration as mentioned above.

### 5.2.1	Solution 3: LPP transported inside NRPPa messages

Since LPP provides all the functionality for positioning capability transfer, assistance data delivery, and location information transfer, LPP can also be used if the PRU is part of a gNB. The LPP PDU could be transported inside a NRPPa message container. NRPPa would then enable the gNB-CU to perform message routing in agreement with Rel-16 gNB-LMF signalling principles. However, the actual positioning message payload can be a LPP PDU, as illustrated in Figure 6.

Figure 6: Protocol stack for the support of LPP for eTRPs.

The procedure for PRU capability exchange, assistance data delivery, and location information transfer via LPP is shown in Figure 7.

Figure 7: PRU Positioning Operation (Solution 3).

1. 	The LMF invokes the Namf_Communication_NonUeN2MessageTransfer service operation towards the AMF to request the transfer of a NRPPa PDU to a NG-RAN node (gNB-CU). The NRPPa message includes an embedded LPP PDU. The LPP PDU may request location information from the PRU, provide assistance data to the PRU or query for the PRU capabilities.
The service operation includes the target NG-RAN node identity.

2. 	The AMF forwards the NRPPa PDU (which includes an embedded LPP PDU) to the identified gNB-CU in an NGAP Downlink Non UE Associated NRPPa Transport message [9] and includes a Routing ID identifying the LMF.

3. 	The gNB-CU extracts the LPP PDU from the received NRPPa message and forwards the LPP PDU to the identified eTRP in a F1AP DL Transport container [14].

4.	The target eTRP (PRU) extracts the LPP PDU and decodes the LPP message. If the LPP message is of type Request Location Information, the eTRP obtains the requested location measurements (e.g., RSTD, RSRP, Rx-Tx Time Difference measurement).

5.	The target eTRP (PRU) returns the location information obtained in step 4 to the gNB-CU in an F1AP UL Transport container. The F1AP UL Transport container includes the LPP PDU.

6.	The target gNB-CU extracts the LPP PDU from the F1AP message and includes the LPP PDU in a NRPPa message. The gNB-CU then provides the NRPPa PDU (which includes the LPP PDU) to the AMF in a NGAP UL Non UE Associated NRPPa Transport message. The gNB-CU also includes the Routing identifier in the NGAP UL Non UE Associated NRPPa Transport message received in step 2.

7.	The AMF invokes the Namf_Communication_NonUeN2InfoNotify service operation towards the LMF indicated by the Routing Identifier received in step 6. The service operation includes the NRPPa PDU with the embedded LPP PDU received in step 6 in the N2 Info Container as defined in TS 29.518 [6].

8.	The LMF determines correction terms from the received PRU measurements using the known location of the PRU. The LMF stores the correction terms and uses them to correct location measurements from other UEs.

The LMF repeats steps 1-7 whenever new Reference Unit Measurements are needed.

This solution has minimal impact to the Rel-16 positioning procedures and signalling. The only additional impact would be to define a new NRPPa and F1AP message which can carry an embedded LPP PDU container.

For the transmission of UL-PRS (SRS), the Rel-16 NRPPa and F1AP procedures can also be reused. However, for the PDU transport between LMF and gNB, the corresponding messages would also be Non UE Associated, similar to Figure 7.

A procedure to request an SRS configuration/SRS Transmission from the eTRP reference unit to support UL positioning reference measurements is shown in Figure 8.

Figure 8: PRU SRS Configuration (Solution 3).

1. 	The LMF invokes the Namf_Communication_NonUeN2MessageTransfer service operation towards the AMF to request the transfer of a NRPPa PDU. The NRPPa PDU includes a Positioning Information Request message [10] to request eTRP (PRU) SRS configuration information. The message may include the Requested UL-SRS Transmission Characteristics as specified in TS 38.455 [10], which provides recommended SRS configuration parameter for the eTRP (PRU) .
The service operation includes the target NG-RAN node identity.

2. 	The AMF forwards the NRPPa PDU to the identified gNB-CU in an NGAP Downlink Non UE Associated NRPPa Transport message [9] and includes a Routing ID identifying the LMF.

3. 	Since the NRPPa PDU is received in a Non UE-Associated NRPPa Transport message by the gNB-CU, the gNB-CU realizes that the NRPPa Positioning Information Request message is intended for a target eTRP/PRU, and not for a normal UE. The NRPPa Positioning Information Request message includes the eTRP ID(s) for the target eTRP(s)/PRU(s).

The gNB-CU sends a F1AP Positioning Information Request message [14] to the eTRP(s)/PRU(s) (as indicated in the NRPPa message received in Step 2) to indicate to the eTRP(s)/PRU(s) the need to transmit SRS signals and to retrieve the SRS configuration from the eTRP/PRU(s).

4.	The eTRP/PRU takes the Requested SRS Transmission Characteristics information included in the F1AP Positioning Information Request message into account when determining the SRS configuration. 
If the Requested SRS Transmission Characteristics indicates a periodic SRS, the eTRP/PRU starts transmitting the SRS. Otherwise (e.g., in case of semi-persistent SRS), the eTRP/PRU waits with the transmission of the SRS until a F1AP Positioning Activation Request message is received.

5.	The target eTRP(s) (PRU(s)) return a F1AP Positioning Information Response message including the configured SRS configuration information to the gNB-CU.

6.	The target gNB-CU encodes the information received at Step 5 into a NRPPa Positioning Information Response message and provides the NRPPa PDU (including the NRPPa Positioning Information Response message) to the AMF in a NGAP UL Non UE Associated NRPPa Transport message. The gNB-CU also includes the Routing identifier in the NGAP UL Non UE Associated NRPPa Transport message received in step 2.

7.	The AMF invokes the Namf_Communication_NonUeN2InfoNotify service operation towards the LMF indicated by the Routing Identifier received in step 6. The service operation includes the NRPPa PDU with the NRPPa Positioning Information Response message received in step 6 in the N2 Info Container as defined in TS 29.518 [6].

The same procedure as above can be used for other NRPPa/F1AP messages, such as Positioning Activation Request/Response to activate semi-persistent or trigger aperiodic UL SRS transmission by the eTRP/PRU.

Therefore, the Rel-16 NRPPa and F1AP positioning messages can be reused for PRUs in the case the PRU is considered part of a gNB (gNB-DU/eTRP). Instead of using UE associated transport procedures for these messages, the Non UE associated transport mechanism would be used in this solution, which requires only small changes to the NRPPa/F1AP messages to include the identities of the target gNB-DU/eTRP.

Observation 4:	In the case the PRU is considered as part of a gNB from LMF perspective, explicit "PRU Registration" would not be needed and LPP could still be used for requesting/providing the 	reference location measurements.

# 6.	Comparison of Solutions

The main impacts of the three solutions discussed above are summarized in Table 1 below. More favourable characteristics in Table 1 are shown in green highlight.

Table 1: Comparison of Solutions.

|  | Solution 1 | Solution 2 | Solution 3 |
| --- | --- | --- | --- |
| Device impact | Small. Any MT (e.g., UE) which supports positioning and LPP can function as PRU with some small changes to the protocols. | Small. Any MT (e.g., UE) which supports positioning and LPP can function as PRU. Additional SS procedure need to be supported, but no changes to existing protocols. | Smallest. Any MT (e.g., UE) which supports positioning and LPP can function as PRU but no registration procedure need to be supported. |
| NG-RAN Node Impacts | None. | None. | Small. Additional "TRP Type" needs to be supported, but with similar functionality as existing TRPs. |
| AMF Impacts | High. A new service operation with associated functionality need to be supported. | None. Same as other LCS SS. | None. |
| LMF Impacts | Medium. PRU context need to be stored and location requests for PRUs need to be initiated, etc. | Medium. PRU context need to be stored and location requests for PRUs need to be initiated, etc. | Small. The PRU functionality need to be supported (e.g., determination of correction terms, etc.), but PRUs are handled like normal (existing) TRPs. |
| LPP Impacts | Small. Some PRU specific clarifications and additions are needed. | None. All PRU specific information can be specified in SS messages. | None. All PRU specific information can be specified in an NRPPa message header (if needed). |
| NRPPa Impacts | None | None. | Small. New NRPPa message pair, which however, need to include primarily an LPP container. |
| F1AP Impacts | None | None. | Small. New F1AP message pair, which however, need to include primarily an LPP container. |

Observation 5:	If the PRU is considered as a "UE" from LMF perspective, Solution 2 (using a new Supplementary Services message pair) has less overall impact compared to Solution 1 (using UE access 	registration procedures).

Observation 6:	With solution 3 (PRU is considered as part of a gNB), only a new NRPPa/F1AP message pair seem required, which however, need to include primarily an LPP message container.

# 7.	Summary

In this contribution, we discussed the signalling and procedures for supporting Positioning Reference Units (PRUs) in a network. The following was observed:

Observation 1:	The PRU-terminated positioning protocols could be the same protocols as used for normal UE positioning.

Observation 2:	For PRUs, an AMF/LMF would not receive a location request from an LCS client. Instead, the location client for PRU measurements would be the LMF itself.

Observation 3:	In the case the PRU is considered as a "UE" from LMF perspective, a "PRU Registration" 	procedure can enable an LMF to instigate the needed positioning procedures with a PRU in a similar way as currently specified for target UEs.

Observation 4:	In the case the PRU is considered as part of a gNB from LMF perspective, explicit "PRU Registration" would not be needed and LPP could still be used for requesting/providing the reference location measurements.

Observation 5:	If the PRU is considered as a "UE" from LMF perspective, Solution 2 (using a new Supplementary Services message pair) has less overall impact compared to Solution 1 (using UE access 	registration procedures).

Observation 6:	With Solution 3 (PRU is considered as part of a gNB), only a new NRPPa/F1AP message pair seem required, which however, need to include primarily an LPP message container.

Therefore, the following proposals are made:

Proposal 1:	The Positioning Reference Units (PRUs) use LPP as positioning protocol. All LPP positioning methods can be supported by a PRU, dependent on PRU capabilities.

Proposal 2:	RAN3 to discuss and decide whether the Positioning Reference Unit (PRU) is considered as part of a gNB from LMF point of view or not.

Proposal 3:	In the case the Positioning Reference Unit (PRU) is considered as part of a gNB from LMF point of view, specify a new NRPPa/F1AP message pair to transport an LPP container.

Proposal 4:	Sent an LS to RAN2 and SA2 with the RAN3 agreements.

# References

[1]		RP-210627, "Status Report to TSG: NR Positioning Enhancements".

[2]		R3-213109 (R1-2106326), "LS on Positioning Reference Units (PRUs) for enhancing positioning 						performance", RAN1.

[3]		3GPP TS 38.305: "Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[4]		3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[5]		3GPP TS 29.572: "Location Management Services; Stage 3".

[6]		3GPP TS 29.518: "Access and Mobility Management Services; Stage 3".

[7]		3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[8]		3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[9]		3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[10]	3GPP TS 38.455: "NR Positioning Protocol A (NRPPa)".

[11]	3GPP TS 37.355: "LTE Positioning Protocol (LPP)".

[12]	3GPP TS 24.571: "Control plane Location Services (LCS) procedures; Stage 3".

[13]	3GPP TS 24.080: "Mobile radio interface layer 3 supplementary services specification; Formats and coding".

[14]	3GPP TS 38.473: "F1 application protocol (F1AP)".