3GPP TSG-RAN WG3 Meeting #113-e	R3-213408

E-meeting, 16-26 Aug 2021

Title: 	(TP for SON BLCR for 38.423) MRO for SN Change Failure

Source: 	Huawei

Agenda item:	10.2.1.6

Document Type:	other

# 1. Introduction

The MRO for SN Change Failure was discussed during RAN3#112-e.

# 2. Background

In RAN3-112-e meeting, there were some progresses and open issues on the MRO for SN Change Failure.

A class 2 procedure is defined for transmitting SCGFailureInformation from the MN to the SN that caused the failure, unless class-1 is found needed to resolve the issue of intra-SN PSCell change.

Waiting for RAN2 on the contents in SCGFailureInformation.

FFS whether include the following IEs in the new XnAP message for carrying SCGfailureinformation:

a)	 PSCell failure type

b) Source PSCell CGI

c)	 Failed PSCell CGI

d) Suitable PSCell CGI

e) Mobility Information

f)	PSCell selection assistant information, e.g. UE history information

g) Initiating node type i.e. MN or SN

h) S-NG-RAN node UE X2AP ID

i) M-NG-RAN node UE XnAP ID

To be continued...

In this paper, we’ll give our considerations based on the agreements and open issues.

# 3. Discussion

## 3.1 Intra-SN PSCell change

Technically speaking, we believe that though the MN cannot know whether there was an intra-SN PSCell before the SCG failure, the MN can take into consideration the measurement results from the UE for further decision to identify the node that caused the failure. In our understanding, if the measurement reports indicate that the best cell is still part of the last serving SN, it is reasonable for the MN to determine that the SCG failure is related to the last serving SN. Otherwise, if a cell in another SN is the best, this means MN did not trigger inter-SN PSCell change in time. This is too late PSCell change from MN perspective, no matter whether there is an intra-SN PSCell change.

Proposal 1: The MN can depend on the measurement results to decide the node that caused the SCG failure if the MN cannot know whether there is an intra-SN PSCell change for pre-Rel-17 UE.

Therefore, the MN can know whether the SN caused the failure and it is also worth noting that a class 2 procedure is enough to transmit the SCGFailureInformation from the MN to the SN.

Proposal 2: Class 2 procedure is defined for transmitting SCGFailureInformation form the MN to the SN that caused the failure.

## 3.2 Information in the Xn

In RAN3-112-e meeting, there is an open issue on the potential information carried in the new XnAP message SCG Failure Information.

FFS whether include the following IEs in the new XnAP message for carrying SCGfailureinformation:

a)	 PSCell failure type

b) Source PSCell CGI

c)	 Failed PSCell CGI

d) Suitable PSCell CGI

e) Mobility Information

f)	PSCell selection assistant information, e.g. UE history information

g) Initiating node type i.e. MN or SN

h) S-NG-RAN node UE X2AP ID

i) M-NG-RAN node UE XnAP ID

Here we’ll give our analysis on the required IEs from the MN to the SN. Currently, when the UE detects SCG failure, it can send the SCGfailureinformation message to the MN. There are the latest measurement results in the SCGfailureinformation message.

In RAN3-111-e meeting, it was agreed that:

MN performs initial analysis to identify the node that caused the failure. The node that caused the failure performs root cause analysis.

In our understanding, upon receiving SCGFailureInformation message, MN can perform the similar initial analysis as the failure node in the MRO scheme. In MRO, the RLF report will first be sent to the failure node where the UE detected the RLF. The failure node can perform the initial analysis to decide the failure type and the right node to optimize the corresponding mobility parameters. Likewise, for the PSCell change failure, the MN can also decide the PSCell failure type, e.g., Too late PSCell change, Too early PSCell change or Triggering PSCell change to wrong PSCell, based on the time elapsed since the last PSCell change initialization until SCG failure and the possibly suitable cell(s). Based on the above analysis, the MN can identify the right S-NG-RAN node and send the SCGFailureInformation message to the initiating SN.

For the IEs b) Source PSCell CGI and c) Failed PSCell CGI, they may be beneficial for pre-Rel-17 UE if the MN can know these information and provide to the last serving SN.

Besides, upon the PSCell change failure, if the MN decides to continue the DC for the UE, it is highly anticipated that the MN selects a new target PSCell based on the measurement results in the SCGFailureInformation message. This can quickly recover the SCG configuration and ensure the UE experience compared to the SN decision solution.

If we wait for the source SN to re-decide the new target PSCell, it will bring additional latency due to the delivery of the SCGFailureInformation and the SN Modification Required messages. Besides, it is a bit questionable how the source SN can know whether the new PSCell belonging to a new SN is a suitable candidate for SCG connection, e.g. whether there is an Xn connection between the MN and the new SN. In this way, the MN can also send the new suitable target PSCell decision to the initiating SN node.

In other hand, if the MN decides not to continue the DC configuration for the UE. It depends on MN implementation whether to include the new suitable target PSCell in the XnAP message SCG Failure Information.

Finally, to retrieve more relevant information collected at the initiating SN side as part of the UE context, the MN provides the UE ID for the initiating SN to identify the UE. But in MR-DC, the MN does not know the C-RNTI of UE in the source SN. Therefore it is preferred to use the S-NG-RAN node UE X2AP ID as a reference to the UE context in the source SN.

According to the above analysis and consideration, it is preferred to include the following information:

Proposal 3: The XnAP message SCG Failure Information includes:

a) the PSCell failure type.

b) Source PSCell CGI

c) Failed PSCell CGI.

d) the suitable PSCell CGI.

h) S-NG-RAN node UE X2AP ID for the initiating SN to identify the UE.

Here we show a possible flow chart in case of too early PSCell change.

Fig. the SCG Failure Information

For the other remaining IEs, all of them are not needed. For the e) Mobility Information, if the SCG failure information is reported immediately, it is desirable that the failure SN node can keep the UE context, including the mobility information. For f) PSCell selection assistant information, e.g., UE history information, this is being discussed in the UE history information topic to support the report of the PSCell related UHI. Besides, there are the latest measurements results provided from the UE, which is workable for the SN node to perform the PSCell selection. For g) Initiating node type, if the failure is caused by the MN, no SCGFailureInformation will be delivered to the SN node. The transmission of the SCGFailureInformation is kind of implicit indicator of the initiating node type. For i) M-NG-RAN node UE XnAP ID, this is kind of UE ID to assist the failure SN node to identify the UE. If the h) S-NG-RAN node UE XnAP ID is provided, it is not needed to duplicately transmit i).

Proposal 4: Other IEs e), f), g) and i) will not be included in the XnAP message SCG Failure Information.

# 4. Conclusion

Based on the discussion in this paper, we propose the following:

Proposal 1: The MN can depend on the measurement results to decide the node that caused the SCG failure if the MN cannot know whether there is an intra-SN PSCell change for pre-Rel-17 UE.

Proposal 2: Class 2 procedure is defined for transmitting SCGFailureInformation form the MN to the SN that caused the failure.

Proposal 3: The XnAP message SCG Failure Information includes:

a) the PSCell failure type.

b) Source PSCell CGI

c) Failed PSCell CGI.

d) the suitable PSCell CGI.

h) S-NG-RAN node UE X2AP ID for the initiating SN to identify the UE.

Proposal 4: Other IEs e), f), g) and i) will not be included in the XnAP message SCG Failure Information.

# 5. Reference

- R3-206191, (TP for SON BLCR for 38.423) MRO for SN Change Failure, Huawei

# Annex – TP on TS 38.423

Start of the first change

## 8.1	Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs.

Table 8.1-1: Class 1 Elementary Procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |  |
| --- | --- | --- | --- | --- |
|  |  | Response message | Response message |  |
| Handover Preparation | HANDOVER REQUEST | HANDOVER REQUEST ACKNOWLEDGE | HANDOVER PREPARATION FAILURE |
| Retrieve UE Context | RETRIEVE UE CONTEXT REQUEST | RETRIEVE UE CONTEXT RESPONSE | RETRIEVE UE CONTEXT FAILURE |
| S-NG-RAN node Addition Preparation | S-NODE ADDITION REQUEST | S-NODE ADDITION REQUEST ACKNOWLEDGE | S-NODE ADDITION REQUEST REJECT |
| M-NG-RAN node initiated S-NG-RAN node Modification Preparation | S-NODE MODIFICATION REQUEST | S-NODE MODIFICATION REQUEST ACKNOWLEDGE | S-NODE MODIFICATION REQUEST REJECT |
| S-NG-RAN node initiated S-NG-RAN node Modification | S-NODE MODIFICATION REQUIRED | S-NODE MODIFICATION CONFIRM | S-NODE MODIFICATION REFUSE |
| S-NG-RAN node initiated S-NG-RAN node CHANGE | S-NODE CHANGE REQUIRED | S-NODE CHANGE CONFIRM | S-NODE CHANGE REFUSE |
| M-NG-RAN node initiated S-NG-RAN node Release | S-NODE RELEASE REQUEST | S-NODE RELEASE REQUEST ACKNOWLEDGE | S-NODE RELEASE REJECT |
| S-NG-RAN node initiated S-NG-RAN node Release | S-NODE RELEASE REQUIRED | S-NODE RELEASE CONFIRM |  |
| Xn Setup | XN SETUP REQUEST | XN SETUP RESPONSE | XN SETUP FAILURE |
| NG-RAN node Configuration Update | NG-RAN NODE CONFIGURATION UPDATE | NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE | NG-RAN NODE CONFIGURATION UPDATE FAILURE |
| Cell Activation | CELL ACTIVATION REQUEST | CELL ACTIVATION RESPONSE | CELL ACTIVATION FAILURE |
| Reset | RESET REQUEST | RESET RESPONSE |  |
| Xn Removal | Xn REMOVAL REQUEST | Xn REMOVAL RESPONSE | Xn REMOVAL FAILURE |
| E-UTRA - NR Cell Resource Coordination | E-UTRA - NR CELL RESOURCE COORDINATION REQUEST | E-UTRA - NR CELL RESOURCE COORDINATION RESPONSE |  |
| Resource Status Reporting Initiation | RESOURCE STATUS REQUEST | RESOURCE STATUS RESPONSE | RESOURCE STATUS FAILURE |
| Mobility Settings Change | MOBILITY CHANGE REQUEST | MOBILITY CHANGE ACKNOWLEDGE | MOBILITY CHANGE FAILURE |

Table 8.1-2: Class 2 Elementary Procedures

| Elementary Procedure | Initiating Message |
| --- | --- |
| Handover Cancel | HANDOVER CANCEL |
| SN Status Transfer | SN STATUS TRANSFER |
| RAN Paging | RAN PAGING |
| Xn-U Address Indication | XN-U ADDRESS INDICATION |
| S-NG-RAN node Reconfiguration Completion | S-NODE RECONFIGURATION COMPLETE |
| S-NG-RAN node Counter Check | S-NODE COUNTER CHECK REQUEST |
| UE Context Release | UE CONTEXT RELEASE |
| RRC Transfer | RRC TRANSFER |
| Error Indication | ERROR INDICATION |
| Notification Control Indication | NOTIFICATION CONTROL INDICATION |
| Activity Notification | ACTIVITY NOTIFICATION |
| Secondary RAT Data Usage Report | SECONDARY RAT DATA USAGE REPORT |
| Trace Start | TRACE START |
| Deactivate Trace | DEACTIVATE TRACE |
| Handover Success | HANDOVER SUCCESS |
| Conditional Handover Cancel | CONDITIONAL HANDOVER CANCEL |
| Early Status Transfer | EARLY STATUS TRANSFER |
| Failure Indication | FAILURE INDICATION |
| Handover Report | HANDOVER REPORT |
| Resource Status Reporting | RESOURCE STATUS UPDATE |
| Access And Mobility Indication | ACCESS AND MOBILITY INDICATION |
| SCG Failure Information | SCG FAILURE INFORMATION |

Start of the next change

### 8.4.x	SCG Failure Information

#### 8.4.x.1	General

The purpose of the SCG Failure Information procedure is to transfer SCG mobility related information from M-NG-RAN node to S-NG-RAN node.

The procedure uses non UE-associated signalling.

#### 8.4.x.2	Successful Operation

Figure 8.4.x.2-1: SCG Failure Information, successful operation

An M-NG-RAN node initiates the procedure by sending an SCG FAILURE INFORMATION message to S-NG-RAN node. By sending the message M-NG-RAN node indicates to S-NG-RAN node that a PSCell mobility-related problem was detected.

If the PSCell Change Failure Type IE is set to "too early PSCell change" or "triggering PSCell change to wrong PSCell", then the M-NG-RAN node indicates to S-NG-RAN node that, following a PSCell change from a source PSCell to a target PSCell, an SCG failure occurred and the UE established connection either at the original PSCell (too early PSCell change), or at another PSCell (triggering PSCell change to wrong PSCell). If the PSCell Change Failure Type IE is set to "too late PSCell change", then the M-NG-RAN node indicates to S-NG-RAN node that an SCG failure occurred and there was no recent PSCell change before the SCG failure.

#### 8.4.x.3	Unsuccessful Operation

Not applicable.

#### 8.4.x.4	Abnormal Conditions

Void.

Start of the next change

#### 9.1.3.x	SCG FAILURE INFORMATION

This message is sent by the M-NG-RAN node to report a PSCell change failure event to S-NG-RAN node.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| PSCell UE Context ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| CHOICE PSCell Change Failure Type | M |  |  |  |  |  |
| > too early PSCell change |  |  |  |  |  |  |
| >>Source PSCell CGI | M |  | NR CGI 9.2.2.7 | CGI of source PSCell for PSCell change procedure | YES | ignore |
| >>Failure PSCell CGI | M |  | NR CGI 9.2.2.7 | CGI of PSCell where the failure occurred | YES | ignore |
| > too late PSCell change |  |  |  |  |  |  |
| >>Failure PSCell CGI | M |  | NR CGI 9.2.2.7 | CGI of PSCell where the failure occured | YES | ignore |
| >>Suitable PSCell CGI | O |  | NR CGI 9.2.2.7 | CGI of the new suitable PSCell | YES | ignore |
| > triggering PSCell change to wrong PSCell |  |  |  |  |  |  |
| >>Source PSCell CGI | M |  | NR CGI 9.2.2.7 | CGI of source PSCell for PSCell change procedure | YES | ignore |
| >>Failure PSCell CGI | M |  | NR CGI 9.2.2.7 | CGI of PSCell where the failure occurred | YES | ignore |
| >>Suitable PSCell CGI | O |  | NR CGI 9.2.2.7 | CGI of the new suitable PSCell | YES | ignore |
| UE SCGfailureinformation Report Container | M |  | OCTET STRING | The received SCGFailureInformation or SCGFailureInformationEUTRA message defined in TS 38.331 [10] from UE. | YES | ignore |

End of the last change