3GPP TSG-RAN WG3 Meeting #113-e	R3-213445

Online, 16 – 26 August 2021

Agenda item:	19.2.1

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP for NR_pos_enh BL CR for TS 38.455) UL-AOA/ZOA assistance information

Document for:	Discussion and Decision

# 1	Introduction

RAN3 has received an LS from RAN1, requesting RAN3 to define UL-AOA/ZOA assistance signalling for NR positioning enhancements [1].

In this paper, we discuss the impacts of the RAN1 agreements on NRPPa and provide a text proposal.

# 2	Discussion

RAN1 has agreed that the LMF can provide Expected UL-AOA/ZOA information to assist the TRP in finding the best Rx beam for measurement, which can lead to improved accuracy.

The RAN1 agreements can be summarized as follows:

-	NR supports at least the following additional assistance signaling from LMF to gNB/TRP to facilitate UL measurements of UL-AOA:

-	Uncertainty range for expected UL AoA/ZoA is defined as follows:

-	Expected azimuth angle of arrival as (φAOA - ΔφAOA/2, φAOA + ΔφAOA/2)

φAOA - expected azimuth angle of arrival, and

ΔφAOA – uncertainty range for expected azimuth angle of arrival

-	Expected zenith angle of arrival as (θZOA – ΔθZOA/2, θZOA + ΔθZOA/2) where

θZOA - expected zenith angle of arrival

ΔθZOA – uncertainty range for expected zenith angle of arrival

-	Both GCS and LCS are supported for UL AoA/ZoA assistance information indication.

-	Granularity of 0.1 degrees is applied for the expected AoA (φAOA), expected ZoA (θZOA) and the corresponding uncertainty values.

-	LMF to gNB signaling of UL AoA/ZoA assistance information (expected value and uncertainty range) is supported for UL-TDOA and Multi-RTT positioning methods.

The above RAN1 agreements can be introduced into NRPPa as follows:

Proposal 1: 	Introduce the Expected UL Angle of Arrival IE in the MEASUREMENT REQUEST message, within the TRP Measurement Request List IE.

Proposal 2: 	The Expected UL Angle of Arrival IE includes the Expected Azimuth Angle of Arrival IE (mandatory), Expected Zenith Angle of Arrival IE (optional), and LCS to GCS Translation IE (optional).

Proposal 3: 	The Expected Azimuth Angle of Arrival IE includes both a Value component (mandatory) and an Uncertainty Range component (mandatory).

Proposal 4: 	If included, the Expected Zenith Angle of Arrival IE also includes both a Value component (mandatory) and an Uncertainty Range component (mandatory).

# 3	Conclusions

In this paper, the NRPPa impacts of the RAN1 agreements on UL-AOA/ZOA assistance information were analysed and the following proposed:

Proposal 1: 	Introduce the Expected UL Angle of Arrival IE in the MEASUREMENT REQUEST message, within the TRP Measurement Request List IE.

Proposal 2: 	The Expected UL Angle of Arrival IE includes the Expected Azimuth Angle of Arrival IE (mandatory), Expected Zenith Angle of Arrival IE (optional), and LCS to GCS Translation IE (optional).

Proposal 3: 	The Expected Azimuth Angle of Arrival IE includes both a Value component (mandatory) and an Uncertainty Range component (mandatory).

Proposal 4: 	If included, the Expected Zenith Angle of Arrival IE also includes both a Value component (mandatory) and an Uncertainty Range component (mandatory).

A TP for NRPPa reflecting the above proposals is provided in Appendix A.

# References

R3-213107 LS on support of UL-AOA/ZOA assistance information signalling for NR positioning, RAN1

# A	Appendix: Text Proposal for TS 38.455 BL CR

Start of Text Proposal for TS 38.455 BL CR

#### 9.1.4.1	MEASUREMENT REQUEST

This message is sent by the LMF to request the NG-RAN node to configure a positioning measurement.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| TRP Measurement Request List |  | 1 |  |  | YES | reject |
| >TRP Measurement Request Item |  | 1..<maxnoofMeasTRPs> |  |  | EACH | reject |
| >>TRP ID | M |  | 9.2.24 |  | - |  |
| >>Search Window Information | O |  | 9.2.26 |  | - |  |
| >>Cell ID | O |  | NR CGI 9.2.9 | The Cell ID of the TRP identified by the TRP ID IE. | YES | ignore |
| >>Expected UL Angle of Arrival | O |  | 9.2.z |  | YES | ignore |
| Report Characteristics | M |  | ENUMERATED (OnDemand, Periodic, ...) |  | YES | reject |
| Measurement Periodicity | C-ifReportCharacteristicsPeriodic |  | ENUMERATED (120ms, 240ms, 480ms, 640ms, 1024ms, 2048ms, 5120ms, 10240ms, 1min, 6min, 12min, 30min, 60min,…, 20480ms, 40960ms) | The codepoint 60min is not applicable | YES | reject |
| TRP Measurement Quantities |  | 1 |  |  | YES | reject |
| >TRP Measurement Quantities Item |  | 1 .. <maxnoPosMeas> |  |  | EACH | reject |
| >TRP Measurement Type | M |  | ENUMERATED (gNB-RxTxTimeDiff, UL-SRS-RSRP, UL-AoA, UL-RTOA,…) |  | - |  |
| >Timing Reporting Granularity Factor | O |  | INTEGER (0..5) | Value (0..5) corresponds to (k0..k5) TS 38.133 [16] | - |  |
| SFN initialisation Time | O |  | Relative Time 1900 9.2.36 | If this IE is not present, the TRP may assume that the value is same as its own SFN initialisation time. | YES | ignore |
| SRS Configuration | O |  | 9.2.28 |  | YES | ignore |
| Measurement Beam Information Request | O |  | ENUMERATED (true,...) |  | YES | ignore |
| System Frame Number | O |  | INTEGER(0..1023) |  | YES | ignore |
| Slot Number | O |  | INTEGER(0..79) |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifReportCharacteristicsPeriodic | This IE shall be present if the Report Characteristics IE is set to the value "Periodic". |

| Range bound | Explanation |
| --- | --- |
| maxnoPosMeas | Maximum no. of measured quantities that can be configured and reported with one positioning measurement message. Value is 16384. |
| maxnoofMeasTRPs | Maxmum no. of TRPs that can be included within one message. Value is 64. |

Next Change

### 9.2.z	Expected UL Angle of Arrival

This information element contains the expected uplink Angle of Arrival.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Expected Azimuth Angle of Arrival |  | 1 |  | Defined as (φAOA - ΔφAOA/2, φAOA + ΔφAOA/2) |
| >Expected Azimuth Angle of Arrival Value | M |  | INTEGER(0..3599) | φAOA component of Expected Azimuth AoA |
| >Expected Azimuth Angle of Arrival Uncertainty Range | M |  | INTEGER(0..3599) | ΔφAOA component of Expected Azimuth AoA |
| Expected Zenith Angle of Arrival |  | 0..1 |  | Defined as (θZOA – ΔθZOA/2, θZOA + ΔθZOA/2) |
| >Expected Zenith Angle of Arrival Value | M |  | INTEGER(0..1799) | θZOA component of Expected Zenith AoA |
| >Expected Zenith Angle of Arrival Uncertainty Range | M |  | INTEGER(0..1799) | ΔθZOA component of Expected Zenith AoA |
| LCS to GCS Translation |  | 0..1 |  | If absent, the azimuth and zenith are provided in GCS. |
| >Alpha | M |  | INTEGER (0..3599) |  |
| >Beta | M |  | INTEGER (0..3599) |  |
| >Gamma | M |  | INTEGER (0..3599) |  |

End of Text Proposal for TS 38.455 BL CR