3GPP TSG-RAN WG3 #113-e	R3-213517

# Online, 16th  - 26th August 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 0606 | rev | 1 | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of maximum extended slice support items |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2021-08-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In R3-204394, the S-NSSAI range is extended from 1024 to 65535. The maxnoofExtSliceItems value should be 16bits and there seems no strong reason to reduce the maximum number of slice support items from 65536 to 65535. Therefore, maximum number of slice support items should be corrected to 65536. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the maxnoofExtSliceItems from 65535 to 65536. Imapct analysis: This CR has isolated impact towards the previous release of the specfication. This CR has ASN.1 impact with non backward compatible way. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The maximum number of slice item that can be supported by network is less than what it should be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.191, 9.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

UNCHANGED TEXT OMITTED

9.3.1.191	Extended Slice Support List

This IE indicates a list of supported slices.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Slice Support Item |  | 1..<maxnoofExtSliceItems> |  |  |
| >S-NSSAI | M |  | 9.3.1.24 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofExtSliceItems | Maximum no. of signalled slice support items. Value is 65536. |

UNCHANGED TEXT OMITTED

NEXT CHANGE

9.4.7	Constant Definitions

UNCHANGED TEXT OMITTED

-- **************************************************************

--

#### -- Lists

--

-- **************************************************************

maxnoofAllowedAreas					INTEGER ::= 16

maxnoofAllowedCAGsperPLMN			INTEGER ::= 256

maxnoofAllowedS-NSSAIs				INTEGER ::= 8

maxnoofBluetoothName				INTEGER ::= 4

maxnoofBPLMNs						INTEGER ::= 12

maxnoofCAGSperCell					INTEGER ::= 64

maxnoofCellIDforMDT					INTEGER ::= 32

maxnoofCellIDforWarning				INTEGER ::= 65535

maxnoofCellinAoI					INTEGER ::= 256

maxnoofCellinEAI					INTEGER ::= 65535

maxnoofCellinTAI					INTEGER ::= 65535

maxnoofCellsingNB					INTEGER ::= 16384

maxnoofCellsinngeNB					INTEGER ::= 256

maxnoofCellsinUEHistoryInfo			INTEGER ::= 16

maxnoofCellsUEMovingTrajectory		INTEGER ::= 16

maxnoofDRBs							INTEGER ::= 32

maxnoofEmergencyAreaID				INTEGER ::= 65535

maxnoofEAIforRestart				INTEGER ::= 256

maxnoofEPLMNs						INTEGER ::= 15

maxnoofEPLMNsPlusOne				INTEGER ::= 16

maxnoofE-RABs						INTEGER ::= 256

maxnoofErrors						INTEGER ::= 256

maxnoofExtSliceItems					INTEGER ::= 65536

maxnoofForbTACs						INTEGER ::= 4096

maxnoofFreqforMDT					INTEGER ::= 8

maxnoofMDTPLMNs						INTEGER ::= 16

maxnoofMultiConnectivity			INTEGER ::= 4

maxnoofMultiConnectivityMinusOne	INTEGER ::= 3

maxnoofNeighPCIforMDT				INTEGER ::= 32

maxnoofNGConnectionsToReset			INTEGER ::= 65536

maxnoofNRCellBands					INTEGER ::= 32

maxnoofPC5QoSFlows 					INTEGER ::= 2048

maxnoofPDUSessions					INTEGER ::= 256

maxnoofPLMNs						INTEGER ::= 12

maxnoofQosFlows						INTEGER ::= 64

maxnoofQosParaSets					INTEGER ::= 8

maxnoofRANNodeinAoI					INTEGER ::= 64

maxnoofRecommendedCells				INTEGER ::= 16

maxnoofRecommendedRANNodes			INTEGER ::= 16

maxnoofAoI							INTEGER ::= 64

maxnoofSensorName					INTEGER ::= 3

maxnoofServedGUAMIs					INTEGER ::= 256

maxnoofSliceItems					INTEGER ::= 1024

maxnoofTACs							INTEGER ::= 256

maxnoofTAforMDT						INTEGER ::= 8

maxnoofTAIforInactive				INTEGER ::= 16

maxnoofTAIforPaging					INTEGER ::= 16

maxnoofTAIforRestart				INTEGER ::= 2048

maxnoofTAIforWarning				INTEGER ::= 65535

maxnoofTAIinAoI						INTEGER ::= 16

maxnoofTimePeriods					INTEGER ::= 2

maxnoofTNLAssociations				INTEGER ::= 32

maxnoofWLANName						INTEGER ::= 4

maxnoofXnExtTLAs					INTEGER ::= 16

maxnoofXnGTP-TLAs					INTEGER ::= 16

maxnoofXnTLAs						INTEGER ::= 2

maxnoofCandidateCells				INTEGER ::= 32

maxNRARFCN							INTEGER ::= 3279165

UNCHANGED TEXT OMITTED

END OF CHANGES