3GPP TSG-RAN WG3 Meeting #113-e	R3-213577

E-meeting, 16-26 Aug 2021

Title: 	(TP to TS 38.401 BL CR) Support of PTP and PTM switch

Source: 	Huawei, CBN, China Unicom, China Telecom

Agenda item:	22.2.3

Document Type:	Other

1. Introduction

In RAN3#112-e meeting, a working assumption was achieved on PTP and PTM switch:

WA: For the RAN2 agreed split MRB bearer with a common PDCP: the decision of using PTP (RLC leg) or PTM (RLC leg) is made by the gNB-DU

In this contribution, we provide further analyses on this, and provide the TP to TS 38.401 BL CR on that.

2. Discussion

From gNB-CU point of view, there is a semi-statistic switch between PTP MRB and PTM MRB, and this kind of switch should be performed thorough e.g. RRC signalling, therefore is up to the gNB-CU to make the decision.

On the other hand, from gNB-DU point of view, there is a dynamic switch between PTP leg and PTM leg. In the RAN3 achieved WA, for the RAN2 agreed split MRB bearer with a common PDCP, it is said that the decision of using PTP (RLC leg) or PTM (RLC leg) is made by the gNB-DU.

Our understanding of the dynamic PTP leg and PTM leg switch for the split MRB bearer with a common PDCP is shown in Figure 1:

Figure 1. Dynamic PTP leg and PTM leg switch for the split MRB bearer with a common PDCP

Upon receiving the MBS data from core network, the gNB-CU transmits PDCP PDUs towards the gNB-DU via a shared F1-U tunnel established for this MRB, and then the gNB-DU makes the decision on whether PTP leg or/and PTM leg to use. For example, in case both PTP and PTM are used, the gNB-DU duplicates the data to both PTP RLC leg and PTM RLC leg, in case of PTM to PTP switch, the gNB-DU changes to transmit the data via PTP RLC leg from PTM RLC leg.

Proposal: For the split MRB bearer with a common PDCP, the gNB-CU transmits the MBS data of that MRB to the gNB-DU via a shared F1-U tunnel, and the gNB-DU makes decision of using PTP (RLC leg) or PTM (RLC leg).

3. Conclusion

Based on the discussion in this paper, about PTP and PTM switch, we propose the following:

Proposal: For the split MRB bearer with a common PDCP, the gNB-CU transmits the MBS data of that MRB to the gNB-DU via a shared F1-U tunnel, and the gNB-DU makes decision of using PTP (RLC leg) or PTM (RLC leg).

- The corresponding TP is provided in section 4.

4. Text Proposal to TS 38.401 BL CR

--------------------------------Start of the Change-----------------------------

7	NG-RAN functions description

//skip unchanged part

7.x	Support for NR MBS

The Support of NR MBS in non-split gNB case is specified in TS 38.300 [2].

7.X.1	Support of dynamic PTP and PTM switching

NG-RAN supports dynamic switch between PTP and PTM for MBS as specified in TS 38.300 [2].

In case of split gNB architecture, for the split MRB bearer with common PDCP, upon receiving the MBS data from the gNB-CU via a shared F1-U tunnel, the gNB-DU makes decision of using PTP (RLC leg) or PTM (RLC leg), the gNB-DU duplicates the data to different RLC legs as needed.

--------------------------------End of the Change-----------------------------