3GPP TSG-RAN WG3 Meeting #113-e	R3-213610

E-meeting, 16-27 Aug 2021

Title: 	Discussion on determination of location estimates in local co-ordinates

Source: 	Huawei

Agenda item:	8.1

Document Type:	Discussion

# 1. Introduction

An LS from SA location estimates in local co-ordinates has been received [1]. The LS asks whether determination of local co-ordinates would be limited to certain positioning methods.

In this paper, we discuss the supported positioning methods for the local coordinates.

# 2. Discussion

In RAN3 specification, the Geographical Coordinates IE consists of direct global coordinates and referenced local coordinates:

9.2.46 Geographical Coordinates This information element contains the geographical coordinates for the TRP. IE/Group Name; Presence; Range; IE Type and Reference; Semantics Description. CHOICE TRP Position Definition Type; M. >Direct. >>CHOICE Accuracy; M. >>>normal accuracy. >>>>TRP Position; M; NG-RAN Access Point Position 9.2.10; The configured estimated geographical position of the antenna of the cell/TRP.. >>>high accuracy. >>>>TRP High Accuracy Access Position; M; NG-RAN High Accuracy Access Point Position 9.2.49; The configured estimated geographical high accuracy position of the antenna of the cell/TRP.. >Referenced. >>Reference Point; M; 9.2.51; The reference point is used to derive the TRP position. >>CHOICE Type; M. >>>Geodetic. >>>>TRP Position Relative Geodetic; M; Relative Geodetic Location 9.2.48; The configured estimated relative geodetic coordinate of the antenna of the cell/TRP. >>>Cartesian. >>>>TRP Position Relative Cartesian; M; Relative Cartesian Location 9.2.50; The configured estimated relative Cartesian coordinate of the antenna of the cell/TRP. DL-PRS Resource Coordinates; O; 9.2.47; DL-PRS Resource Coordinates relative to the TRP coordinate.

Both the global coordinates and the local coordinates are used to express the geographical coordinate of the antenna of the cell/TRP. These coordinates can support the positioning methods based on the TRP/RAN. There is no reason to limit the positioning methods that based on the TRP/RAN to use the local coordinates. For the RAT-independent positioning methods, which do not use the TRP information, may not use the local coordinates. Therefore, the local coordinates should be applicable to all the RAT-dependent positioning methods.

Proposal 1: The local coordinates should be applicable to all the RAT-dependent positioning methods.

A reply LS has been prepared to inform SA2 the RAN3 conclusion [2]. It is proposed to agree the reply LS.

Proposal 2: Agree the reply LS to SA2.

# 3. Conclusion

Based on the discussion in this paper, we propose the following:

Proposal 1: The local coordinates should be applicable to all the RAT-dependent positioning methods.

Proposal 2: Agree the reply LS to SA2.

The draft reply LS is provided in [2].

# 4. Reference

- R1-2106326, LS on Positioning Reference Units (PRUs) for enhancing positioning performance.

- R3-213611 [Draft] Reply LS on determination of location estimates in local co-ordinates