3GPP TSG-RAN WG3 Meeting #113-e	R3-213837

Online, 16-26 August 2021

Title:	[Draft] Reply LS on the coordination between gNBs on the supporting of RedCap UEs

Response to:	LS on the coordination between gNBs on the supporting of RedCap UEs (R2-2106536)

Release:	Rel-17

Work Item:	NR_redcap

Source:	Ericsson [to be RAN3]

To:	RAN2

Cc:

Contact Person:

#### Name:	Yazid Lyazidi

Tel. Number:	+ 46 7 25 93 1234

E-mail Address:	yazid.lyazidi@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

RAN3 thanks RAN2 for their LS.

From RAN3's perspective, the current handover design in RAN allows a target node to inspect the UE's Radio Capability Information, then conclude that it cannot configure that UE and reject the incoming HO, if such occurs, while informing the source of the reason for rejection. If RedCap UE (in-)capabilities are defined in a backwards compatible way, both new and legacy target gNBs may reject RedCap UEs if they are configured not to accept those UEs, or since they do not support them, respectively.

Therefore, RAN3 considers that coordination between gNBs for RedCap UEs during handover can be achieved by adding RedCap (in)capabilities in a backward compatible way to the UE's Radio Capability Information and relying on existing Xn-AP (and F1-AP) procedures and fields to achieve this goal.

2. Actions:

To RAN2

ACTION: 	RAN3 kindly asks RAN2 to take the above into account.

3. Date of Next TSG-RAN WG3 Meetings:

TSG-RAN WG3 Meeting #114-e                       November 2021	E-Meeting

TSG-RAN WG3 Meeting #114bis-e                  January 2022 	E-Meeting