3GPP TSG-RAN3 Meeting # 113-e	   R3-213852

# 16th - 26th  August 2021

Agenda Item:	19.2.1

Source:	Ericsson

Title: 	TP to F1AP BL CR for Rel-17 UL AoA enhancements

Document for:	Other

# 1	Introduction

This contribution presents the F1AP proposed changes to support Rel-17 UL AoA enhancements that have been presented in our NRPPa companion TP in R3-213852.

# 2	TP to F1AP BL CR

START OF CHANGES

#### 9.2.12.14	POSITIONING INFORMATION RESPONSE

This message is sent by the gNB-DU to provide the configured SRS information to the gNB-CU.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| SRS Configuration | O |  | 9.3.1.192 |  | YES | ignore |
| SFN Initialisation Time | O |  | 9.3.1.183 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| AoA Assistance Information Request | O |  | ENUMERATED (True, …) |  | YES | ignore |

NEXT CHANGE

#### 9.2.12.26	POSITIONING INFORMATION UPDATE

This message is sent by the gNB-DU to indicate that a change in the SRS configuration has occurred.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| SRS configuration | O |  | 9.3.1.192 |  | YES | ignore |
| SFN Initialisation Time | O |  | 9.3.1.183 |  | YES | ignore |
| AoA Assistance Information Request | O |  | ENUMERATED (True, …) |  | YES | ignore |

NEXT CHANGE

#### 9.2.12.3	POSITIONING MEASUREMENT REQUEST

This message is sent by the gNB-CU to request the gNB-DU to configure a positioning measurement.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| RAN Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| TRP Measurement Request List |  | 1 |  |  | YES | reject |
| >TRP Measurement Request Item |  | 1..<maxnoofMeasTRPs> |  |  |  |  |
| >>TRP ID | M |  | 9.3.1.197 |  |  |  |
| >>Search Window Information | O |  | 9.3.1.204 |  |  |  |
| >>NR CGI | O |  | 9.3.1.12 | The Cell ID of the TRP identified by the TRP ID IE. | YES | ignore |
| >>AoA Search Window Information | O |  | UL-AoA assistance information 9.2.Y |  | YES | ignore |
| Positioning Report Characteristics | M |  | ENUMERATED (OnDemand, Periodic, …) |  | YES | reject |
| Positioning Measurement Periodicity | C-ifReportCharacteristicsPeriodic |  | ENUMERATED (120ms, 240ms, 480ms, 640ms, 1024ms, 2048ms, 5120ms, 10240ms, 1min, 6min, 12min, 30min, …, 20480ms, 40960ms) |  | YES | reject |
| Positioning Measurement Quantities |  | 1 |  |  | YES | reject |
| > Positioning Measurement Quantities Item |  | 1..<maxnoofPosMeas> |  |  | EACH |  |
| >> Positioning Measurement Type | M |  | ENUMERATED (gNB RX-TX, UL-SRS-RSRP, UL AoA, UL RTOA, …, UL Angle of Arrival Report, UL Angle of Arrival linear array) |  |  | - |
| >>Timing Reporting Granularity Factor | O |  | INTEGER (0..5) | TS 38.133 [38] |  |  |
| SFN Initialisation Time | O |  | Relative Time 1900 9.3.1.183 | If this IE is not present, the TRP may assume that the value is same as its own SFN initialisation time. | YES | ignore |
| SRS Configuration | O |  | 9.3.1.192 |  | YES | ignore |
| Measurement Beam Information Request | O |  | ENUMERATED (true, ...) |  | YES | ignore |
| System Frame Number | O |  | INTEGER(0..1023) |  | YES | ignore |
| Slot Number | O |  | INTEGER(0..79) |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPosMeas | Maximum no. of measured quantities that can be configured and reported with one message. Value is 16384. |
| maxnoofMeasTRPs | Maximum no. of TRPs that can be included within one measurement message. Value is 64. |

| Condition | Explanation |
| --- | --- |
| ifReportCharacteristicsPeriodic | This IE shall be present if the Positioning Report Characteristics IE is set to the value "Periodic". |

NEXT CHANGE

#### 9.3.1.166	Positioning Measurement Result

The purpose of this information element is to provide the measurement result(s).

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |  | Criticality | Assigned Criticality |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Positioning Measured Result Item |  | 1 .. <maxnoofPosMeas> |  |  |  |  |  |  |
| >CHOICE Measured Results Value | M |  |  |  |  |  |  |  |
| >>UL Angle of Arrival | M |  | 9.3.1.167 |  |  |  |  |  |
| >>UL SRS-RSRP | M |  | INTEGER (0..126) |  |  |  |  |  |
| >>UL RTOA | M |  | UL RTOA Measurement 9.3.1.168 |  |  |  |  |  |
| >>gNB Rx-Tx Time Difference | M |  | 9.3.1.170 |  |  |  |  |  |
| >> UL Angle of Arrival Report | M |  | 9.3.1.X0 |  | YES | ignore |
| >> UL Angle of Arrival linear array | M |  | 9.3.1.Y0 |  | YES | ignore |
| >Time Stamp | M |  | 9.3.1.171 |  |  |  |  |  |
| >Measurement Quality | O |  | TRP Measurement Quality 9.3.1.172 |  |  |  |  |  |
| >Measurement Beam Information | O |  | 9.3.1.173 |  |  |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPosMeas | Maximum no. of measured quantities that can be configured and reported with one message. Value is 16384. |

#### 9.3.1.167	UL Angle of Arrival

This information element contains the uplink Angle of Arrival measurement.

| IE/Group Name |  | Presence |  | Range |  | IE Type and Reference |  | Semantics Description |  | Criticality |  | Assigned Criticality |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Azimuth Angle of Arrival |  | M |  |  |  | INTEGER(0..3599) |  | TS 38.133 [38] |  |  |  |  |  |
| Zenith Angle of Arrival |  | O |  |  |  | INTEGER(0..1799) |  | TS 38.133 [38] |  |  |  |  |  |
| LCS to GCS Translation |  |  |  | 0..1 |  |  |  | If absent, the azimuth and zenith are provided in GCS. |  |  |  |  |  |
| >Alpha |  | M |  |  |  | INTEGER (0..3599) |  |  |  |  |  |  |  |
| >Beta |  | M |  |  |  | INTEGER (0..3599) |  |  |  |  |  |  |  |
| >Gamma |  | M |  |  |  | INTEGER (0..3599) |  |  |  |  |  |  |  |
| Azimuth AoA Uncertainty range | O |  | have |  | YES | ignore |
| Zenith AoA Uncertainty range | O |  | INTEGER(-899..899) |  | YES | ignore |

NEXT CHANGE

9.3.1.X0	UL Angle of Arrival Report

This information element contains the list of the detected uplink Angle of Arrival measurements.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| List of UL AoA |  | 1..<maxnoofAddAoAs > |  |  |
| > UL-AoA assistance information | M |  | 9.3.1.X1 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofAddAoAs | Maximum no of additional AoAs that can be reported. Value is FFS |

### 9.3.1.X1	UL-AoA assistance information (FFS)

This information element contains the expected AoA/ZoA values and uncertainty range for addressing ambiguities in UL-AoA estimation.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Azimuth Angle of Arrival |  | 1 |  |  |
| >Expected value | M |  | INTEGER(0..3599) | TS 38.133 [16] |
| >Uncertainty range | O |  | INTEGER(-1799..1799) |  |
| Zenith Angle of Arrival |  | 0..1 |  |  |
| >Expected value | M |  | INTEGER(0..1799) | TS 38.133 [16] |
| >Uncertainty range | O |  | INTEGER(-899..899) |  |
| LCS to GCS Translation |  | 0..1 |  |  |
| >Alpha | M |  | INTEGER (0..3599) | Antenna array aligned LCS to GCS Translation |
| >Beta | M |  | INTEGER (0..3599) | Antenna array aligned LCS to GCS Translation |
| >Gamma | M |  | INTEGER (0..3599) | Antenna array aligned LCS to GCS Translation |

### 9.3.1.Y0	UL Angle of Arrival linear array

This information element contains the uplink Angle of Arrival linear array measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Zenith Angle of Arrival | M |  | INTEGER(0..1799) | TS 38.133 [16] |
| LCS to GCS Translation |  | 1 |  |  |
| >Alpha | M |  | INTEGER (0..3599) |  |
| >Beta | M |  | INTEGER (0..3599) |  |
| >Gamma | M |  | INTEGER (0..3599) |  |

NEXT CHANGE

ASN.1 in progress

NEXT CHANGE