3GPP TSG-RAN WG3 Meeting #113-e	R3-213895

E-meeting, 16– 26 August, 2021

Agenda item:	18.4

Source:	Nokia, Nokia Shanghai Bell

Title:	Standards Impacts for the AI/ML Mobility Use Case

Document for:	Discussion and Decision

# 1	Introduction

In RAN3 #112-e, the following agreements were made:

Progress the prioritized use cases on energy saving, load balancing, traffic steering/mobility optimization, i.e. at least by identifying their impact on the specifications, before considering any new use case.

Where ML functionality resides within the current RAN architecture, depends on deployment and on the specific use cases

It was also discussed that the following need to be investigated:

- The detailed standard impacts for mobility optimization should continue to discuss at next meeting.

In this contribution, we focus on the use case of Mobility Optimization and we discuss standardization impacts to support AI/ML for mobility optimization.

# 2	AI/ML Mobility Optimization

In this contribution we discuss standardization impacts for distributed ML Mobility Optimization in the RAN. We consider that the ML functionality is located entirely in the gNB. In case of split architectures, ML Mobility Optimization should be placed in the gNB-CU. However, if mobility is allowed to optimize beam operations an ML Inference Component at the gNB-DU may optimize beam-related operations needed for the mobility.

Proposal 1: AI/ML Mobility Optimization can be executed in the gNB. In case of split architectures, AI/ML functionality may be located in the gNB-CU while in case of beam-based AI/ML mobility solutions the AI/ML component can be located in the gNB-DU.

In case of normal Handover, a AI/ML Mobility Optimization algorithm in the gNB(gNB-CU) can consume UE Trajectory prediction information for its AI/ML decisions. The gNB (gNB-CU) using UE Trajectory prediction information can predict the Target cell where the UE will Handover to, along with related timing information of when the Handover will take place. An ML Algorithm at the gNB (gNB-CU) can also receive a reward from a neighbouring gNB for each successful Handover.

Proposal 2: An ML Mobility Optimization algorithm at the gNB (gNB-CU) can predict UE Trajectory for a UE connected to it and can use this information to predict the target cell where the UE will be handed over to, together with timing information about when the Handover will take place.

Costs and Rewards comprise essential input to Reinforcement Learning, since they are part of the feedback the Agent receives when it takes an Action. The goal of a Reinforcement Learning problem is to have the Agent maximize a long-term reward (or minimize a long-term cost). The Agent at each state uses a policy and takes an Action. As a consequence, it receives an immediate reward/cost which may have “good” values or “bad” values depending on the experience after taking the Action. The immediate and long-term rewards/costs depend on the use case under consideration. Therefore, to support Reinforcement Learning in the RAN, reward and cost information will be useful. Here we focus on costs related to Conditional Handover (CHO).

In case of CHO, a gNB prepares one or more candidate target gNBs for the Handover and configures a UE with one or more CHO Conditions. Those conditions are evaluated at the UE from the moment the CHO configuration is received by the network. Evaluation stops when the handover is executed to another cell. Even though CHO is more robust in the sense that the risk that a UE loses coverage of the source cell is very small, this comes at a cost of reserving a much higher number of network resources which have to be reserved for a longer period of time than in normal Handover since configuration is typically very early to achieve robustness. The impact of in-vain preparation at different target gNBs may not be the same, depending on their load situation, number of concurrent CHO preparations, proportion of in-vain preparations compared to successful, etc. Without any cost information measuring the impact, namely how painful or too long preparation time is for a specific target, a source gNB may only assume that the costs are the same for all Candidate Target gNBs.

Some cost(rewards) examples that relate to Conditional Handover (CHO) use case are described next.

- Rewards can be calculated at a gNB when an event is successful: In such a case a gNB sends a reward (negative cost) when a Handover or Conditional Handover is successful.

- Costs can be calculated at a gNB when an event is not successful: In such a case a gNB sends a (negative) reward(cost). This can be the case when a Conditional Handover fails. Another possibility is to assign a smaller cost for cases that are undesirable, but that did not lead to a failure. This could be the case when a Conditional Handover is configured at a Candidate Target gNB, but the execution condition is not met for some UE.

There are several options on what the exact value of a cost or reward can be and this depends on the optimization criterion and the use case. If the target is to maximize the number of successful CHOs, then the reward can be the number of successful handovers per action (e.g., per number of configured cells). In some other case, the objective may be to minimize the impact of a Conditional Handover at a Target gNB. In such a case the cost could be an internal load, related to CHO. Some other alternatives could be to equalize in the network the time duration of CHO reservations by different Sources. In this latter case, the cost can be calculated as the time difference between the time duration of CHO reservations by a given source gNB towards a given Target gNB minus the respective time duration of the CHO reservation by other sources. This can provide information to a source about how much longer (or shorter) with respect to other sources its CHO reservations are towards a Target gNB.

Proposal 3: Study how to define costs or rewards and exchange of those between neighbours in a standardized way.

# 5	Conclusion

We make the following proposals:

Proposal 1: AI/ML Mobility Optimization can be executed in the gNB. In case of split architectures, AI/ML functionality may be located in the gNB-CU while in case of beam-based AI/ML mobility solutions the AI/ML component can be located in the gNB-DU.

Proposal 2: An ML Mobility Optimization algorithm at the gNB (gNB-CU) can predict UE Trajectory for a UE connected to it and can use this information to predict the target cell where the UE will be handed over to, together with timing information about when the Handover will take place.

Proposal 3: Study how to define costs or rewards and exchange of those between neighbours in a standardized way.