3GPP TSG-RAN WG3 Meeting #113-e	R3-213958

E-meeting, 16-27 Aug 2021

Title: 	Corrections on positioning

Source: 	Huawei, CMCC

Agenda item:	9.3.4.1

Document Type:	Discussion

1. Introduction

This contribution provides some corrections on the rel-16 positioning function.

2. Discussion

The Measurement Request message in NRPPa and F1AP contains the “System Frame Number” IE and “Slot Number” IE. These two IEs have specific meaning of the activation time of SRS transmission. However, the current spec does not have any descriptions regarding these two IEs in Measurement Request message. The gNBs cannot know how to use this information when receiving these two IEs. Therefore, it is proposed to use the following procedural text to clarify the meaning of these two IEs.

If the System Frame Number IE and/or the Slot Number IE are included in the MEASUREMENT REQUEST message, the NG-RAN node shall consider that the respective information indicates the activation time of SRS transmission by the UE.

Proposal 1: RAN3 to agree to add procedural text for “System Frame Number” IE and “Slot Number” IE in NRPPa and F1AP to avoid ambiguity.

Furthermore, the positioning function needs the support of F1AP. The current stage 2 spec in TS 38.305 only list the F1 interface in clause 6.1. More descriptions regarding the positioning function and procedures of F1AP is still missing and should be added.

Additionally, the current stage 2 spec lacks reference to NRPPa, NGAP, and F1AP.

Proposal 2: RAN3 to agree to add correct references and the description for positioning function and procedures of F1AP into TS 38.305.

4. Conclusion

In this contribution, we provide analysis on some corrections on positioning rel-16 and propose:

Proposal 1: RAN3 to agree to add procedural text for “System Frame Number” IE and “Slot Number” IE in NRPPa and F1AP to avoid ambiguity.

Proposal 2: RAN3 to agree to add correct references and the description for positioning function and procedures of F1AP into TS 38.305.

The corresponding CRs are provided in [1] ~ [3].

5. Reference

- R3-213959	Adding procedural text for System Frame Number and Slot Number

- R3-213960	Adding procedural text for System Frame Number and Slot Number

- R3-213961	Adding F1AP support of positioning in stage 2