3GPP TSG-RAN WG3 #113-e     					                                                   R3-213997

- 16 - 26 Aug 2021

Online

Agenda item:	9.3.4.1

Source: 	ZTE, China Telecom, China Unicom

Title: 	Discussion on E-RAB Modification Indication Procedure

Document for:	Discussion

Introduction

In real deployment, there could be one scenario which leads to conflict between the E-RAB Modification Indication procedure triggered by eNB and the UE Context Modification procedure triggered by MME.

In this contribution, we will discuss the details and provide the clarification on this scenario.

Discussion

In real deployment, a UE could be switched from standalone NR to standalone LTE. After the inter-system handover, the UE within the eNB is configured with EN-DC by performing a SN Addition procedure. And the overall illustration is shown in Figure 1.

Figure 1: A possible scenario in real deployment

After the inter-system handover, the tracking area update for the UE should be performed. Then, the MME shall trigger a UE Context Modification procedure to modify the QoS related information, e.g. AMBR. While, after the SN Addition procedure for EN-DC, the eNB shall trigger a E-RAB Modification Indication procedure to modify the DL address of the E-RAB. In this case, the E-RAB Modification Indication procedure may conflict with the ongoing UE Context Modification procedure.

However, in current S1AP specification, there is no corresponding description to solve this potential issue for the E-RAB Modification Indication procedure. And the solution could be referred to the following description in TS 36.413.

-------------------------------------------------------------TS 36.413--------------------------------------------------------------------

Interactions with E-RAB Setup procedure or E-RAB Modify procedure:

If the E-RAB MODIFICATION INDICATION message is received by the MME during an ongoing E-RAB Setup procedure or an ongoing E-RAB Modify procedure, the MME shall proceed with the E-RAB Modification Indication procedure.

-------------------------------------------------------------TS 36.413--------------------------------------------------------------------

When the potential conflict happens, it should follow the similar principle of the above interactions, i.e., If the E-RAB MODIFICATION INDICATION message is received by the MME during an ongoing UE Context Modification procedure, the MME shall proceed with the E-RAB Modification Indication procedure instead of discarding this procedure.

Proposal: If the E-RAB MODIFICATION INDICATION message is received by the MME during an ongoing UE Context Modification procedure, the MME shall proceed with the E-RAB Modification indication procedure.

The corresponding CRs for Rel-15 and Rel-16 are given in [2] and [3] respectively.

Conclusion

Proposal: If the E-RAB MODIFICATION INDICATION message is received by the MME during an ongoing UE Context Modification procedure, the MME shall proceed with the E-RAB Modification indication procedure.

References

- TS 36.413

- R3-213999 Correction on E-RAB Modification Indication Procedure for Rel-15  ZTE, China Telecom, China Unicom

- R3-214000 Correction on E-RAB Modification Indication Procedure for Rel-16  ZTE, China Telecom, China Unicom