3GPP TSG-RAN WG3 Meeting #113-e	R3-213999

# 16-26 Aug 2021

# Online

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.413 | CR | 1830 | rev | - | Current version: | 15.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on E-RAB Modification Indication Procedure for Rel-15 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, China Telecom, China Unicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2021-07-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - After the Inter-system Handover from Standalone NR to LTE, the UE is configured with EN-DC. - In this scenario, after the inter-system handover, the tracking area update for the UE should be performed. Then, the MME shall trigger a UE Context Modification procedure to modify the QoS related information, e.g. AMBR. While, after the SN Addition procedure for EN-DC, the eNB shall trigger a E-RAB Modification Indication procedure to modify the DL address of the E-RAB. In this case, the E-RAB Modification Indication procedure may conflict with the ongoing UE Context Modification procedure. - However, in current specification, there is not any specific description to solve the potential conflict between the two procedures. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add the description of the interactions with UE Context Modification procedure for the E-RAB Modification Indication procedure. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has no impact with the previous version of the specification for implementations following the statement introduced by this CR. No ASN.1 impact. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The conflict may happen between the E-RAB modification Indication procedure and the ongoing UE Context Modification procedure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

8.2.4	E-RAB Modification Indication

8.2.4.1	General

The purpose of the E-RAB Modification Indication procedure is to enable the eNB to request modifications of already established E-RABs for a given UE. The procedure uses UE-associated signalling.

8.2.4.2	Successful Operation

Figure 8.2.4.2-1: E-RAB Modification Indication procedure. Successful operation.

The eNB initiates the procedure by sending an E-RAB MODIFICATION INDICATION message to the MME.

The Transport Layer Address IE and DL GTP TEID IE included in the E-RAB To Be Modified Item IEs IE in the E-RAB MODIFICATION INDICATON message shall be considered by the MME as the new DL address of the E-RABs. The Transport Layer Address IE and DL GTP TEID IE included in the E-RAB Not To Be Modified Item IEs IE in the E-RAB MODIFICATION INDICATION message shall be considered by the MME as the E-RABs with unchanged DL address

If the Secondary RAT Usage Report List IE is included in the E-RAB MODIFICATION INDICATION message, the MME shall handle this information as specified in TS 23.401 [11].

The E-RAB MODIFICATION CONFIRM message shall contain the result for all the E-RABs that were requested to be modified according to the E-RAB To Be Modified Item IEs IE of the E-RAB MODIFICATION INDICATION message as follows:

-	A list of E-RABs which are successfully modified shall be included in the E-RAB Modify List IE.

-	A list of E-RABs which failed to be modified, if any, shall be included in the E-RAB Failed to Modify List IE.

-	A list of E-RABs which are to be released, if any, shall be included in the E-RAB To Be Released List IE.

If the E-RAB Failed to Modify List IE is received in the E-RAB MODIFICATION CONFIRM message, the eNB shall either

-	release all corresponding E-UTRA and E-UTRAN resources for the concerned E-RAB or

-	keep the previous transport information before sending the E-RAB MODIFICATION INDICATION message unchanged for the concerned E-RAB.

If the E-RAB To Be Released List IE is received in the E-RAB MODIFICATION CONFIRM message, the eNB shall release all corresponding E-UTRA and E-UTRAN resources for the concerned E-RAB.

If the CSG Membership Info IE is included in the E-RAB MODIFICATION INDICATION message, the MME shall use the information for CSG membership verification as specified in TS 36.300 [14] and provide the result of the membership verification in the CSG Membership Status IE contained in the E-RAB MODIFICATION CONFIRM message.

If PLMN Identity IE is received in the CSG Membership Info IE in the E-RAB MODIFICATION INDICATION message, the MME shall use it for CSG membership verification as specified in TS 36.300 [14].

When the MME reports unsuccessful modification of an E-RAB, the cause value should be precise enough to enable the eNB to know the reason for an unsuccessful modification.

If the Tunnel Information for BBF IE is received in the E-RAB MODIFICATION INDICATION message, the MME shall, if supported, use it in the core network as specified in TS 23.139 [37].

Interactions with E-RAB Setup procedure or E-RAB Modify procedure or UE Context Modification procedure:

If the E-RAB MODIFICATION INDICATION message is received by the MME during an ongoing E-RAB Setup procedure or an ongoing E-RAB Modify procedure or an ongoing UE Context Modification procedure, the MME shall proceed with the E-RAB Modification Indication procedure.