3GPP TSG-RAN WG3 Meeting #113-e	R3-214037

- 16-26 Aug 2021

- Online

Agenda item:	10.2.2

Source: 	ZTE, China Telecom, China Unicom

Title: 	(TP for SON BL CR 38.300) Coverage and Capacity Optimization

Document for:	Agreement

Introduction

This contribution provides a TP for SON BL CR 38.300 on the CCO.

Text Proposal for BL CR 38.300

15.5.x	Support of Coverage and Capacity Optimization

Each NG-RAN node may be configured with alternative coverage configurations and a NG-RAN node may autonomously select and switch between these configurations, e.g. using the Active Antenna Systems functions.

An NG-RAN node may notify its neighbour NG-RAN nodes about the coverage reconfiguration using the NG-RAN NODE CONFIGURATION UPDATE message with the list of cells or beams with modified coverage included. The list contains the NR CGI of each modified cell or the SSB Index of each modified beam and its coverage state indicator. The indicator may be used at the receiving NG-RAN node to adjust the functions of the Mobility Robustness Optimisation, e.g. by using the indicator to retrieve a previously stored Mobility Robustness Optimisation state. If the list includes the indication about planned reconfiguration and possibly a list of replacing cells or beams, the receiving NG-RAN node may use this to avoid connection or re-establishment failures during the reconfiguration. Also, if the sending NG-RAN node adds cells or beams in inactive state, the receiving NG-RAN node may use this information to avoid connection or re-establishment failures.