3GPP TSG-RAN WG3 #113-e	R3-214058

16 – 26 Aug 2021

Online

Title: 	(TP for SON BL CR for TS 38.423) Successful Handover support

Source: 	ZTE

Agenda item:	10.2.1.3

Document for:	Approval

1	Introduction and discussion

This contribution provided the TP for based on the following proposal.

Proposal: In order to support inter-RAT SHR, to define Choice IE type of“Successful HO Report”in XnAP, NGAP and F1AP.

2	TP to TS 38.423

*** Start changed ***

      - 9.1.3.25	ACCESS AND MOBILITY INDICATION

This message is sent by NG-RAN node1 to transfer access and mobility related information to NG-RAN node2.

Direction: NG-RAN node 1  NG-RAN node 2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| RACH Report List |  | 0..1 |  |  | YES | ignore |
| >RACH Report List Item |  | 1 .. <maxnoofRACHReports> |  |  | EACH | ignore |
| >>RACH Report Container | O |  | OCTET STRING | RACH-ReportList-r16 IE as defined in subclause 6.2.2 in TS 38.331 [10]. | YES | ignore |
| CHOICE SHR type | O |  |  |  |  |  |
| >NR |  |  |  |  |  |  |
| >>NR Successful HO Report List |  | 0..1 |  |  | YES | ignore |
| >>>NR Successful HO Report List Item |  | 1 .. <maxnoofSuccessfulHOReports> |  |  | EACH | ignore |
| >>>> NR Successful HO Report Container | O |  | OCTET STRING | FFS on the definition | YES | ignore |
| >LTE |  |  |  |  |  |  |
| >>LTE Successful HO Report List |  | 0..1 |  |  | YES | ignore |
| >>>LTE Successful HO Report List Item |  | 1 .. <maxnoofSuccessfulHOReports> |  |  | EACH | ignore |
| >>>> LTE Successful HO Report Container | O |  | OCTET STRING | FFS on the definition | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofRACHReports | Maximum no. of RACH Reports, the maximum value is 64. |
| maxnoofSuccessfulHOReports | Maximum no. of Successful HO Reports, the maximum value is 64. FFS |

*** End of changed ***