3GPP TSG-RAN WG3 #113-e	R3-214059

16 – 26 Aug 2021

Online

Title: 	(TP for SON BL CR for TS 38.473) Successful Handover support

Source: 	ZTE

Agenda item:	10.2.1.3

Document for:	Approval

1	Introduction and discussion

This contribution provided the TP for based on the following proposal.

Proposal: In order to support inter-RAT SHR, to define Choice IE type of“Successful HO Report”in XnAP, NGAP and F1AP.

2	TP to TS 38.473

*** Start changed ***

9.2.10.1	ACCESS AND MOBILITY INDICATION

This message is sent by gNB-CU to gNB-DU to provide access and mobility information to the gNB-DU.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| RACH Report Information List |  | 0..1 |  |  | YES | ignore |
| >RACH Report Information Item |  | 1 .. <maxnoofRACHReports> |  |  | - |  |
| >>RACH Report Container | M |  | OCTET STRING | RACH-ReportList-r16 IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| RLF Report Information List |  | 0..1 |  |  | YES | ignore |
| >RLF Report Information Item |  | 1 .. <maxnoofRLFReports> |  |  | - |  |
| >>NR UE RLF Report Container | M |  | OCTET STRING | nr-RLF-Report-r16 IE contained in the UEInformationResponse message defined in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| CHOICE SHR type | O |  |  |  |  |  |
| >NR |  |  |  |  |  |  |
| >>NR Successful HO Report Information List |  | 0..1 |  |  | YES | ignore |
| >>>NR Successful HO Report Information Item |  | 1 .. <maxnoof SuccessfulHOReports> |  |  |  | ignore |
| >>>> NRSuccessful HO Report Container | M |  | OCTET STRING | FFS on the definition | YES | ignore |
| >LTE |  |  |  |  |  |  |
| >>LTE Successful HO Report List |  | 0..1 |  |  | YES | ignore |
| >>>LTE Successful HO Report List Item |  | 1 .. <maxnoofSuccessfulHOReports> |  |  | EACH | ignore |
| >>>> LTE Successful HO Report Container | O |  | OCTET STRING | FFS on the definition | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofRACHReports | Maximum no. of RACH Reports, the maximum value is 64. |
| maxnoofRLFReports | Maximum no. of RLF Reports, the maximum value is 64. |
| maxnoofSuccessfulHOReports | Maximum no. of Successful HO Reports, the maximum value is 64.FFS |

*** End of changed ***