3GPP TSG-RAN WG3 Meeting #112e	R3-213745

Online, 16 - 27 August 2021

Agenda item:	  22.2.2

Source:	  Lenovo, Motorola Mobility, Huawei

Title:		(TP to TS 38.423 BL CR) Group paging for Multicast Session Activation Notification

Document for:		Other

1	Introduction

In this contribution, the text proposal are provided for the XnAP BL CR according to the proposal in [1]；

A new class 2 XnAP RAN MBS Grouping Paging procedure is introduced, of which the contents include a list of UE Identity Index values and paging DRX (optional) of the UEs in the MBS group, RAN paging area, and multicast session ID.

[1] R3-213743. Discussion on Group paging for Multicast Session Activation Notification. Lenovo, Motorola Mobility.

2	Text Proposal

----------------------------------------------------------Change Start --------------------------------------------------------------

Table 8.1-2: Class 2 Elementary Procedures

| Elementary Procedure | Initiating Message |
| --- | --- |
| Handover Cancel | HANDOVER CANCEL |
| SN Status Transfer | SN STATUS TRANSFER |
| RAN Paging | RAN PAGING |
| Xn-U Address Indication | XN-U ADDRESS INDICATION |
| S-NG-RAN node Reconfiguration Completion | S-NODE RECONFIGURATION COMPLETE |
| S-NG-RAN node Counter Check | S-NODE COUNTER CHECK REQUEST |
| UE Context Release | UE CONTEXT RELEASE |
| RRC Transfer | RRC TRANSFER |
| Error Indication | ERROR INDICATION |
| Notification Control Indication | NOTIFICATION CONTROL INDICATION |
| Activity Notification | ACTIVITY NOTIFICATION |
| Secondary RAT Data Usage Report | SECONDARY RAT DATA USAGE REPORT |
| Trace Start | TRACE START |
| Deactivate Trace | DEACTIVATE TRACE |
| Handover Success | HANDOVER SUCCESS |
| Conditional Handover Cancel | CONDITIONAL HANDOVER CANCEL |
| Early Status Transfer | EARLY STATUS TRANSFER |
| Failure Indication | FAILURE INDICATION |
| Handover Report | HANDOVER REPORT |
| Resource Status Reporting | RESOURCE STATUS UPDATE |
| Access And Mobility Indication | ACCESS AND MOBILITY INDICATION |
| RAN MBS Group Paging | RAN MBS GROUP PAGING |

----------------------------------------------------------Next Change--------------------------------------------------------------

X.X.X	RAN MBS Group Paging

X.X.X.1	General

The purpose of the RAN MBS Group Paging procedure is to enable the NG-RAN node1 to request paging of UEs that have joined a MBS Session in the NG-RAN node2.

The procedure uses non UE-associated signalling.

X.X.X.2	Successful operation

Figure 8.2.5.2-1: RAN MBS Group Paging: successful operation

The RAN MBS Group Paging procedure is triggered by the NG-RAN node1 by sending the RAN MBS GROUP PAGING message to the NG-RAN node2, in which the necessary information e.g. the MBS Session Identity IE and the UE Identity Index List IE associated with the MBS session should be provided.

When available, the NG-RAN node1 shall include the UE Specific DRX IE in the RAN MBS GROUP PAGING message towards the NG-RAN node2. If UE specific DRX IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 36.304 [34].

----------------------------------------------------------Next Change  --------------------------------------------------------------

X.X.X.X	RAN MBS GROUP PAGING

This message is sent by the NG-RAN node1 to NG-RAN node2 to page UEs for an MBS session.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| MBS Session ID | M |  | FFS |  | YES | reject |
| UE Identity Index List |  |  |  | [FFS pending to RAN2] |  |  |
| >UE Identity Index Item | M | 0 .. <maxnoofUEsofMBSPaging> |  |  | YES | reject |
| >> CHOICE UE Identity Index Value |  |  |  |  |  |  |
| >>>Length-10 |  |  |  |  |  |  |
| >>>>Index Length-10 | M |  | BIT STRING (SIZE(10)) | Coded as specified in TS 38.304 [33]. | – |  |
| >>UE specific DRX | O |  | 9.2.3.143 | Includes the UE specific paging cycle as defined in TS 38.304 [33]. | YES | ignore |
| RAN Paging Area | M |  | 9.2.3.38 |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofUEsofMBSPaging | Maximum no. UEs that should be paged regarding a MBS Session. Value is FFS. |