3GPP TSG-RAN WG3 Meeting #113-e	R3-214186

Online, 16 – 26 Aug 2021

Agenda item:	21.2

Source:	ZTE (moderator)

Title:	Summary of offline: NRIIOT2-PDC

Document for:	Discussion and Decision

1	Introduction

This paper summarizes the following email discussion:

CB: # NRIIOT2_PDC

- During the handover, what kinds of time synchronization assistance information should the source gNB notify the target gNB, e.g., the time synchronization accuracy budget; the estimated time synchronization accuracy of the UE or the referenceTimeInfo periodicity; the timestamp associated with the latest RTI sent to the UE; the reference time synchronization requirements for one-way transmission, the Time synchronization error budget…?

- For enhancements for support of time synchronization, the UE mobility does not impact the RAN3 specification?

- The time synchronization assistance information IE is introduced over NG, Xn and F1 interfaces?

- The details for the time synchronization assistance information should be discussed after further progress in SA2?

- Capture agreements and open issues

- TPs if agreeable

(ZTE - moderator)

Summary of offline disc in R3-214186

2	For the Chairman’s Notes

[TBD]

3	Discussion (Phase 1)

At RAN3#112e, the following open issue for PDC was captured in the Chair’s Minutes:

Wait for RAN2/SA2 decision on Time Synchronization assistance parameters before further discussing in RAN3.

Further discuss assistance information that may be useful for the target gNB to maintain timing accuracy required by the UE following handover, focusing on RAN3 aspects if any issue identified.

In this meeting, papers are provided further to discuss the assistance information for PDC.

Overview of papers:

The papers mainly discuss the assistance information for PDC in the following two aspects:

For time synchronization error budget information

-	ZTE [1]: In the UE mobile scenario, in view of the different synchronization budgets and reference time synchronization requirements of the two scenarios, the source gNB indicates to the target gNB the reference time synchronization requirements (control-to-control or smart grid) for one-way transmission through the Handover request. The target gNB adopts different configuration strategies based on different time synchronization budgets to achieve the goal of energy saving.

-	Nokia [5] thinks that a time synchronization accuracy budget is provided by the CN and can be passed from the source gNB to the target gNB.

-	Huawei [6] thinks that RAN3 can further discuss whether this assistance information can be UE level, PDU session level or QoS flow level upon confirmation from other groups.

-	CATT [9] thinks that the time synchronization error budget needs to be included in the QoS flow parameters.

-	Samsung [8]: RAN2 has discussed and concluded that RAN2 sees some benefits to having this information. However, SA2 hasn’t made consensus or agreed CR for the time synchronization assistance information yet. So, the time synchronization assistance information IE is introduced over NG, Xn and F1 interfaces. The details for the time synchronization assistance information should be discussed after further progress in SA2.

Moderator’s Summary and Proposal:

Some of the related contributions propose to deliver the time synchronization budget information over NG, Xn, F1 interface, and other contributions discuss the time synchronization budget information delivery level (e.g. UE level, PDU session level, or QoS flow level). But all contributions do not provide the details about the the time synchronization budget information (e.g. parameter name, parameter meaning, and value range), and there is not RAN2/SA2 decision yet on Time Synchronization assistance parameters.

Question 1: Do companies confirm that the time synchronization error budget needs to be delivered over NG, Xn and F1 interface?

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes | We think that the core network needs to send the time synchronization error budget to the gNB over NG interface so that gNB can adopt different configuration strategies for RTI according to different time synchronization error budget. With the same reason as that in NG interface, the source gNB needs to deliver the time synchronization error budget to the target gNB during HO (e.g. over NG or Xn interface). Since the RTI information is set in DU, with the same reason as that in NG interface, the time synchronization error budget should also be delivered from gNB-CU to gNB-DU over F1 interface. |

Proposed conclusion: Capture the following in the Chair’s Notes:

Question 2: What level do companies prefer to deliver the time synchronization error budget, if necessary?

Option 1: UE level

Option 2: PDU session level

Option 3: QoS flow level

Option 4: others

| Company | Option | Comments |
| --- | --- | --- |
| ZTE | Option3 | Since the time synchronization error budget is used for RTI decision, and the RTI is mainly for TSC service, the time synchronization error budget should be delivered with the same level as that for TSC Traffic Characteristics. Considering that the TSC Traffic Characteristics is delivered with QoS flow level, the time synchronization error budget should also be delivered with QoS flow level. |

Proposed conclusion: Capture the following in the Chair’s Notes:

Question 3: For the details about the the time synchronization budget information (e.g. parameter name, parameter meaning, and value range), do companies agree to further wait the RAN2/SA2 conclusion?

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes | Wait for RAN2 and SA2 decision. |

For the referenceTimeInfo(RTI) related information delivery during HO

Two companies think that UE mobility does not impacts RAN3 specification on RTI related information delivery:

-	ZTE [1] thinks that there is no RAN3 impact due to UE mobility, with the assumption that any information need to 	be exchanged between source and target can be carried within RRC containers.

-	CATT [9] thinks that there is no optimization needed for the time synchronization during handover based on the 	following RAN2 assumptions:

Assumptions:

-	There is no UE clock drift issue to be addressed

-	The source and target gNB are tightly synchronized to the same master clock within the budget and there is no need to optimize anything for HO.

Agreements

-	gPTP message interruption during mobility is not considered in the Rel-17 IIoT WI (i.e. no further specification impact are considered)

There companies think that UE mobility impacts RAN3 specification on RTI related information delivery:

-	Ericsson [2]: After a successful handover, there is delay for the target NG-RAN node to deliver Reference Time Information (RTI) to the UE, not only because the propagation delay from the target to the UE is different than from the source to the UE, but also because of the time it takes for the target to prepare the information comprising the reference time. In the disaggregated deployment, the CU needs to extract the time reference information from the DU. Therefore, it is beneficial for the target NG-RAN node to know that a UE TSN timing reference (e.g. Uncertainty, Time Information Type, TSN distribution, and Periodicity) used in the source NG-RAN node as early as possible during NG [3] and Xn [4] Handover.

-	Nokia [5]: After the UE establishes the connection to the target gNB, the target gNB needs to determine a Reference Time Information (RTI) configuration that will satisfy the timing accuracy required by the UE. This requires the target gNB to appropriately configure time synchronization parameters, and potentially also handover parameters that can reduce the handover interruption time. During handover, the source gNB provides the target gNB with the following assistance information: 1)	the time synchronization accuracy budget; 2)	the estimated time synchronization accuracy of the UE or the referenceTimeInfo periodicity; and 3)	the timestamp associated with the latest RTI sent to the UE. Through the above information, the target gNB derives the current accuracy of the UE clock with the help of some formulas.

-	Huawei [6]: During the inter-gNB handover procedure, the target gNB needs to know whether the UE needs the time sync and the 5G synchronisation time period needed by UE, so as to timely send the 5G sync time to UE to meet the time sync accuracy. The Reference Time Information configuration in the source gNB is useful for the synchronisation time configuration reference by the target RAN, it can help the target gNB determine the suitable time sync deliver mode (e.g. broadcast/unicast mode) and deliver period after the NG [7] and Xn [6] handover completion.

Moderator’s Summary and Proposal:

There are divergence for the RTI related information delivery and need to be further discussed, e.g. what information should be delivered, and whether RAN 3 specification will be impacted.

Question 4: What RTI related information do companies agree to deliver during HO?

Information 1: UE TSN timing reference (e.g. Uncertainty, Time Information Type, TSN distribution, and Periodicity) used in the source NG-RAN node

Information 2: The estimated time synchronization accuracy of the UE or the referenceTimeInfo delivery periodicity.

Information 3: The timestamp associated with the latest RTI sent to the UE.

Information 4: referenceTimeInfoPreference from UE in UEAssistanceInformation message to indicate whether the UE prefers being provisioned with the timing information specified in the IE ReferenceTimeInfo.

Information 5: others.

| Company | Information | Comments |
| --- | --- | --- |
| ZTE | Information 4 | We think that referenceTimeInfoPreference is necessary for the target gNB to decide whether RTI will be send to UE. Based on the RAN2 specification, only referenceTimeInfoPreference is provided from UE to gNB, only the referenceTimeInfoPreference should be delivered to target gNB during HO. |

Proposed conclusion: Capture the following in the Chair’s Notes:

Question 5: What RAN3 specification will be impacted to deliver the RTI related information, if any?

| Company | Comments |
| --- | --- |
| ZTE | No RAN3 specification impacts during HO. Since referenceTimeInfoPreference-r16 IE is already included in the HandoverPreparationInformation by RRC container, based on our answer to Question 4, there is no RAN3 specification impacts during HO to deliver the RTI related information. |

Proposed conclusion: Capture the following in the Chair’s Notes:

4	Conclusions, Recommendations

Capture the following in the Chair’s Notes:

References

R3-213241	Further discussion on Propagation Delay Compensation enhancements	ZTE

R3-213377	Discussion on Further enhanced NR-IIoT: Enhancements for support of time synchronization	Ericsson

R3-213378	Enhancements for support of time synchronization	Ericsson

R3-213379	Enhancements for support of time synchronization	Ericsson

R3-213449	Impact of handover on time synchronization	Nokia, Nokia Shanghai Bell

R3-213645	(TP for eIIoT BLCR for 38.423) Propagation delay compensation and timing accuracy during handover	Huawei

R3-213646	(TP for eIIoT BLCR for 38.413) Propagation delay compensation and timing accuracy during handover	Huawei

R3-213907	Discussion on supporting the time synchronization error budget	Samsung

R3-213951	Discussion on Propagation Delay Compensation Enhancements	CATT