3GPP TSG-RAN WG3 #113-e	R3-214274
Online, 16-26 August 2021

Agenda Item:	31.2.4

Source:	Ericsson (moderator)

Title:		CB: # 118_CSIRSConifg

Document for:	Discussion

# Introduction

CB: # 118_CSIRSConifg

- Check details of solution

- The possibility to request CSI-RS infor by the source node?

(E/// - moderator)

# To the chair’s notes

R3-213860 endorsed

R3-213861 endorsed

The possibility to request CSI-RS info by the source node to be continued. Can be contribution driven in next e-meeting.

# Discussion

## Check details of solutions

Comments on the stage 3 details of the Xn [1] and X2 [2] CRs can be provided below (e.g. procedural text, tabulars, asn.1, etc.)

| Company | Comment |
| --- | --- |
| Huawei | We are fine to the CRs. |
| ZTE | Fine with the CRs. |
| China Telecom | Support the CRs |
| Ericsson | Thank you all for the support! |

The moderator would like to report the following discussion to bring more clarification about the proposals:

- The purpose is to extract the CSI-RS info inside the MTC and carry them explicitly in served cell info on XnAP?

Ericsson: Yes. by “extracting”, we propose to simply list explicitly the MTCs that have CSI-RS and their neighbour relationship, so that node can forward them correctly to the right target SN.

- The CSI-RS info is linked to neighbor cells’s CSI-RS index ?

Ericsson: Yes. Otherwise, there would be a problem in configuring a UE to measure and report neighbor cell CSI-RSs, i.e. the UE would be either configured with too many beams to measure, impacting several aspects of performance (latency in target beam identification, impacting current service, etc.).

- The usage or the benefit is to enable the receiving node configure the CSI-RS measurement to the cells that neighboring to neighbor cells to avoid too much CSI-RS Config to the UE that even not neighouring to the UE serving cell?

Ericsson: exactly! Also to address the problem of message size limitation that will end up creating a processing limitation at the receiver

A simple example:

node A sends the following CSI-RS config to node B:

node a cell 1 csi-rs config:

-CSI-RS index 1

-Neighbour cell ID x1 in node B

-cellID x1’s CSI-RS config

For node B, it will configure CSI-RS measurement to the UEs only in cell x1 .

Is my understanding right?

Ericsson: yes, if x1 has status ‘activated’

## Possibility to request CSI-RS info by the source node?

It has been commented online to allow for the request CSI-RS information by the source node. Any comments on this proposal.

| Company | Comment |
| --- | --- |
| Huawei | Maybe there is no time to discuss this at this meeting. Suggest to mark it to be continued and discuss it next meeting. |
| ZTE | Share the view as Huawei, can be continue. |
| China Telecom | Can be continue |
| Ericsson | We have sympathy with this proposal and believe it should be added. We are ok to discuss it in next meeting |

# References

| [1] | R3-213860 | Signalling of Neighbour cell CSI-RS configuration information over Xn [CSIRSXn] (Ericsson, China Telecom) |
| --- | --- | --- |
| [2] | R3-213861 | Signalling of Neighbour cell CSI-RS configuration information over X2 [CSIRSX2] (Ericsson, China Telecom) |