3GPP TSG-RAN WG3 Meeting #113-e	R3-214342

16-26 August 2021

Agenda item:	20.2.4

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP for BL CR for TS 38.300) Discussion on feeder link switchover

Document for:	Discussion & decision

Per the discussion on CB: # 2005_NTN_Feeder_Link, this contribution proposes Stage-2 TP to the BL CR for TS 38.300.

Text proposal to Stage-2 BL CR

### 16.x.4 Switch over

#### 16.x.4.4 Procedures

The NTN Control function determines the point in time when the feeder link switch over between 2 gNBs is performed. The transfer of the affected UE(s)’ context between the two gNBs at feeder link switch over is performed by means of either NG based or Xn based handover, and it depends on the gNBs’ implementation and configuration information provided to the gNBs by the NTN Control function.

<<<<<<<<<<<<<<<<<<<< Next Changes Begin >>>>>>>>>>>>>>>>>>>>

# Annex B		Example implementation of Non-Terrestrial Networks (informative)

The following figure illustrates an example implementation of an Non-Terrestrial Network within an NG-RAN infrastructure for transparent NTN payload:

Figure B-1: NTN based NG-RAN

The gNB depicted in Figure B-1 may be subdivided into non-NTN infrastructure gNB functions and the NTN Service Link provisioning System. The NTN infrastructure may be thought of being subdivided into the NTN Service Link provisioning System and the NTN Control function. The NTN Service Link provisioning System may consist of one or more NTN payloads and NTN Gateways.

The NTN payload is embarked on a spaceborne (or airborne) vehicle, providing a structure, power, commanding, telemetry, attitude control for the satellite (resp. HAPS) and possibly an appropriate thermal environment, radiation shielding.

The NTN Service Link provisioning System maps the NR-Uu radio protocol over radio resources of the NTN infrastructure (e.g. beams, channels, Tx power).

The NTN control function controls the spaceborne (or airborne) vehicles as well as the radio resources of the NTN infrastructure (NTN payload(s) & NTN Gateway(s)). It provides control data, e.g. Ephemeris, to the non-NTN infrastructure gNB functions of the gNB.

Provision of NTN control data to the gNB is out of 3GPP scope.

NOTE:	The transport of NR-Uu protocol between the NTN Service Link provisioning system and the non-NTN infrastructure gNB functions is out of 3GPP scope.

Editor’s Note: The list of NTN related parameters provided below needs further discussion.

At least the following NTN related parameters are expected to be provided by O&M to the gNB for its operation

a) Earth fixed beams: for each beam provided by a given NTN-payload:

-	The Cell identifier (NG and Uu) mapped to the beam;

-	The Cell’s reference location (e.g. cell’s center and range).

b) Quasi Earth fixed beams: for each beam provided by a given NTN-payload:

-	The Cell identifier (NG and Uu) and time window mapped to a beam;

-	The Cell’s/beam’s reference location (e.g. cell’s center and range);

-	The time window of the successive switch overs (feeder link, service link);

-	The identifier and time window of all serving satellites and NTN-Gateways;

c) Earth moving beams: for each beam provided by a given NTN-payload:

-	The Uu Cell identifier mapped to a beam and mapping information to fixed geographical areas reported on NG, including information about the beams direction and motion of the beam’s foot print on Earth;

-	Its elevation wrt NTN-payload;

-	Schedule of successive serving NTN-Gateways/gNBs;

-	Schedule of successive switch overs (feeder link, service link).

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>