3GPP TSG-RAN WG3 #113e	 R3-214427

# Online, 16th – 26th August 2021

Agenda Item:	18.2

Source:	Lenovo, Motorola Mobility

Title: 	       TP for AI RAN general framework

Document for:	Discussion and Approval

TP to TR 37.817

-----------------------------------Start of Changes-----------------------------------

- 4	General Framework

Editor Note: high level principles for RAN intelligence enabled by AI, the functional framework (e.g. the AI functionality and the input/output of the component for AI enabled optimization)

Editor Note: FFS if the study assumes single vendor environment, e.g., if the model deployment/update procedure is proprietary.

4.1	High-level Principles

The following high level principles should be applied for AI-enabled RAN intelligence:

- The detailed AI/ML algorithms and models for use cases are implementation specific and out of RAN3 scope.

- The study focuses on AI/ML functionality and corresponding types of inputs/outputs.

- The input/output and the location of the Model Training and Model Inference function should be studied case by case.

- The study focuses on the analysis of data needed at the Model Training function from Data Collection, while the aspects of how the Model Training function uses inputs to train a model are out of RAN3 scope.

- The study focuses on the analysis of data needed at the Model Inference function from Data Collection, while the aspects of how the Model Inference function uses inputs to derive outputs are out of RAN3 scope.

- Where AI/ML functionality resides within the current RAN architecture, depends on deployment and on the specific use cases.

- The Model Training and Model Inference functions should be able to request, if needed, specific information to be used to train or execute the AI/ML algorithm and to avoid reception of unnecessary information. The nature of such information depends on the use case and on the AI/ML algorithm.

- The Model Inference function should signal the outputs of the model only to nodes that have explicitly requested them (e.g. via subscription), or nodes that are subject to actions based on the output from model inference.

- An AI/ML model used in a Model Inference function has to be initially trained, validated and tested before deployment.

- NG-RAN is prioritized; EN-DC is included in the scope. FFS on whether MR-DC should be down-prioritized.

- A general framework and workflow for AI/ML optimization should be defined and captured in the TR. The generalized workflow should not prevent to “think beyond” the workflow if the use case requires so.

- User data privacy and anonymisation should be respected during AI/ML operation.

4.2	Functional Framework

Editor Note: FFS whether and how to signal metrics (e.g., accuracy, uncertainty, etc.) and validity time together with or as part of the inference output.

Editor Note: FFS on whether model testing / generating of model performance metrics is performed in Model Inference.

Figure 4.2-1: Functional Framework for RAN Intelligence

This section introduces the common terminologies related to the functional framework for RAN intelligence illustrated in Figure 4.2-1. For the functions and data/information flows shown in the Figure 4.2-1, whether there is any standardization impact and what is the standardization impact are discussed in clause 5.

- Data Collection is a function that provides input data to Model Training and Model Inference functions. AI/ML algorithm specific data preparation (e.g., data pre-processing and cleaning, formatting, and transformation) is not carried out in the Data Collection function.  
Examples of input data may include measurements from UEs or different network entities, feedback from Actor, output from an AI/ML model.

  - Training Data: Data needed as input for the AI/ML Model Training function.

  - Inference Data: Data needed as input for the AI/ML Model Inference function.

- Model Training is a function that performs the ML model training, validation, and testing. The Model training function is also responsible for data preparation (e.g., data pre-processing and cleaning, formatting, and transformation) based on Training Data delivered by a Data Collection function, if required.

  - (FFS) Model Deployment/Update: Deploy or update an AI/ML model to Model Inference function.

- Model Inference is a function that provides AI/ML model inference output (e.g. predictions or decisions). The Model Inference function is also responsible for data preparation (e.g. data pre-processing and cleaning, formatting, and transformation) based on Inference Data delivered by a Data Collection function, if required.

  - Output: The inference output of the AI/ML model produced by a Model Inference function.

- Actor is a function that receives the output from the Model Inference function and triggers or performs corresponding actions. The Actor may trigger actions directed to other entities or to itself.

  - Feedback: Information that may be needed to derive Training Data or Inference Data or performance feedback.

-----------------------------------End of Changes-----------------------------------