3GPP TSG-RAN WG3 Meeting #113-e	R3-214438

E-meeting, 16-26 Aug 2021

Agenda item:	20.2.5

Source:	CATT, Thales

Title:	(TP for BL CR for TS 38.300) Configuration for CGI mapping

Document for:	Discussion & decision

Introduction

As been discussed in CB: # 3_NTNUELocation, this contribution provides a Stage-2 TP to the BL CR for TS 38.300 to capture the following agreement.

Proposal: Add the following sentence in clause “16.x.5 Signalling” of draft stg2 CR for TS 38.300.

“NG-RAN is responsible for constructing the mapped cell ID based on the UE location info received from the UE. The mapping may be pre-configured (e.g., up to operator’s policy) or up to implementation.”

TP for BL CR for TS 38.300

16.x.5 Signalling [FFS]

The Cell Identity used in following cases corresponds to a fixed geographical area, irrespective of the orbit of the NTN payload or the types of service links supported.

-	The Cell Identity indicated by the gNB to the Core Network as part of the User Location Information (as defined in TS 38.413 [26]);

-	The Cell Identity used for Paging Optimization in NG interface (as defined in TS 38.413 [26]);

-	The Cell Identity used for Area of Interest (as defined in TS 38.413 [26]);

-	The Cell Identity used for PWS (as defined in TS 38.413 [26]).

Editor’s note:	it is FFS whether the Cell Identity indicated by the gNB to the Core Network should be specified per function/procedure/feature as above.

The Cell Identity included within the target identification of the handover messages (as defined in TS 38.413 [26] and TS 38.423 [x]) allows identifying the correct target cell.

Editor’s note:	it is FFS whether the Cell Identity used in other cases is only allowed to correspond to a Uu Cell Identity or is allowed to correspond to both, a Uu and a fixed geographical area.

Editor’s note:	It is FFS whether the RAN Paging Area as signalled over Xn includes a mapped cell ID, or a Uu cell ID.

Editor’s note:	 from rapporteur if the text stay as it is, the reference for TS 38.423 should be introduced in TS 38.300

The mapping between Cell Identities and geographical areas is configured in the RAN and Core Network.

Editor’s note:	It is assumed that the method how the gNB derives the Cell Identity provided to the CN is a matter of implementation and configuration, and it is FFS whether this fact requires normative or informative specification text.

NG-RAN is responsible for constructing the mapped cell ID based on the UE location info received from the UE. The mapping may be pre-configured (e.g., up to operator’s policy) or up to implementation.