3GPP TSG-RAN3 Meeting #113-e	R3-214335

Electronic meeting, 16 - 26 August 2021

# 3GPP TSG RAN WG2#115-e		R2-2108966

Electronic Meeting, 16th August – 27th August, 2021

Title:	Reply LS on Report Amount for M4, M5, M6, M7 measurements

Response to:	R3-212961

Release:	Release 17

Work Item:	NR_ENDC_SON_MDT_enh-Core

Source:	RAN2

To:	RAN3

Cc:	SA5

Contact person:

Name:	Pradeepa Ramachandra

E-mail Address:	pradeepa.ramachandra@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1	Overall description

RAN2 thanks RAN3 for the the LS in R3-212961.

RAN2 confirms that reportAmount is necessary to configure the UE with D1 measurement (part of M6 measurement) reports from the UE.

Even when the RAN has not recieved reportAmount as part of M6 configuration from the AMF, the RAN can choose any allowed value for the reportAmount and configure the UE in a similar way to M6 configuration in LTE. Such an implementation could lead to the following limitations:

- RAN might configure the UE with infinity as the reportAmount which could lead to unnecessary usage of the radio resources over Uu interface and also increase the UE battery consumption.

- If the RAN node arbitrarily chooses a different value than infinity as the reportAmount, then the OAM might fail to correlate the M6 measurements and the corresponding M1 measurements as M6 measurement would cease to be reported after certain time despite receiving M1 measurement for the same UE.

Further, RAN2 would like to indicate that reprotAmount configuration for M4, M5 and M7 measurements is not within the scope of RAN2 and leaves the decision to RAN3 and/or SA5.

2. Actions:

To 3GPP RAN3

ACTION:

RAN2 respectfully asks RAN3 to take the above information into consideration.

3. Date of next TSG RAN WG2 meetings:

RAN2#116-e                         1st November - 12th November 2021	Online