3GPP TSG-RAN WG3 #114-e												R3-214736

Online, 1 Nov – 11 Nov 2021

Agenda item:		21.2

Source:			ZTE

Title:	(TP for Introduction of Enhanced IIoT support over F1) Time Synchronization enhancements

Document for:		Approval

- Introduction

This TP is based on the following proposals and observations.

Observation 1: In order to achieve accurate time synchronization, SA2 has agreed to introduce two UE-level parameters (e.g., a time distribution indication and a Uu time synchronization error budget).

Proposal 2b: When the CU/DU is separated, the gNB-CU notifies the gNB-DU of the Uu time synchronization error budget information through the REFERENCE TIME INFORMATION REPORTING CONTROL message.

Detailed analysis can be seen in the related discussion paper (R3-214735).

- Text Proposal for TS38.473

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< Begin of Change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

  - 8.12	Reference Time Information Reporting procedures

  - 8.12.1	Reference Time Information Reporting Control

  - 8.12.1.1	General

The purpose of the Reference Time Information Reporting Control procedure is to command the gNB-DU to send the requested accurate reference time information to the gNB-CU. The procedure uses non-UE associated signalling.

  - 8.12.1.2	Successful Operation

Figure 8.12.1.2-1: Reference Time Information Reporting Control

The gNB-CU initiates the procedure by sending REFERENCE TIME INFORMATION REPORTING CONTROL message to the gNB-DU. Upon reception of the REFERENCE TIME INFORMATION REPORTING CONTROL message, the gNB-DU shall, if supported, perform the requested reference time information reporting action.

The Report Type IE indicates to the gNB-DU whether:

-	to report on demand;

-	to report periodic, with a frequency as specified by the Report Periodicity IE;

-	to stop periodic reporting.

If the Uu time synchronization error budget IE is included in the REFERENCE TIME INFORMATION REPORTING CONTROL message, the gNB-DU node may use it as described in TS 23.501 [21].

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

  - 9.2.11.1	REFERENCE TIME INFORMATION REPORTING CONTROL

This message is sent by the gNB-CU and is used to request the gNB-DU to deliver the accurate reference time information.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Reporting Request Type | M |  | 9.3.1.147 |  | YES | reject |
| Uu Time Synchronization Error Budget | O |  | 9.3.1.yyy |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

9.3.1.yyy	Uu time synchronization error budget

This IE indicates the time synchronization error budget for the time synchronization service as specified in TS 23.501 [21].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Uu Time Synchronization Error Budget | M |  | FFS |  |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< End of Change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>