3GPP TSG-RAN WG3 #114-e																		   R3-214743

1-11 Nov 2021

Online

Agenda Item:	8.1

Source:	ZTE

Title:	Discussion on CHO with SCG configuration

Document for:	Approval

- Introduction

In the previous RAN3 # 112 e-meeting, we had the following agreement for the issue on CHO with SCG configuration based on the RAN2 LS “R3-211433 LS on Conditional Handover with SCG configuration scenarios”.

The scenarios listed in the LS from RAN2 (R3-211433) require changes in RAN3 specifications. Identified possible impacts:

-	an indicator of CHO,

-	signalling for the early data forwarding to the target SN,

-	enhancements to avoid over-booking of resources at the SN.

RAN3 has not identified any other scenario, not listed in the LS from RAN2.

RAN3 will ask RAN2 for guidance if the changes are to be introduced in Rel-16.

In the RAN2 #115 e-meeting, RAN2 sent the related LS (R3-214691, Response LS on CHO with SCG configuration) to RAN3 including the following content.

1. Overall Description: RAN2 would like to thank RAN3 for the response LS on Conditional Handover with SCG configuration. RAN2 has further discussed the topic, considering the feedback provided by RAN3. It has been concluded that CHO with SCG configuration is not supported in Rel-16. However, RAN2 agreed this shall be specified in Rel-17. Thus, RAN2 kindly asks RAN3 to take this decision into account and define necessary signaling in Rel-17.

This paper is used to provide the interface signalling design to support CHO with SCG configuration.

- Discussion

Where to discuss the issue on CHO with SCG configuration?

It is agreed that this issue shall be supported in Rel-17, it seems that it can be discussed as TEI17. However, we already have WID “Further Multi-RAT Dual Connectivity Enhancements” including CPAC.

During previous progress, some signaling design to support CHO with SCG configuration is related to CPAC, so it is better to include it into the CPAC.

Proposal 1: The issue on CHO with SCG configuration shall be discussed with CPAC.

Signalling design

In CPAC topic, the new IE “CPAC Indicator” is added in the SN addition request message. For the same reason, another new IE (e.g., “CHO DC Indicator” for EN-DC and “CHO MR-DC Indicator” for MR-DC) shall be also added in this message.

Proposal 2: The new IE (e.g., “CHO DC Indicator” for EN-DC and “CHO MR-DC Indicator” for MR-DC) shall be added in the SN addition request message to inform candidate target SN that it is for CHO with SCG configuration issue.

Early data forwarding

For CHO and CPAC, we have already agreed to support Early data forwarding.

In Rel-16, in order to support CHO in Master Node to eNB/gNB Change case, both stage 2 and stage 3 Rel-16 specifications have been enhanced for support of Early data forwarding.

Meanwhile, in Rel-17. In order to support CPAC, the signaling enhanced for support of Early data forwarding is also included.

Observation 1: The signaling for Early data forwading is already enhanced in both Rel-16 specification and CPAC related BLCR, no other signaling enhancement is needed to support Early data forwarding for CHO with SCG configuration case.

- Conclusion

Based on above analysis, we provide the following observations and proposals.

Proposal 1: The issue on CHO with SCG configuration shall be discussed with CPAC.

Proposal 2: The new IE (e.g., “CHO DC Indicator” for EN-DC and “CHO MR-DC Indicator” for MR-DC) shall be added in the SN addition request message to inform candidate target SN that it is for CHO with SCG configuration issue.

Observation 1: The signaling for Early data forwading is already enhanced in both Rel-16 specification and CPAC related BLCR, no other signaling enhancement is needed to support Early data forwarding for CHO with SCG configuration case.

We also provide corresponding TPs for this issue.

Please see:  R3-214744 for TS36.423.

R3-214745 for TS38.423

R3-214746 for TS37.340