3GPP TSG-RAN WG3 #114-e																		   R3-214748

1-11 Nov 2021

Online

Agenda Item:	14.3

Source:	ZTE

Title:	(TP for CPAC BL CR to TS 37.340) Remaining issues for MN initiated CPC

Document for:	Approval

- Introduction

In the last RAN3 e-meeting, we have the following agreement.

- CPC triggered and CPC executed - In case of MN initiated inter-SN CPC, use XnAP: Xn-U Address Indication procedure to indicate “CPC triggered” from MN to source SN. FFS for X2AP on whether to use the new CPAC Notification procedure or existing Data Address Indication procedure. - For MN initiated inter-SN CPC, upon receiving the MN RRC reconfiguration complete message from the UE, MN informs “CPC triggered” to the source SN. - In case of both MN and SN initiated inter-SN CPC, use XnAP: Xn-U Address Indication procedure to indicate “CPC execute” from MN to source SN.

This paper is used to further discuss the remaining issues based on the above progress.

- Discussion

In the previous RAN3 e-meeting, the issue on correction for CHO early data forwarding in MN to eNB/gNB Change has been discussed (Seen in R3-212741), and then the corresponding CRs have been agreed to be captured into the latest specification. Please refer to the following blue highlighted sentences, which are used to resolve the issues of Early Data Forwarding in case of CHO with EN-DC and MR-DC.

In TS37.340: Early data forwarding in case of CHO with EN-DC and MR-DC

CHO with EN-DC

NOTE 1b:	In case the handover is a conditional handover, the Data Forwarding Address Indication procedure is executed right after step 2. This Data Forwarding Address Indication procedure notifies conditional handover to the source SN, for which it may decide to perform, if applicable, early data forwarding for SN-terminated bearers, together with the sending of an EARLY STATUS TRANSFER message to the source MN. Separate Data Forwarding Address Indication procedures may be invoked to provide different forwarding addresses of the prepared conditional handovers. In this case, it is up to the source MN and SN implementations to make sure that the EARLY STATUS TRANSFER message(s) from the source SN, if any, is forwarded to the right target destination. The Data Forwarding Address Indication procedure may further be invoked to indicate to the source SN to stop already initiated early data forwarding for some SN-terminated bearers if they are no longer subject to data forwarding due to the modification or cancellation of the prepared conditional handovers. If applicable, the normal data forwarding and SN STATUS TRANSFER message would follow from the source SN once it receives the SgNB Release Request message of the step 3a that is performed after step 6.

CHO with MR-DC

NOTE 1b:	In case the handover is a conditional handover, the step 3c is executed right after step 2. The Xn-U Address Indication message notifies conditional handover to the source SN, for which it may decide to perform, if applicable, early data forwarding for SN-terminated bearers, together with the sending of an EARLY STATUS TRANSFER message to the source MN. Separate Xn-U Address Indication procedures may be invoked to provide different forwarding addresses of the prepared conditional handovers. In this case, it is up to the source MN and SN implementations to make sure that the EARLY STATUS TRANSFER message(s) from the source SN, if any, is forwarded to the right target destination. The Xn-U Address Indication procedure may further be invoked to indicate to the source SN to stop already initiated early data forwarding for some SN-terminated bearers if they are no longer subject to data forwarding due to the modification or cancellation of the prepared conditional handovers. If applicable, the normal data forwarding and SN STATUS TRANSFER message would follow from the source SN once it receives the SN Release Request message of the step 3a that is performed after step 6. In case the step 3c Xn-U Address Indication procedure corresponding to the conditional handover that the UE successfully accessed was rejected by the source SN, the source MN re-sends it after the step 3b that is performed after step 6.

In BLCR for TS37.340: Early data forwarding in case of inter-SN MN initiated CPC

MN initiated inter-SN CPC with EN-DC

In case of CPC, upon receiving the RRCConnectionReconfigurationComplete message from the UE, the MN triggers the Conditional PSCell Change Notification procedure to the source SN to inform that the CPC has been triggered, the source SN, if applicable, starts early data forwarding. The PDCP PDU and/or PDCP SDU forwarding may take place during early data forwarding.

MN initiated inter-SN CPC with MR-DC

In case of CPC, upon receiving the MN RRC reconfiguration complete message from the UE, the MN informs the SN that the CPC has been triggered via Xn-U Address Indication procedure, the source SN, if applicable, starts early data forwarding. The PDCP PDU and/or PDCP SDU forwarding may take place during early data forwarding.

Early Data Forwarding is supported for CPC, which indicates that similar issues of Early Data Forwarding presented in R3-212741 also exist. Thus, it is proposed to use the same method of Early Data Forwarding for CHO with EN-DC/MR-DC to solve the issues in Early Data Forwarding for CPC.

Proposal 1: The method of Early Data Forwarding for CHO with EN-DC/MR-DC can be used for CPC.

Proposal 2: The existing X2AP: Data Forwarding Address Indication is reused for support of Early data forwarding.

- Text Proposal for TS37.340

Note that the words highlighted in yellow are the clarifications introduced by this TP.

Start of the change

10.5	Secondary Node Change (MN/SN initiated)

10.5.1	EN-DC

The Secondary Node Change procedure is initiated either by MN or SN and used to transfer a UE context from a source SN to a target SN and to change the SCG configuration in UE from one SN to another. In inter-SN CPC, this procedure initiated either by MN or SN is also used to configure CPC configuration.

NOTE 1:	Inter-RAT SN change procedure with single RRC reconfiguration is not supported in this version of the protocol (i.e. no transition from EN-DC to DC).

The Secondary Node Change procedure always involves signalling over MCG SRB towards the UE.

MN initiated SN Change

Figure 10.5.1-1: SN Change – MN initiated

Figure 10.5.1-1 shows an example signalling flow for the MN initiated Secondary Node Change:

1/2.	The MN initiates the SN change by requesting the target SN to allocate resources for the UE by means of the SgNB Addition procedure. The MN may include measurement results related to the target SN. If forwarding is needed, the target SN provides forwarding addresses to the MN. The target SN includes the indication of the full or delta RRC configuration.

In inter-SN CPC, the MN indicates the CPC initiation and provides the upper limit for the number of PSCells to the candidate SN in step 1. If inter-SN CPC has been requested, the candidate SN provides the prepared PSCell ID(s) to the MN in step 2.

NOTE 2:	The MN may trigger the MN-initiated SN Modification procedure (to the source SN) to retrieve the current SCG configuration before step 1.

3.	If the allocation of target SN resources was successful, the MN initiates the release of the source SN resources including a Cause indicating SCG mobility. The Source SN may reject the release. If data forwarding is needed the MN provides data forwarding addresses to the source SN. If direct data forwarding is used for SN terminated bearers, the MN provides data forwarding addresses as received from the target SN to source SN. Except inter-SN CPC, reception of the SgNB Release Request message triggers the source SN to stop providing user data to the UE and, if applicable, to start data forwarding.

4/5.	The MN triggers the UE to apply the new configuration. The MN indicates to the UE the new configuration in the RRCConnectionReconfiguration message including the NR RRC configuration message generated by the target SN. The UE applies the new configuration and sends the RRCConnectionReconfigurationComplete message, including the encoded NR RRC response message for the target SN, if needed. In case the UE is unable to comply with (part of) the configuration included in the RRCConnectionReconfiguration message, it performs the reconfiguration failure procedure. [Subject to RAN2]In case of CPC, the UE applies the new configuration not including the CPC configuration and replies to MN with RRCConnectionReconfigurationComplete message, without NR RRC response message.

In case of CPC, upon receiving the RRCConnectionReconfigurationComplete message from the UE, the MN triggers the Data Forwarding Address Indication procedure to the source SN to inform that the CPC has been triggered, the source SN, if applicable, starts early data forwarding. The PDCP PDU and/or PDCP SDU forwarding may take place during early data forwarding. Separate Data Forwarding Address Indication procedures may be invoked to provide different forwarding addresses of the prepared conditional PSCell Changes. In this case, it is up to the source MN and source SN implementations to make sure that the EARLY STATUS TRANSFER message(s) from the source SN, if any, is forwarded to the right target destination. The Data Forwarding Address Indication procedure may further be invoked to indicate to the source SN to stop already initiated early data forwarding for some SN-terminated bearers if they are no longer subject to data forwarding due to the modification or cancellation of the prepared conditional PSCell changes. If applicable, the normal data forwarding and SN STATUS TRANSFER message would follow from the source SN once it receives the SN Release Request message of the step 3a that is performed after step 6. In case the step 3c Xn-U Address Indication procedure corresponding to the conditional handover that the UE successfully accessed was rejected by the source SN, the source MN re-sends it after the step 3b that is performed after step 6.

5a.	[subject to RAN2]The UE completes the CPC execution procedure by sending an RRCConnectionReconfigurationComplete message, including a NR RRC RRCReconfigurationComplete message for the new PSCell, to the MN.

6.	Except inter-SN CPC, if the RRC connection reconfiguration procedure was successful, the MN informs the target SN via SgNBReconfigurationComplete message with the encoded NR RRC response message for the target SN, if received from the UE.

In inter-SN CPC, the MN sends the SgNB Release Request message(s), to the other candidate SN(s), if any, for the cancellation of CPC preparation, and to the source SN to stop providing user data to the UE and, if applicable, to start data forwarding.

7.	If configured with bearers requiring SCG radio resources, the UE synchronizes to the target SN.

8.	For SN terminated bearers using RLC AM, the source SN sends the SN Status Transfer, which the MN sends then to the target SN, if needed.

9.	If applicable, data forwarding from the source SN takes place. It may be initiated as early as the source SN receives the SgNB Release Request message from the MN.

10.	The source SN sends the Secondary RAT Data Usage Report message to the MN and includes the data volumes delivered to and received from the UE over the NR radio for the related E-RABs.

NOTE 3:	The order the SN sends the Secondary RAT Data Usage Report message and performs data forwarding with MN is not defined. The SN may send the report when the transmission of the related bearer is stopped.

11-15.	If applicable, a path update is triggered by the MN.

16.	Upon reception of the UE Context Release message, the source SN releases radio and C-plane related resources associated to the UE context. Any ongoing data forwarding may continue.

<Skip unchanged part>

10.5.2	MR-DC with 5GC

MN initiated SN Change

The MN initiated SN change procedure is used to transfer a UE context from the source SN to a target SN and to change the SCG configuration in UE from one SN to another. This procedure can also be used to initiate inter-SN CPC.

The Secondary Node Change procedure always involves signalling over MCG SRB towards the UE.

Figure 10.5.2-1: SN change procedure - MN initiated

Figure 10.5.2-1 shows an example signalling flow for the SN Change initiated by the MN:

1/2.	The MN initiates the SN change by requesting the target SN to allocate resources for the UE by means of the SN Addition procedure. The MN may include measurement results related to the target SN. If data forwarding is needed, the target SN provides data forwarding addresses to the MN. The target SN includes the indication of the full or delta RRC configuration.

In inter-SN CPC, the MN indicates the CPC initiation and provides the upper limit for the number of PSCells to the candidate SN in step 1. If inter-SN CPC has been requested, the candidate SN provides the prepared PSCell ID(s) to the MN in step 2.

NOTE 1:	The MN may trigger the MN-initiated SN Modification procedure (to the source SN) to retrieve the current SCG configuration and to allow provision of data forwarding related information before step 1.

3.	If the allocation of target SN resources was successful, the MN initiates the release of the source SN resources including a Cause indicating SCG mobility. The Source SN may reject the release. If data forwarding is needed the MN provides data forwarding addresses to the source SN. If direct data forwarding is used for SN terminated bearers, the MN provides data forwarding addresses as received from the target SN to source SN. Except inter-SN CPC, reception of the SN Release Request message triggers the source SN to stop providing user data to the UE.

4/5.	The MN triggers the UE to apply the new configuration. The MN indicates the new configuration to the UE in the MN RRC reconfiguration message including the target SN RRC reconfiguration message. The UE applies the new configuration and sends the MN RRC reconfiguration complete message, including the SN RRC response message for the target SN, if needed. In case the UE is unable to comply with (part of) the configuration included in the MN RRC reconfiguration message, it performs the reconfiguration failure procedure. [Subject to RAN2]In case of CPC, the UE applies the new configuration not including the CPC configuration and replies to MN with MN RRC reconfiguration complete message, without NR RRC response message.

In case of CPC, upon receiving the MN RRC reconfiguration complete message from the UE, the MN informs the SN that the CPC has been triggered via Xn-U Address Indication procedure, the source SN, if applicable, starts early data forwarding. The PDCP PDU and/or PDCP SDU forwarding may take place during early data forwarding. Separate Xn-U Address Indication procedures may be invoked to provide different forwarding addresses of the prepared conditional PSCell Changes. In this case, it is up to the source MN and SN implementations to make sure that the EARLY STATUS TRANSFER message(s) from the source SN, if any, is forwarded to the right target destination. The Xn-U Address Indication procedure may further be invoked to indicate to the source SN to stop already initiated early data forwarding for some SN-terminated bearers if they are no longer subject to data forwarding due to the modification or cancellation of the prepared conditional PSCell Changes. If applicable, the normal data forwarding and SN STATUS TRANSFER message would follow from the source SN once it receives the SN Release Request message of the step 3a that is performed after step 6. In case the step 3c Xn-U Address Indication procedure corresponding to the conditional handover that the UE successfully accessed was rejected by the source SN, the source MN re-sends it after the step 3b that is performed after step 6.

5a.	[subject to RAN2]The UE completes the CPC execution procedure by sending an MN RRC reconfiguration complete message, including a SN RRC reconfiguration complete message for the new PSCell, to the MN.

6.	Except inter-SN CPC, if the RRC connection reconfiguration procedure was successful, the MN informs the target SN via SN Reconfiguration Complete message with the included SN RRC response message for the target SN, if received from the UE.

In inter-SN CPC, the MN sends the SN Release Request message(s), to the other candidate SN(s), if any, for the cancellation of CPC preparation, and to the source SN to stop providing user data to the UE and, if applicable, to start data forwarding.

7.	If configured with bearers requiring SCG radio resources the UE synchronizes to the target SN.

8.	If PDCP termination point is changed for bearers using RLC AM, the source SN sends the SN Status Transfer, which the MN sends then to the target SN, if needed.

9.	If applicable, data forwarding from the source SN takes place. It may be initiated as early as the source SN receives the SN Release Request message from the MN.

10.	The source SN sends the Secondary RAT Data Usage Report message to the MN and includes the data volumes delivered to and received from the UE as described in clause 10.11.2.

NOTE 2:	The order the SN sends the Secondary RAT Data Usage Report message and performs data forwarding with MN is not defined. The SN may send the report when the transmission of the related QoS flow is stopped.

11-15.	If applicable, a PDU Session path update procedure is triggered by the MN.

16.	Upon reception of the UE Context Release message, the source SN releases radio and C-plane related resources associated to the UE context. Any ongoing data forwarding may continue

<Skip unchanged part>

End of the change

- Annex

Latest version: TS 37.340 for information

## 10.8 Master Node to eNB/gNB Change ### 10.8.1 EN-DC The Master Node to eNB Change procedure is used to transfer context data from a source MN/SN to a target eNB. Figure 10.8.1-1: Master Node to eNB Change procedure Figure 10.8.1-1 shows an example signalling flow for the Master Node to eNB Change procedure: 1. The source MN starts the MN to eNB Change procedure by initiating the X2 Handover Preparation procedure, including both MCG and SCG configuration. NOTE 1: The source MN may trigger the MN-initiated SN Modification procedure (to the source SN) to retrieve the current SCG configuration before step 1. 2. The target eNB includes the field in HO command which releases SCG configuration, and may also provide forwarding addresses to the source MN. 3. If the allocation of target eNB resources was successful, the MN initiates the release of the source SN resources towards the source SN including a Cause indicating MCG mobility. The SN acknowledges the release request. If data forwarding is needed, the MN provides data forwarding addresses to the source SN. Reception of the SgNB Release Request message triggers the source SN to stop providing user data to the UE and, if applicable, to start data forwarding. NOTE 1a: In case the handover is a conditional handover, step 3a and step 3b are performed after the source MN receives an indication that the UE has successfully accessed one of the potential target eNB as described in TS 36.300 [2], (i.e. after step 6) NOTE 1b: In case the handover is a conditional handover, the Data Forwarding Address Indication procedure is executed right after step 2. This Data Forwarding Address Indication procedure notifies conditional handover to the source SN, for which it may decide to perform, if applicable, early data forwarding for SN-terminated bearers, together with the sending of an EARLY STATUS TRANSFER message to the source MN. Separate Data Forwarding Address Indication procedures may be invoked to provide different forwarding addresses of the prepared conditional handovers. In this case, it is up to the source MN and SN implementations to make sure that the EARLY STATUS TRANSFER message(s) from the source SN, if any, is forwarded to the right target destination. The Data Forwarding Address Indication procedure may further be invoked to indicate to the source SN to stop already initiated early data forwarding for some SN-terminated bearers if they are no longer subject to data forwarding due to the modification or cancellation of the prepared conditional handovers. If applicable, the normal data forwarding and SN STATUS TRANSFER message would follow from the source SN once it receives the SgNB Release Request message of the step 3a that is performed after step 6. 4. The MN triggers the UE to apply the new configuration. Upon receiving the new configuration, the UE releases the entire SCG configuration. 5/6. The UE synchronizes to the target eNB. 7. For SN terminated bearers using RLC AM, the SN sends the SN Status Transfer, which the source MN sends then to the target eNB. 8. If applicable, data forwarding takes place from the source side. 9a. The source SN sends the Secondary RAT Data Usagee Report message to the source MN and includes the data volumes delivered to and received from the UE over the NR radio for the related E-RABs. NOTE 2: The order the SN sends the Secondary RAT Data Usage Report message and performs data forwarding with MN is not defined. The SN may send the report when the transmission of the related bearer is stopped. 9b. The source MN sends the Secondary RAT Report message to MME to provide information on the used NR resource. 10-14. The target eNB initiates the S1 Path Switch procedure. 15. The target eNB initiates the UE Context Release procedure towards the source MN. 16. Upon reception of the UE Context Release message, the SN releases radio and C-plane related resources associated to the UE context. Any ongoing data forwarding may continue. NOTE 3: Inter-system HO from E-UTRA with EN-DC configuration to NR or to E-UTRA connected to 5GC is also supported. ### 10.8.2 MR-DC with 5GC The MN to ng-eNB/gNB Change procedure is used to transfer UE context data from a source MN/SN to a target ng-eNB/gNB. Both the cases where the source MN and the target node belong to the same RAT (i.e. they are both ng-eNBs or both gNBs) and the cases where the source MN and the target node belong to different RATs are supported. NOTE 0: Inter-system HO from ng-eNB/gNB MN to eNB is also supported. Figure 10.8.2-1: MN to ng-eNB/gNB Change procedure Figure 10.8.2-1 shows an example signalling flow for the MN to ng-eNB/gNB Change procedure: 1. The source MN starts the MN to ng-eNB/gNB Change procedure by initiating the Xn Handover Preparation procedure, including both MCG and SCG configuration. NOTE 1: The source MN may trigger the MN-initiated SN Modification procedure (to the source SN) to retrieve the current SCG configuration and to allow provision of data forwarding related information before step 1. 2. The target ng-eNB/gNB includes the field in HO command which releases the SCG configuration, and may also provide forwarding addresses to the source MN. 3. If the resource allocation of target ng-eNB/gNB was successful, the MN initiates the release of the source SN resources towards the source SN including a Cause indicating MCG mobility. The SN acknowledges the release request. If data forwarding is needed, the MN provides data forwarding addresses to the source SN. Reception of the SN Release Request message triggers the source SN to stop providing user data to the UE and, if applicable, to start data forwarding. NOTE 1a: In case the handover is a conditional handover, step 3a and step 3b are performed after the source MN receives an indication that the UE has successfully accessed one of the potential target gNB as described in TS 38.300 [3], (i.e. after step 6) NOTE 1b: In case the handover is a conditional handover, the step 3c is executed right after step 2. The Xn-U Address Indication message notifies conditional handover to the source SN, for which it may decide to perform, if applicable, early data forwarding for SN-terminated bearers, together with the sending of an EARLY STATUS TRANSFER message to the source MN. Separate Xn-U Address Indication procedures may be invoked to provide different forwarding addresses of the prepared conditional handovers. In this case, it is up to the source MN and SN implementations to make sure that the EARLY STATUS TRANSFER message(s) from the source SN, if any, is forwarded to the right target destination. The Xn-U Address Indication procedure may further be invoked to indicate to the source SN to stop already initiated early data forwarding for some SN-terminated bearers if they are no longer subject to data forwarding due to the modification or cancellation of the prepared conditional handovers. If applicable, the normal data forwarding and SN STATUS TRANSFER message would follow from the source SN once it receives the SN Release Request message of the step 3a that is performed after step 6. In case the step 3c Xn-U Address Indication procedure corresponding to the conditional handover that the UE successfully accessed was rejected by the source SN, the source MN re-sends it after the step 3b that is performed after step 6. 4. The MN triggers the UE to perform HO and apply the new configuration. Upon receiving the new configuration, the UE releases the entire SCG configuration. 5/6. The UE synchronizes to the target ng-eNB/gNB. 7. If PDCP termination point is changed for bearers using RLC AM, the SN sends the SN Status Transfer, which the source MN sends then to the target ng-eNB/gNB. 8. If applicable, data forwarding takes place from the source side. 9a. The source SN sends the Secondary RAT Data Usage Report message to the source MN and includes the data volumes delivered to and received from the UE as described in clause 10.11.2. NOTE 2: The order the SN sends the Secondary RAT Data Usage Report message and performs data forwarding with MN is not defined. The SN may send the report when the transmission of the related QoS flow is stopped. 9b. The source MN sends the Secondary RAT Report message to AMF to provide information on the used NR/E-UTRA resource. 10-14. The target ng-eNB/gNB initiates the Path Switch procedure. 15. The target ng-eNB/gNB initiates the UE Context Release procedure towards the source MN. 16. Upon reception of the UE Context Release message from the MN, the source SN releases radio and C-plane related resources associated to the UE context. Any ongoing data forwarding may continue.