3GPP TSG-RAN WG3 Meeting #114-e	   				   R3-214836

Online, November 1st – 12th 2021

Agenda Item:		20.2.5

Source:			China Telecom

Title:				Country specific routing issue

Document for:	 	Discussion

- Introduction

In the last meeting, the scenario of NTN cell covering multiple countries was discussed and the related agreements are as follows[1]:

The UE location information reported from the UE (as specified by RAN2) is accurate enough for AMF (re-)selection.

Continue to discuss whether and how to address the use case “UE moves across the country (within the same cell) and not support the new PLMN of the country”

To be continued...

In this contribution, we will continue to discuss the above mentioned problem.

- Discussion

For NTN, it can be understood that the geographical size of the cell should be much larger than the TN cell, i.e. the radius is up to ~ 1000 km and it is possible to span large cross-border areas, this will lead to the problem of country specific routing. RAN3 has been discussed many times about the network behaviour regarding inter-country mobility, relevant conclusions are included in TS 38.300 and TS 38.410 that when the UE moves across the country border within the coverage area of the same satellite, NG-RAN should perform an NG handover to change the UE to an AMF in the UE’s country.

Some companies indicated that UE may only support local PLMN in many cases[2]. In this scenario, NG-RAN can attempt to handover the UE to a candidate cell with a local PLMN, otherwise the UE should be released. There is no appropriate description of this behavior reflected in current specification when UE moves across the country border and does not support the new PLMN. So we see it is necessary to include this on TS 38.300 in principle.

Proposal: If the NG-RAN node detects that the UE is in a different country to that served by the serving AMF, the NG-RAN should perform an NG handover to change to an appropriate AMF or release the UE.

- Conclusions

In this paper, we have the following proposals:

Proposal: If the NG-RAN node detects that the UE is in a different country to that served by the serving AMF, the NG-RAN should perform an NG handover to change to an appropriate AMF or release the UE.

- References

[1]	R3-214364, Summary of email Discussion on CB: # 2006_NTN_Country_Routing, CATT.

[2] 	R3-213344, Country-specific routing issues, Huawei.