3GPP TSG-RAN WG3 Meeting #114e                                                      	R3-214911

E-Meeting: Nov 1st – Nov 11th, 2021

Agenda item:	15.3

Source:	Qualcomm Inc.

Title:	RAN visible QoE

Document for:	Discussion and Decision

# Introduction

In this paper, we discuss further on RAN Visible QoE based on some agreements and open issues identified in previous meetings.

# Discussion

## RVQOE metrics

The service types supported in the Rel17 RAN-visible QoE framework are DASH streaming and VR.

The following is supported within the RVQOE framework:

- RAN-visible QoE metrics: a subset of legacy QoE metrics data collected from UE, which are useful for RAN.

- The following metrics continue to be considered for RVQoE metrics (no additional metrics are expected):

  - Buffer Level

  - Playout Delay

  - FFS: Play List (simplified version)

  - To be discussed: Buffer Level Alarm, Interaction latency (VR only).

Detailed evaluation of all RVQoE metrics is available in the Appendix.

Proposal 1: Buffer level and Playout Delay can be considered as RVQOE metrics for DASH and VR services.

In last meeting, a buffer level alarm (derived from buffer level) was proposed as a potential RVQoE metric. The idea is that if the application client can determine if that the buffer level falls below an "alarm" level (defined by a second threshold, lower than the one used for nominal E2E flow control) and indicate it to the NG-RAN via RVQoE report. In our view defining this “alarm threshold” is same as defining a RVQoE value which is the qualitative representation of a RVQoE metric. Please refer to section 2.8 for further discussion on RVQoE values.

Observation 1: Defining a buffer level alarm indication instead of reporting an absolute buffer level seems to be under the scope of RVQoE values where UE can represent qualitative representation of RVQoE metrics.

Proposal 2: The discussion on whether to define a buffer level alarm should happen under RVQoE values as this is a qualitative representation of a RVQoE metric and not an absolute value.

Observation 2: Reporting the QoE metrics “VR Interaction Latency” and “Play List” by UE seems cumbersome as it includes a lot of information elements.

Proposal 3: VR Interaction Latency and Play List seems a lot of signalling overhead in RRC and should not be considered as RVQOE metrics for Rel-17

In previous meetings, a simplified version of Play List (e.g., an indication from the application to the AS whenever the video representation changes, or the video stalls) was proposed. But that also can cause too much signaling overhead if the video representation changes or video stalls are quite frequent.

## RVQoE priority w.r.t. legacy QoE

The RAN decides whether RVQOE measurement collection and reporting is activated

ID used to identify QoE measurements is reused for identifying the RVQoE measurements

RVQoE collection can be configured only if legacy QoE measurements are configured for the same service type

RVQoE report is provided inside a dedicated IE, outside the QoE report container

Send an LS asking RAN2 whether RVQOE metric can be reported over high-priority SRB (SRB1, SRB3) or whether low-priority SRB (SRB4?) should be used

The gNB-CU may signal RVQoE report to gNB-DU over F1

FFS whether RVQoE and legacy QoE can be reported separately

Before we discuss the RVQOE metrics, RVQoE configuration/reporting and other aspects, we must establish some common understanding and principles for RVQOE. Two options are tabulated below.

| Option 1 (Qualcomm’s preference) | Option 2 |
| --- | --- |
| RVQOE has the same priority as legacy QoE. The main purpose of RVQOE is to expose QoE metrics to RAN and not primarily for enabling QoE aware real-time use cases. Similar to legacy QoE, RVQOE can be used for historical/statistical/non-real time use cases. | RVQOE has higher priority compared to legacy QoE. RVQOE metrics must be reported more frequently compared to legacy QoE for enabling real-time use cases (e.g., QoE aware scheduling decisions) |
| SRB4 can be reused for RVQoE (similar as legacy QoE) | RAN2 needs to support SRB1/2 (high priority SRB) for real-time use cases |
| RVQOE and legacy QoE are reported together | RVQOE and legacy QoE can be reported independently |

Although Option 2 offers more flexibility and supports real-time use cases, we think we should not complicate RVQOE configuration and reporting by adding a different periodicity and event trigger for RVQOE. Otherwise, we feel it would become too complicated. For example,

- Case 1: RVQOE periodicity (e.g.  1 sec) < legacy QoE periodicity (e.g., 2 sec)

  - AT command needs to be enhanced to provide RVQOE periodicity, so that UE APP provides RVQOE faster than the legacy QoE report. UE APP needs to maintain two periodicities and should buffer QOE measurements accordingly.

- Case 2: RVQOE periodicity (e.g., 2sec) > legacy QoE periodicity (e.g., 1 sec)

  - This might not be a typical case if RVQOE has higher priority. But RAN has no idea what legacy QoE reporting periodicity is. In this case, it is not clear whether RVQOE should be reported based on the last 1 sec or last 2 sec? Similar as case 1, UE APP would need to buffer QoE measurement results longer than usual for the purpose of RVQOE or maintain two separate buffers.

- Case 3: Different event triggers for RVQOE and legacy QoE  complicated signaling

For the sake of simplicity, we therefore prefer Option 1.

Observation 3: If RVQoE and legacy QoE are to be reported separately, it might increase the processing complexity and storage requirements at UE Application to handle RVQoE and legacy QoE independently

Proposal 4: RVQOE has the same priority as legacy QoE. The main purpose of RVQOE is to expose QoE metrics to RAN and not for enabling QoE aware real-time use cases

## RVQoE configuration

RAN generates the RVQoE measurement configuration

The RVQoE configuration can be configured flexibly (i.e., it is not fixed)

The RVQoE configuration sent to UE should contain the metrics to be reported, as a mandatory IE

FFS whether to include the following in RVQoE configuration: Sample percentage, Start Time, Duration, Reporting Interval for periodic case, Triggering Event, DRB information (or QoS flow information) to be reported

Proposal 5: RVQoE and legacy QoE can be configured independently i.e., RVQoE can be configured at a later time after configuring legacy QoE by using the same measConfigAppLayerID

Proposal 6: NG-RAN can release a list of RVQOE configurations while not releasing corresponding legacy QoE configurations

Proposal 7: If the legacy QoE configuration is released/paused/resumed, the corresponding RVQOE configuration is released/paused/resumed as well

Proposal 8: A common indicator is used to pause/resume both legacy QoE and RVQoE configurations i.e., there is no support to pause/resume a list of RVQOE configurations while not pausing/resuming corresponding legacy QoE configurations

Proposal 9: RVQoE configuration should simply include the RVQoE metrics to be reported. There is no need to define a separate periodicity or event trigger for RVQoE or other metrics such as Sample percentage, Start Time, Duration

## RVQoE report

For the sake of simplicity as discussed in section 2.2,  RVQoE and legacy QoE should be reported together

Proposal 10: RVQOE and legacy QoE should be reported together

FFS whether PDU session information should and can be included in the RVQoE report

It was discussed last meeting whether reporting PDU session information (e.g., DRB ID, PDU session ID, QoS Flow ID) in RVQoE report can enable RAN to optimize parameters configured for a radio bearer, QoS flow to DRB mapping, etc. However, it is to be noted that QoE support for MR-DC is not supported in Rel-17 i.e., QoE is always configured and reported over MN. Therefore, we don’t see any benefits in reporting PDU session information in RVQoE report.

Observation 4: QoE support for MR-DC is not supported in Rel-17 i.e., QoE is always configured and reported over MN in Rel-17

Proposal 11: There is no need to include PDU session information (e.g., DRB ID, PDU session ID, QoS Flow ID) in the RVQoE report

## Mobility Support for RVQoE

FFS whether the RVQoE configuration should be propagated from the source to target node upon mobility in RRC_CONNECTED and during context retrieval upon resumption from RRC_INACTIVE. The target/new RAN node may assemble a different RVQoE configuration

Proposal 12: RVQoE configuration can be propagated from the source to target node upon mobility in RRC_CONNECTED and during context retrieval upon resumption from RRC_INACTIVE. The target/new RAN node may assemble a different RVQoE configuration

FFS whether the RVQoE report be signalled from the target to the source node after a successful handover

There might be scenarios where the RVQoE report for the source node is sent by the UE after a handover. In such cases, RVQoE report sent by the UE can be used by the source node to optimize the network configuration and to evaluate the handover performance. Therefore, there are merits in propagating the RVQoE report from the target to the source node after a successful handover.

Proposal 13: RVQoE report can be signaled from the target to the source node after a successful handover

## How does NG-RAN know which RVQoE metrics are available to be collected?

FFS whether the OAM indicates to the RAN, outside the QoE configuration container, which RVQoE metrics are available for the RAN to configure the UE to collect, or the RAN can conclude this from the UE capability indication and the service type configured for the UE

RAN can figure out by itself which RVQoE it can configure to the UE. RAN knows the UE capability for each RVQoE metric and should also know which service type is already configured at the UE via legacy QoE configuration. There is no need for OAM to indicate to the RAN, outside the QoE configuration container, which RVQoE metrics are available for the RAN to configure the UE to collect.

Proposal 14:  NG-RAN can conclude which RVQoE metrics are available to be collected from the UE via UE capability signaling and the service type configured for the UE (no need for OAM to configure this explicitly in the QoE configuration container)

## Multiple simultaneous RVQoE

Multiple simultaneous RVQoE measurements are supported

Observation 5: SA5 confirmed that multiple simultaneous legacy QoE measurements for the same service type can be supported

In light of Observation, we should also address whether multiple simultaneous RVQoE for the same service type needs to be supported.

For example, a sample RVQoE configuration can be as follows:

[

Service Type X, measConfigAppLayerID 1, RVQoE metrics = Buffer Level

Service Type X, measConfigAppLayerID 2, RVQoE metrics = Buffer Level

Service Type Y, measConfigAppLayerID 3, RVQoE metrics = Playout delay

]

It is possible that a UE configured with multiple simultaneous legacy QoE measurements for the same service type (e.g., measConfigAppLayerID 1 and 2) can be configured to provide RVQoE metrics as well.

Proposal 15: Multiple simultaneous RVQoE for the same service type can be supported

## RAN Visible QoE Values

For defining RVQOE values, it was discussed in SI phase whether we can derive simplified values from individual useful SA4-defined QoE metrics or combinations of these values in the form of e.g.:

- Numeric values on scale from 0 to x

- Binary flags

- Objective qualitative representations (“good QoE”, “moderate QoE”, “bad QoE”)

For the above to work, a formula to represent the QoE values should be defined e.g., a threshold “X” needs to be defined for categorizing good QoE (small buffer) vs. bad QoE (large buffer) as shown below:

| Metric | Unit | Formula for qualitative QoE metric |
| --- | --- | --- |
| Buffer level | milliseconds | Example: - Buffer level < X ms – small buffer - Buffer level >= X ms – large buffer |
| Playout Delay for media start-up | milliseconds | Example: - Playout Delay < Y ms – good QoE - Playout Delay >= Y ms – bad QoE |

Observation 6: Qualitative representation of QoE metrics in terms of a numerical value or an objective representation requires a model/function to be defined for each RAN visible QoE metric

There are three options which are possible for defining RVQOE values:

Option 1: Pre-defined formula in SA4 specs

- Formula/threshold for RVQOE values are pre-defined in the SA4 specifications

- UE APP computes (e.g., checks whether buffer level is small or large) and reports the RVQOE value to the NG-RAN

Option 2: Configurable by NG-RAN

- NG-RAN can configure the formula/threshold to UE APP via RRC and AT commands

- UE APP then computes and reports the RVQOE value based on the received configuration

Option 3: Implementation specific to NG-RAN

- NG-RAN receives the RVQOE metrics defined in section 2.1

- NG-RAN can compute the RVQOE values by own implementation (e.g., via thresholds internal to NG-RAN)

Option 3 seem to be the simplest and won’t need any SA4 spec impact. Whether option 1 or option 2 can be supported needs to be checked with SA4.

Proposal 16: RAN3 to down select from the 3 options before deciding whether and how to support RVQoE values:

- Option 1: Pre-defined formula in SA4 specs

- Option 2: Configurable by NG-RAN

- Option 3: Implementation specific to NG-RAN

## Other topics

Proposal 17: Per-slice RVQOE and alignment of RVQOE with radio-related measurements can be discussed post progress on the corresponding topics for the legacy QoE.

# Conclusion

Proposal 1: Buffer level and Playout Delay can be considered as RVQOE metrics for DASH and VR services.

Observation 1: Defining a buffer level alarm indication instead of reporting an absolute buffer level seems to be under the scope of RVQoE values where UE can represent qualitative representation of RVQoE metrics.

Proposal 2: The discussion on whether to define a buffer level alarm should happen under RVQoE values as this is a qualitative representation of a RVQoE metric and not an absolute value.

Observation 2: Reporting the QoE metrics “VR Interaction Latency” and “Play List” by UE seems cumbersome as it includes a lot of information elements.

Proposal 3: VR Interaction Latency and Play List seems a lot of signalling overhead in RRC and should not be considered as RVQOE metrics for Rel-17

Observation 3: If RVQoE and legacy QoE are to be reported separately, it might increase the processing complexity and storage requirements at UE Application to handle RVQoE and legacy QoE independently

Proposal 4: RVQOE has the same priority as legacy QoE. The main purpose of RVQOE is to expose QoE metrics to RAN and not for enabling QoE aware real-time use cases

Proposal 5: RVQoE and legacy QoE can be configured independently i.e., RVQoE can be configured at a later time after configuring legacy QoE by using the same measConfigAppLayerID

Proposal 6: NG-RAN can release a list of RVQOE configurations while not releasing corresponding legacy QoE configurations

Proposal 7: If the legacy QoE configuration is released/paused/resumed, the corresponding RVQOE configuration is released/paused/resumed as well

Proposal 8: A common indicator is used to pause/resume both legacy QoE and RVQoE configurations i.e., there is no support to pause/resume a list of RVQOE configurations while not pausing/resuming corresponding legacy QoE configurations

Proposal 9: RVQoE configuration should simply include the RVQoE metrics to be reported. There is no need to define a separate periodicity or event trigger for RVQoE or other metrics such as Sample percentage, Start Time, Duration

Proposal 10: RVQOE and legacy QoE should be reported together

Observation 4: QoE support for MR-DC is not supported in Rel-17 i.e., QoE is always configured and reported over MN in Rel-17

Proposal 11: There is no need to include PDU session information (e.g., DRB ID, PDU session ID, QoS Flow ID) in the RVQoE report

Proposal 12: RVQoE configuration can be propagated from the source to target node upon mobility in RRC_CONNECTED and during context retrieval upon resumption from RRC_INACTIVE. The target/new RAN node may assemble a different RVQoE configuration

Proposal 13: RVQoE report can be signaled from the target to the source node after a successful handover

Proposal 14:  NG-RAN can conclude which RVQoE metrics are available to be collected from the UE via UE capability signaling and the service type configured for the UE (no need for OAM to configure this explicitly in the QoE configuration container)

Observation 5: SA5 confirmed that multiple simultaneous legacy QoE measurements for the same service type can be supported

Proposal 15: Multiple simultaneous RVQoE for the same service type can be supported

Observation 6: Qualitative representation of QoE metrics in terms of a numerical value or an objective representation requires a model/function to be defined for each RAN visible QoE metric

Proposal 16: RAN3 to down select from the 3 options before deciding whether and how to support RVQoE values:

- Option 1: Pre-defined formula in SA4 specs

- Option 2: Configurable by NG-RAN

- Option 3: Implementation specific to NG-RAN

Proposal 17: Per-slice RVQOE and alignment of RVQOE with radio-related measurements can be discussed post progress on the corresponding topics for the legacy QoE.

# 4. References

# 5. Appendix

| RVQOE metric | Description | Potential benefits to RAN |
| --- | --- | --- |
| Buffer Level | Indicates the playout duration for which media data of all active media components is available starting from the current playout time. A list of buffer occupancy level measurements during playout at normal speed. Level of the buffer is included in milliseconds. Time of the measurement of the buffer level is also included. Annex D.4.5 in ISO/IEC 23009-1 defines the metrics for buffer level status events as follows: | Better mobility decisions at RAN based on buffer level (e.g., the selection of HO type between legacy HO, DAPS HO or CHO), leading to seamless connectivity and a lower risk of video stalling Assists NG-RAN to adjust the resource allocation for the UE. For example, the base station will consider scheduling more radio resources to those streams which reports higher buffer level so as to guarantee the stream will be processed properly. |
| Buffer level Alarm | Buffer level corresponds to the DL buffer at the UE application client, indicating the "playout duration for which media data of all active media components is available starting from the current playout time". In normal operation, when the buffer level falls below a given threshold, an end-to-end flow control in the client will request further data to be sent from the application server. If the application server, transport network or the RAN is congested, the client in the UE will not receive the needed information for continued streaming. In this situation we believe that availability of the absolute buffer level measurement in the RAN would not be easily exploitable. However it could be discussed whether there is a benefit that derived information, e.g. that the client determines that the buffer level falls below an "alarm" level (defined by a second threshold, lower than the one used for nominal E2E flow control) is available in the RAN. In that case the RAN could check the DL buffer in the PDCP layer for the corresponding DRB and prioritize it in case data is actually buffered in the RAN (which would mean that the bottle-neck is situated in the RAN). It is still an open question whether such additional mechanism will bring benefit on top of already existing mechanisms ensuring QoS for GBR bearers, or on top of mechanisms allocating resources while ensuring fairness between non-GBR bearers. | A UE going to be “starved” for DL streaming data could get higher scheduling priority, and that would help if DL data is already buffered in the CU-UP (PDCP buffer)). NG-RAN will have to compare the received information with own DL PDCP buffer status for the RB. The play-list may also serve a statistical purpose, redundant with buffer level information. |
| Playout Delay for Media Start-up | The playout delay for media start-up is measured as the time in milliseconds from the time instant of DASH player receives play-back-start trigger to the instant of media playout. This metric indicates the waiting time that the user experiences for media start-up. The metric is only logged at the time point when the media start-up happens. - If the MPD has been delivered earlier before the user clicks, it may include the process time of MPD, the fetch time of some media segments which are required for media presentation, the process time of segments, and the time for media decode and render to the user. - If no MPD has been fetched earlier, it also needs to add the fetch time of MPD. Key; Type. PlayoutDelayforMediaStartup; Integer. | RAN node can leverage this as a time budget to deliver the requested content without video stalling, while, at the same time, not over-allocating the precious radio resources to that service |
| Play List | A list of playback periods. A playback period is the time interval between a user action and whichever occurs soonest of the next user action, the end of playback or a failure that stops playback. Example for Playout - 5 seconds of initial stalling - 10 seconds playing with representation 1 - 15 seconds of stalling - 20 seconds playing with representation 2 - 30 seconds playing with representation 1 In last meeting, a simplified version was also proposed e.g., an indication from the application to the access stratum whenever the video representation changes or the video stalls | Presented video quality and video stalling statistics, can be valuable at RAN for radio resource partitioning and allocation among different users and services |
| VR Interaction latency | From TS 26.929, VR interaction latency is a key metric to evaluate the VR experience and it varies based on the VR content encoding, packing and delivery technologies, as well as VR device capabilities. The latency between user interaction and the corresponding response may involve interaction detection latency, DASH request latency, network delivery latency, receiver re-buffering latency and motion-to-photon latency. - VR Interaction detection latency - VR content access latency - VR decoding latency - VR rendering latency VR interaction latency has the same meaning as “Comparable quality viewport switching latency metric” in TS 26.118. The comparable quality viewport switching latency metric reports the latency and the quality-related factors when viewport movement causes quality degradations, such as when low-quality background content is briefly shown before the normal higher-quality is restored. Note that this metric is only relevant if the Advanced Video Media profile and region-wise packing is used. Also note that the metric currently does not report factors related to foveated rendering. Key; Type; Description. CompQualLatency; List; List of comparable quality viewport switching latencies. Entry; Object. firstViewport; ViewportItem; Specifies information about the first viewport. secondViewport; ViewportItem; Specifies information about the second viewport. worstViewport; ViewportItem; Specifies information about the worst viewport seen during the switch duration. time; Real-Time; Wall-clock time when the switch started. Mtime; Media-Time; Media presentation time when the switch started.. Latency; Integer; Specifies the switching delay in milliseconds.. Accuracy; Integer; Specifies the estimated accuracy of the latency metric in milliseconds. Cause; List; Specifies a list of possible causes for the latency.. Entry; Object. code; Enum; A possible cause for the latency. The value is equal to one of the following: - 0: Segment duration - 1: Buffer fullness - 2: Availability of comparable quality segment - 3: Timeout. |  |