3GPP TSG-RAN WG3 Meeting #114-e	R3-215021

# Online, November 1st – 11th 2021

Agenda Item:	15.2.1.2

Source:	Ericsson, CMCC, China Unicom

Title:	Distribution of QMC Job Attributes for Management Based QoE

Document for:	Agreement

# Discussion

In several RAN3 discussions (e.g., on mobility support) there were different assumptions with respect to whether all the RAN nodes within an Area Scope always receive the corresponding management based QoE measurement configuration. We notice that the above has not been explicitly stipulated in SA5 specifications. Hence, we propose to liaise SA5 and check their understanding on the issue.

Proposal: RAN3 to liaise SA5 asking whether the OAM sends all the attributes of the QMC Job to all the RAN nodes included in the Area Scope.

# Annex: Draft LS to SA5

3GPP TSG-RAN WG3 Meeting #114-e	R3-21xxxx

# Online, November 1st – 11th 2021

Title:	[DRAFT] LS on Distribution of QMC Job Attributes for Management Based QoE

Release:	Rel-17

Work Item:	NR_QoE

Source:	Ericsson [to be RAN3]

To:	SA5

Cc:	RAN2, SA4

Contact Person:

#### Name:	Filip Barac

Tel. Number:	+46730958830

E-mail Address:	filip.barac@ericsson.com

Attachments:	-

RAN3 is discussing mobility aspects for management based QoE and noted that the following Attributes of a QMCJob are used in the activateAreaQMCJob, as specified in TS 28.308, clause 5.3.1.2:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| serviceType | M | T | T | T | F |
| areaScope | M | T | T | T | F |
| qoECollectionEntityAddress | M | T | T | T | F |
| pLMNTarget | M | T | T | T | F |
| qMCTarget | M | T | T | T | F |
| qoEReference | M | T | T | T | F |
| qMCConfigurationFile | M | T | T | T | F |

RAN3 would like to ask SA5 the following:

Q1) In case of management-based activation in LTE, will the OAM send all the attributes of the QMC Job to all the RAN nodes included in the Area Scope?

Q2) Is the LTE approach wrt attributes of the QMC Job going to be reused for NR?

To 3GPP SA5

ACTION:

RAN3 respectfully asks SA5 to provide feedback on the questions above.

3. Date of next TSG RAN WG3 meetings:

RAN3#114bis-e	 January17th - 26th 2022		Online