3GPP TSG-RAN WG3 Meeting #114-e	R3-215035

E-meeting, 1-11 Nov 2021

Title: 	Discussion on CSI-RS configuration request

Source: 	Huawei, China Telecom, China Unicom

Agenda item:	31.2.6

Document Type:	Discussion

1. Introduction

There was an open issue for the CSI-RS configuration exchange topic at last meeting:

The possibility to request CSI-RS info by the source node to be continued.

This contribution provides an option to enable the CSI-RS information request between neighbour gNBs.

2. Discussion

In current X2AP and XnAP specifications, the initiate node can request the peer neighbour gNB to only reply certain cells’ served cell information by indicating the NR CGIs of the requested NR cells in X2/Xn SETUP REQUEST, X2/Xn CONFIGURATION UPDATE messages.

For the CSI-RS configuration request, a similar solution can be applied, which means that the initiate node indicates the list of NR-CGIs for which the CSI-RS configuration information are required in X2/Xn SETUP REQUEST, X2/Xn CONFIGURATION UPDATE messages.

The tentative spec changes are as below.

9.1.3.1	XN SETUP REQUEST

This message is sent by a NG-RAN node to a neighbouring NG-RAN node to transfer application data for an Xn-C interface instance.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Global NG-RAN Node ID | M |  | 9.2.2.3 |  | YES | reject |
| TAI Support List | M |  | 9.2.3.20 | List of supported TAs and associated characteristics. | YES | reject |
| AMF Region Information | M |  | 9.2.3.83 | Contains a list of all the AMF Regions to which the NG-RAN node belongs. | YES | reject |
| <<skip irrelevant part>> |  |  |  |  |  |  |
| Cell and Capacity Assistance Information E-UTRA | O |  | 9.2.2.42 | Contains E-UTRA cell related assistance information. | YES | ignore |
| CSI-RS Configuration Request Information | O |  | 9.2.2.x |  | YES | ignore |

9.2.2.x	CSI-RS Configuration Request Information

The CSI-RS Configuration Request Information IE is used by the NG-RAN node to request CSI-RS configuration information about NR cells.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Request NR Cells List |  | 1 .. < maxnoofCellsinNG-RAN node> |  |  |
| >Request NR Cells Item |  |  |  |  |
| >>NR CGI | M |  | 9.2.2.7 | NR cell for which CSI-RS configuration is requested. |

The benefits to support the CSI-RS configuration request/reply mechanism is that it enables the initiate node to only obtain the CSI-RS configurations of the cells that are neighbouring to the cells in the initiate node.

Proposal: RAN3 to consider above solution for the CSI-RS configuration request if it deems needed.

3. Conclusion

According to the discussion in this document, we propose:

Proposal: RAN3 to consider above solution for the CSI-RS configuration request if it deems needed.

The corresponding CRs are provided in [1] and [2].

4. Reference

- R3-215036	CSI-RS configuration request

- R3-215037	CSI-RS configuration request