3GPP TSG-RAN WG3 #114 electronic	R3-215046

Online, 1-11 Nov, 2021

Agenda Item:	22.2.4

Source:	CATT

Title:	TP for 38.463 on MBS Bearer Management at E1 interface

Document for:	Discussion, Decision

1	Introduction

In last RAN3 meeting, there were some progress on MBS Context management over E1, the following conclusion is still from RAN3#112-e and RAN3#113-e meeting:

-	For broadcast, introduce gNB-CU-CP triggered E1AP procedures: MBS Bearer Setup, MBS Bearer Modification, MBS Bearer Release. Detailed naming FFS.

-	For broadcast, the shared NG-U tunnel is established during the CU-CP triggered E1AP: MBS Bearer Setup procedure. The IP multicast address could be included in the E1AP: MBS Bearer Setup Request, and the unicast transport DL NG-U GTP-U address could be included in the E1AP: MBS Bearer Setup Response.

In this contribution, we provide some proposals on name and content for E1AP and the corresponding TPs is annexed below.

2	Discussion

According to the previous RAN3 meetings, for the gNB-CU-CP triggered E1AP procedures, there are three names to choose: MBS Bearer Context Setup, MBS Session Resource Context Setup or MBS Session Resource Setup. In fact, there are no essential differences among them. The first name is derived from the legacy E1AP naming design of PDU session resource, then for MBS session resource, Naturally, it can be called as MBS bearer context. But the latter two are to better reflect the characteristics of MBS Session resource. Thus, we slightly tend to continue the existing naming design of E1AP, i.e, the MBS Bearer Context Setup is used to define E1AP signaling procedures.

Proposal 1: Use the MBS Bearer Context to define E1AP signaling procedures for broadcast session

For the specific content, there is not much agreements of detail, the only agreement is about the transport address information, but this cannot be directly reflected in the Draft TP, because the upper level IE to which it belongs has not been determined.

To further promote the progress of WID, it is better to define the basic signaling procedure, as well as the table framework, specifically, for MBS Bearer Setup, MBS Bearer Modification, and MBS Bearer Release procedure, three class 1 procedures should be defined respectively.

Proposal 2: For broadcast session, at least three class 1 procedures need to be specified in E1AP, i.e, MBS bearer context setup request/response/failure procedure; MBS bearer context modification request/response/failure procedure; MBS bearer context release command/complete procedure

Accordingly, the draft TP for E1AP is annexed in chapter 5.

3	Conclusion

In the previous sections we made the following observations and proposals:

Proposal 1: Use the MBS Bearer Context to define E1AP signaling procedures for broadcast session

Proposal 2: For broadcast session, at least three class 1 procedures need to be specified in E1AP, i.e, MBS bearer context setup request/response/failure procedure; MBS bearer context modification request/response/failure procedure; MBS bearer context release command/complete procedure

4	Reference

- RP-201038    “New Work Item on NR Multicast and Broadcast Services”

- R3-214307	Summary of discussion on MBS Bearer Management over F1/E1, HUAWEI

5	Text Proposals for stage 3

5.1	TP for TS 38.463

--------------------------------Start of the First Change-----------------------------

8	E1AP procedures

8.1	List of E1AP Elementary Procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

Table 1: Class 1 procedures

| Elementary Procedure |  | Initiating Message |  | Successful Outcome |  | Unsuccessful Outcome |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Response message |  | Response message |  |
| Reset |  | RESET |  | RESET ACKNOWLEDGE |  |  |  |
| gNB-CU-UP E1 Setup |  | GNB-CU-UP E1 SETUP REQUEST |  | GNB-CU-UP E1 SETUP RESPONSE |  | GNB-CU-UP E1 SETUP FAILURE |  |
| gNB-CU-CP E1 Setup |  | GNB-CU-CP E1 SETUP REQUEST |  | GNB-CU-CP E1 SETUP RESPONSE |  | GNB-CU-CP E1 SETUP FAILURE |  |
| gNB-CU-UP Configuration Update |  | GNB-CU-UP CONFIGURATION UPDATE |  | GNB-CU-UP CONFIGURATION UPDATE ACKNOWLEDGE |  | GNB-CU-UP CONFIGURATION UPDATE FAILURE |  |
| gNB-CU-CP Configuration Update |  | GNB-CU-CP CONFIGURATION UPDATE |  | GNB-CU-CP CONFIGURATION UPDATE ACKNOWLEDGE |  | GNB-CU-CP CONFIGURATION UPDATE FAILURE |  |
| E1 Release |  | E1 RELEASE REQUEST |  | E1 RELEASE RESPONSE |  |  |  |
| Bearer Context Setup |  | BEARER CONTEXT SETUP REQUEST |  | BEARER CONTEXT SETUP RESPONSE |  | BEARER CONTEXT SETUP FAILURE |  |
| Bearer Context Modification (gNB-CU-CP initiated) |  | BEARER CONTEXT MODIFICATION REQUEST |  | BEARER CONTEXT MODIFICATION RESPONSE |  | BEARER CONTEXT MODIFICATION FAILURE |  |
| Bearer Context Modification Required (gNB-CU-UP initiated) |  | BEARER CONTEXT MODIFICATION REQUIRED |  | BEARER CONTEXT MODIFICATION CONFIRM |  |  |  |
| Bearer Context Release (gNB-CU-CP initiated) |  | BEARER CONTEXT RELEASE COMMAND |  | BEARER CONTEXT RELEASE COMPLETE |  |  |  |
| Resource Status Reporting Initiation |  | RESOURCE STATUS REQUEST |  | RESOURCE STATUS RESPONSE |  | RESOURCE STATUS FAILURE |  |
| IAB UP TNL Address Update |  | IAB UP TNL ADDRESS UPDATE |  | IAB UP TNL ADDRESS UPDATE ACKNOWLEDGE |  | IAB UP TNL ADDRESS UPDATE FAILURE |  |
| MBS Bearer Context Setup |  | MBS BEARER CONTEXT SETUP REQUEST |  | MBS BEARER CONTEXT SETUP RESPONSE |  | MBS BEARER CONTEXT SETUP FAILURE |  |
| MBS Bearer Context Modification (gNB-CU-CP initiated) |  | MBS BEARER CONTEXT MODIFICATION REQUEST |  | MBS BEARER CONTEXT MODIFICATION RESPONSE |  | MBS BEARER CONTEXT MODIFICATION FAILURE |  |
| MBS Bearer Context Release (gNB-CU-CP initiated) |  | MBS BEARER CONTEXT RELEASE COMMAND |  | MBS BEARER CONTEXT RELEASE COMPLETE |  |  |  |

Table 2: Class 2 procedures

| Elementary Procedure |  |  | Message |  |  |
| --- | --- | --- | --- | --- | --- |
| Error Indication |  |  | ERROR INDICATION |  |  |
| Bearer Context Release Request (gNB-CU-UP initiated) |  |  | BEARER CONTEXT RELEASE REQUEST |  |  |
| Bearer Context Inactivity Notification |  |  | BEARER CONTEXT INACTIVITY NOTIFICATION |  |  |
| DL Data Notification |  |  | DL DATA NOTIFICATION |  |  |
| UL Data Notification |  |  | UL DATA NOTIFICATION |  |  |
| Data Usage Report |  |  | DATA USAGE REPORT |  |  |
| gNB-CU-UP Counter Check |  |  | GNB-CU-UP COUNTER CHECK |  |  |
| gNB-CU-UP Status Indication |  |  | GNB-CU-UP STATUS INDICATION |  |  |
| MR-DC Data Usage Report |  |  | MR-DC DATA USAGE REPORT |  |  |
| Trace Start |  |  | TRACE START |  |  |
| Deactivate Trace |  |  | DEACTIVATE TRACE |  |  |
| Resource Status Reporting |  |  | RESOURCE STATUS UPDATE |  |  |
| Early Forwarding SN Transfer |  |  | EARLY FORWARDING SN TRANSFER |  |  |
| GNB-CU-CP Measurement Results Information |  |  | GNB-CU-CP MEASUREMENT RESULTS INFORMATION |  |  |

------------------------------------------Next Change---------------------------------------

8.3.X	MBS Bearer Context Setup

8.3.X.1	General

The purpose of the MBS Bearer Context Setup procedure is to allow the gNB-CU-CP to establish a MBS bearer context in the gNB-CU-UP.

Editor’s Note: Further specification text is FFS.

8.3.X.2	Successful Operation

Figure 8.3.X.2-1: MBS Bearer Context Setup procedure: Successful Operation

The gNB-CU-CP initiates the procedure by sending MBS BEARER CONTEXT SETUP REQUEST message to the gNB-CU-UP. If the gNB-CU-UP succeeds to establish the MBS bearer context, it replies to the gNB-CU-CP with MBS BEARER CONTEXT SETUP RESPONSE.

Editor’s Note: Further specification text is FFS.

8.3.X.3	Unsuccessful Operation

Figure 8.3.X.3-1: MBS Bearer Context Setup procedure: Unsuccessful Operation.

If the gNB-CU-UP cannot establish the requested MBS bearer context it shall consider the procedure as failed and respond with the MBS BEARER CONTEXT SETUP FAILURE message and an appropriate cause value.

Editor’s Note: Further specification text is FFS.

8.3.X.4	Abnormal Conditions

Editor’s Note: Further specification text is FFS.

8.3.Y	MBS Bearer Context Modification (gNB-CU-CP initiated)

8.3.Y.1	General

The purpose of the MBS Bearer Context Modification procedure is to allow the gNB-CU-CP to modify a MBS bearer context in the gNB-CU-UP.

Editor’s Note: Further specification text is FFS.

8.3.Y.2	Successful Operation

Figure 8.3.Y.2-1: MBS Bearer Context Modification procedure: Successful Operation

The gNB-CU-CP initiates the procedure by sending MBS BEARER CONTEXT MODIFICATION REQUEST message to the gNB-CU-UP. If the gNB-CU-UP succeeds to modify the MBS bearer context, it replies to the gNB-CU-CP with MBS BEARER CONTEXT MODIFICATION RESPONSE.

Editor’s Note: Further specification text is FFS

8.3.Y.3	Unsuccessful Operation

Figure 8.3.Y.3-1: MBS Bearer Context Modification procedure: Successful Operation

If the gNB-CU-UP cannot successfully perform any of the requested MBS bearer context modifications, it replies to the gNB-CU-CP with MBS BEARER CONTEXT MODIFICATION FAILURE and appropriate cause value..

Editor’s Note: Further specification text is FFS

8.3.Y.4	Abnormal Conditions

Editor’s Note: Further specification text is FFS

8.3.Z	MBS Bearer Context Release (gNB-CU-CP initiated)

8.3.Z.1	General

The purpose of the MBS Bearer Context Release procedure is to allow the gNB-CU-CP to command the release of all MBS bearer context in the gNB-CU-UP.

Editor’s Note: Further specification text is FFS

8.3.Z.2	Successful Operation

Figure 8.3.Z.2-1: MBS Bearer Context Modification procedure: Successful Operation

The gNB-CU-CP initiates the procedure by sending the MBS BEARER CONTEXT RELEASE COMMAND message to the gNB-CU-UP. The gNB-CU-UP replies with the MBS BEARER CONTEXT RELEASE COMPLETE message.

Upon reception of the MBS BEARER CONTEXT RELEASE COMMAND message, the gNB-CU-UP shall release all related signalling and user data transport resources and reply with the MBS BEARER CONTEXT RELEASE COMPLETE message.

Editor’s Note: Further specification text is FFS

8.3.Z.3	Abnormal Conditions

Editor’s Note: Further specification text is FFS

------------------------------------------Next Change---------------------------------------

9.2.X.1	MBS BEARER CONTEXT SETUP REQUEST

This message is sent by the gNB-CU-CP to request the gNB-CU-UP to setup a MBS bearer context.

Direction: gNB-CU-CP  gNB-CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| TBD |  |  |  |  |  |  |
| MBS Session ID | M |  | FFS |  | YES | reject |
| MRB Setup List |  | 1 |  |  | YES | reject |
| >MRB To Setup Item |  | 1..<maxnoofMRBs> |  |  | - | - |
| >> TBD | M |  | FFS |  | - | - |
| >>QoS Flows Information To Be Setup | M |  | FFS |  | - | - |
| TBD |  |  |  |  |  |  |

9.2.X.2	MBS BEARER CONTEXT SETUP RESPONSE

This message is sent by the gNB-CU-UP to confirm the setup of the requested MBS bearer context.

Direction:gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| TBD |  |  |  |  |  |  |
| MBS Session ID | M |  | FFS |  | YES | reject |
| MRB Setup List |  | 1 |  |  | YES | reject |
| >MRB Setup Item |  | 1..<maxnoofMRBs> |  |  | - | - |
| TBD |  |  |  |  |  |  |
| MRB Failed List |  | 0.. 1 |  |  | YES | reject |
| >MRB Failed Item |  | 1..<maxnoofMRBs> |  |  | - | - |
| >> TBD | M |  | FFS |  | - | - |
| TBD |  |  |  |  |  |  |

9.2.X.3	MBS BEARER CONTEXT SETUP FAILURE

This message is sent by the gNB-CU-UP to indicate that the setup of the MBS bearer context was unsuccessful.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| TBD. |  |  |  |  |  |  |
| MBS Session ID | M |  | FFS |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

9.2.X.4	MBS BEARER CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU-CP to request the gNB-CU-UP to modify a MBS bearer context.

Direction: gNB-CU-CP  gNB-CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| TBD |  |  |  |  |  |  |
| MBS Session ID | M |  | FFS |  | YES | reject |
| MRB To Setup List |  | 0.. 1 |  |  | YES | reject |
| >MRB To Setup Item |  | 1..<maxnoofMRBs> |  |  | - | - |
| >> TBD | M |  | FFS |  | - | - |
| >>QoS Flows Information To Be Setup | M |  | FFS |  | - | - |
| MRB To Modify List |  | 0.. 1 |  |  | YES | reject |
| >MRB To Modify Item |  | 1..<maxnoofMRBs> |  |  | - | - |
| >> TBD | M |  | FFS |  | - | - |
| MRB To Remove List |  | 0.. 1 |  |  | YES | reject |
| >MRB To Remove Item |  | 1..<maxnoofMRBs> |  |  | - | - |
| >> TBD | M |  | FFS |  | - | - |
| TBD |  |  |  |  |  |  |  |  |  |  |  |  |  |

9.2.X.5	MBS BEARER CONTEXT MODIFICATION RESPONSE

This message is sent by the gNB-CU-UP to confirm the modification of the requested MBS bearer context.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| TBD |  |  |  |  |  |  |
| MBS Session ID | M |  | FFS |  |  |  |
| MRB Setup List |  | 0.. 1 |  |  | YES | reject |
| >MRB Setup Item |  | 1..<maxnoofMRBs> |  |  | - | - |
| >> TBD | M |  | FFS |  | - | - |
| MRB Failed List |  | 0.. 1 |  |  | YES | reject |
| >MRB Failed Item |  | 1..<maxnoofMRBs> |  |  | - | - |
| >> TBD | M |  | FFS |  | - | - |
| MRB Modified List |  | 0.. 1 |  |  | YES | reject |
| >MRB Modified Item |  | 1..<maxnoofMRBs> |  |  | - | - |
| >> TBD | M |  | FFS |  | - | - |
| MRB Failed to Modify List |  | 0.. 1 |  |  | YES | reject |
| >MRB Failed to Modify Item |  | 1..<maxnoofMRBs> |  |  | - | - |
| >> TBD | M |  | FFS |  | - | - |
| TBD |  |  |  |  |  |  |  |  |  |  |  |  |  |

9.2.X.6	MBS BEARER CONTEXT MODIFICATION FAILURE

This message is sent by the gNB-CU-UP to indicate that the modification of the MBS bearer context was unsuccessful.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name |  | Presence |  | Range |  | IE type and reference |  | Semantics description |  | Criticality |  | Assigned Criticality |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Message Type |  | M |  |  |  | 9.3.1.1 |  |  |  | YES |  | reject |  |
| TBD |  |  |  |  |  |  |
| MBS Session ID | M |  | FFS |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

9.2.X.7	MBS BEARER CONTEXT RELEASE COMMAND

This message is sent by the gNB-CU-CP to command the gNB-CU-UP to release all MBS bearer context.

Direction: gNB-CU-CP  gNB-CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| TBD |  |  |  |  |  |  |
| MBS Session ID | M |  | FFS |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |

9.2.X.8	MBS BEARER CONTEXT RELEASE COMPLETE

This message is sent by the gNB-CU-UP to confirm the release of all MBS bearer context.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| TBD |  |  |  |  |  |  |
| MBS Session ID | M |  | FFS |  | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

- -------------------------------------End of the Changes----------------------------------