3GPP TSG-RAN3 #114-e	R3-215158

1st November – 11th November 2021

Online

Agenda Item:	9.3.1

Source:	Ericsson

Title:	How to release SCG configuration between MN and SN over X2

Document for:	Discussion, Decision

# Introduction

In the previous meeting the case of SCG removal/addition was discussed again and some agreements were made. In the following we will further elaborate on the remaining open issues. Based on our analysis we will bring forward our proposals.

# Discussion

In the previous meeting the issue of how to indicate from MN to SN in X2 that an SCG release or addition occurred was discussed. The following agreement was captured:

RAN3 agrees to reuse the EN-DC Resource Configuration IE and clarification on this IE is needed

The following also was minuted in the chairman’s notes:

Further discussion on the details to be continued…

In the following we will discuss the details of reusing the EN-DC Resource Configuration IE.

In TS 36.423 we read:

### 9.2.108	EN-DC Resource Configuration

This IE contains the EN-DC resource configuration for an E-RAB, indicating the presence of PDCP at the en-gNB and Lower Layers at MCG and SCG.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDCP at SgNB | M |  | ENUMERATED (present, not present) |  | – |  |
| MCG resources | M |  | ENUMERATED (present, not present) |  | – |  |
| SCG resources | M |  | ENUMERATED (present, not present) |  | – |  |

In order to use it as an indicator for SCG removal, a clarification is needed in the procedural text. The addition in text would need to state that if the SCG resources for all the E-RABs of a UE are set to not present in the EN-DC Resource Configuration IE, then this implies that the SCG is removed.

- Based on the above the following is proposed:

Proposal 1: clarify in the procedural text in TS 36.423 that if the SCG resources for all the E-RABs of a UE are set to not present in the EN-DC Resource Configuration IE, then this implies that the SCG is removed.

# Conclusion

### In this contribution the issue of SCG removal and corresponding communication between MN and SN in EN-DC has been discussed and the following proposals were made:

Proposal 1: clarify in the procedural text in TS 36.423 that if the SCG resources for all the E-RABs of a UE are set to not present in the EN-DC Resource Configuration IE, then this implies that the SCG is removed.

- CRs reflecting the proposal above are available in R3-215177 and in R3-215178.

# References