3GPP TSG-RAN WG3 Meeting #114-e	R3-215973

E-meeting, 1-11 Nov 2021

Agenda item:	  22.2.2

Source:	  Lenovo, Motorola Mobility, Huawei, Qualcomm Incorporated

Title:		(TP to TS 38.423 BL CR) RAN Multicast Group Paging

Document for:		Other

1	Introduction

In this contribution, the text proposal are provided for the XnAP BL CR according to the proposal in [1]；

Proposal 6: The XnAP: RAN MULTICAST GROUP PAGING message carries the following information:

- MBS Session ID

- RAN Paging Area

- (Optional) Paging UE List {UE Identity Index value, DRX value, RAN Paging Area}

[1] R3-215138 Consideration on Multicast Session Management. Huawei, CBN, Lenovo, Motorola Mobility, Qualcomm Incorporated, China Unicom, China Telecom.

2	Text Proposal

----------------------------------------------------------Next Change--------------------------------------------------------------

X.X.X	RAN Multicast Group Paging

X.X.X.1	General

The purpose of the RAN Multicast Group Paging procedure is to enable the NG-RAN node1 to request paging of UEs that have joined a MBS Session in the NG-RAN node2.

The procedure uses non UE-associated signalling.

X.X.X.2	Successful operation

Figure 8.2.5.2-1: RAN Multicast Group Paging: successful operation

The RAN Multicast Group Paging procedure is triggered by the NG-RAN node1 by sending the RAN MULTICAST GROUP PAGING message to the NG-RAN node2, in which the necessary information e.g. the MBS Session ID IE and the UE Identity Index List IE associated with the MBS session should be provided.

When available, the NG-RAN node1 shall include the UE Specific DRX IE in the RAN MULTICAST GROUP PAGING message towards the NG-RAN node2. If the UE specific DRX IE is included in the RAN MULTICAST GROUP PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 38.304 [33].

----------------------------------------------------------Next Change  --------------------------------------------------------------

X.X.X.X	RAN MULTICAST GROUP PAGING

This message is sent by the NG-RAN node1 to NG-RAN node2 to page UEs for an multicast session.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| MBS Session ID | M |  | FFS |  | YES | ignore |
| UE Identity Index List |  | 0..1 |  |  | YES | reject |
| >UE Identity Index Item |  | 1 .. <maxnoofUEsofMBSPaging> |  |  | - | - |
| >> CHOICE UE Identity Index Value | M |  |  |  | - | - |
| >>>Length-10 |  |  |  |  |  |  |
| >>>>Index Length-10 | M |  | BIT STRING (SIZE(10)) | Coded as specified in TS 38.304 [33]. | – | - |
| >>UE Specific DRX | O |  | 9.2.3.143 | Includes the UE specific paging cycle as defined in TS 38.304 [33]. | - | - |
| >>UE Specific RAN Paging Area | O |  | RAN Paging Area 9.2.3.38 | Indicates UE specific RAN Paging Area. If the UE specific RAN Paging Area IE is included, the gNB-DU ignores the RAN Paging Area IE for this UE. |  |  |
| RAN Paging Area | O |  | 9.2.3.38 |  | YES | reject |
| Paging Priority (FFS) | O |  | 9.2.3.44 |  | YES | ignore |