3GPP TSG-RAN WG3 Meeting #114-e	R3-215973

E-meeting, 1-11 Nov 2021

Agenda item:	  22.2.2

Source:	  Lenovo, Motorola Mobility, Huawei, Qualcomm Incorporated

Title:		(TP to TS 38.423 BL CR) RAN Multicast Group Paging

Document for:		Other

1	Introduction

In this contribution, the text proposal are provided for the XnAP BL CR according to the proposal in [1]；

Proposal 6: The XnAP: RAN MULTICAST GROUP PAGING message carries the following information:

- MBS Session ID

- RAN Paging Area

- (Optional) Paging UE List {UE Identity Index value, DRX value, RAN Paging Area}

[1] R3-215138 Consideration on Multicast Session Management. Huawei, CBN, Lenovo, Motorola Mobility, Qualcomm Incorporated, China Unicom, China Telecom.

2	Text Proposal

----------------------------------------------------------Next Change--------------------------------------------------------------

X.X.X	RAN Multicast Group Paging

X.X.X.1	General

The purpose of the RAN Multicast Group Paging procedure is to enable the NG-RAN node1 to request paging of UEs that have joined a MBS Session in the NG-RAN node2.

The procedure uses non UE-associated signalling.

X.X.X.2	Successful operation

Figure 8.2.5.2-1: RAN Multicast Group Paging: successful operation

The RAN Multicast Group Paging procedure is triggered by the NG-RAN node1 by sending the RAN MULTICAST GROUP PAGING message to the NG-RAN node2.

If the RAN MULTICAST GROUP PAGING message includes the Paging DRX IE, the NG-RAN node2.shall, if supported, , use it according to TS 38.304 [33].

Editor’s Note:	Detailled parameterisation of the message is FFS.

----------------------------------------------------------Next Change  --------------------------------------------------------------

X.X.X.X	RAN MULTICAST GROUP PAGING

This message is sent by the NG-RAN node1 to NG-RAN node2 to page UEs for an multicast session.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| MBS Session ID | M |  | FFS |  | YES | ignore |
| UE Identity Index List |  | 0..1 |  |  | YES | reject |
| >UE Identity Index Item |  | 1 .. <maxnoofUEsofMBSPaging> |  |  | - | - |
| >> CHOICE UE Identity Index Value | M |  |  |  | - | - |
| >>>Length-10 |  |  |  |  |  |  |
| >>>>Index Length-10 | M |  | BIT STRING (SIZE(10)) | Coded as specified in TS 38.304 [33]. | – | - |
| >> Paging DRX | O |  | 9.2.3.143 | Includes the UE specific paging cycle as defined in TS 38.304 [33]. Editor’s Note: It is FFS that this IE is per UE or per MBS session. | - | - |
| Multicast RAN Paging Area | O |  | RAN Paging Area 9.2.3.38 | Editor’s Note: The type definition, the presence and the usage of this IE is type definition of this IE is FFS | YES | reject |
| Paging Priority (FFS) | O |  | 9.2.3.44 | Editor’s Note: The optional inclusion of this IE is FFS. | YES | ignore |