3GPP TSG-RAN WG3 #114-e							R3-216006

1 November – 11 November 2021

Online

Agenda Item:	10.2.1.4

Source:	ZTE (moderator)

Title:	Summary of Offline Discussion on CB: # SONMDT2_UEHistoryInfor

Document for:	Approval

- Introduction

CB: # SONMDT2_UEHistoryInfor

- How to retrieve SN UHI?

- Which node collects SN UHI?

- Correlation of MN and SN UHI

- Correlated MN and SN UHI from MN to SN is benefit?

- SN UHI information content

- Messages for UHI transfer

- Interface impact of SN UHI from UE

- Capture agreements and provide TPs if agreeable

(ZTE - moderator)

Summary of offline disc R3-215851 rev in R3-216006

It is proposed to divide the discussion into two phases:

-	Phase 1: Identify the issues to be discussed in RAN3

Deadline: Please provide your views by 4:00am UTC Friday November 5th

-	Phase 2: Further discussion to capture agreements and open issues

Deadline: Please provide your views by 10:00am UTC Tuesday November 9th

- For the Chairman’s Notes (Phase 2)

According to the discussion, moderator proposes the following agreements:

Correlated MN and SN UHI using a nested structure is transferred from MN to SN.

Only SN UHI is transferred from SN to MN.

UE History Information IE shall be included in the S-NODE ADDITION REQUEST and SGNB ADDITION REQUEST messages.

SN UHI, e.g. SCG UE History Information IE, shall be included in the S-NODE MODIFICATION REQUIRED and SGNB MODIFICATION REQUIRED if MN subscribes to PSCell changes.

Include SN UHI, e.g. SCG UE History Information IE, in the following messages.

- S-NODE CHANGE REQUIRED and SGNB CHANGE REQUIRED messages

- S-NODE MODIFICATION REQUEST ACKNOWLEDGE and SGNB MODIFICATION REQUEST ACKNOWLEDGE messages

- S-NODE RELEASE REQUEST ACKNOWLEDGE and SGNB RELEASE REQUEST ACKNOWLEDGE messages

- S-NODE RELEASE REQUIRED and SGNB RELEASE REQUIRED messages

MN shall correlate MN and SN UHI, FFS on how to correlate MN and SN UHI..

Open issues for next meeting:

Open issue 1: Whether/how to include Time spent without SCG or Time stamp in SN UHI.

Open issue 2: FFS how MN subscribes to PSCell changes from SN.

Open issue 3: FFS on the stage 2 description for SN UHI.

Open issue 4: FFS how to correlate MN and SN UHI.

- Phase 2 discussion

## IE design

### UHI IE design

In the phase 1 discussion, we have agreed to transfer correlated MN and SN from MN to MN. However, whether SN and/or MN UHI shall be transferred from MN to SN and vice versa is still FFS. According to the contributions, companies still have diverging comments on this issue. The reasons for each camp are listed as follows.

MN->SN

Only SN UHI [2][10][16]:

- MN UHI has never been considered as a needed information in the SN [2].

- MN UHI can not help SN optimize RRM or find the SN Ping pong handover problem triggered by SN [10].

- SN cannot choose PSCell according to PCell, even it receives the correlated MN/SN UHI. It means MN UHI is useless for SN [16].

Correlated MN and SN UHI [4][5][6][9]:

- It is easier to signal and maintain the same correlated UHI information (MN+SN UHI) back and forth [4].

- Some companies argue that the SN would also benefit from having the MN UHI [5].

- Correlated UHI is useful for SN to select suitable PSCell [6].

- It has advantages of optimizing the selection of PSCell, avoiding information loss, tracking time with no PSCell, and maintaining consistency with MHI [9].

SN->MN

Only SN UHI [2][5][9][10][16]:

- If MN UHI is not provided to the SN, the SN can’t send it back [2].

- There is no point in providing the MN history back to the MN [5].

- SN does not update or keep MN related UHI [9].

- It’s not mandatory that MN informs SN of PCell information. It means SN could not be aware of PCell change [16].

Correlated MN and SN UHI [4][6][10]:

- It is not easy for MN to know the correlation between PCell and PScell list if SN only provides SN UHI [4].

- If correlated MN and SN UHI is transferred from MN to SN, it is simple and straightforward to also transfer correlated MN and SN UHI from SN to MN [6].

From moderator’s point of view, since we already agreed to transfer correlated MN and SN from MN to MN, it is simple and straightforward to transfer correlated MN and SN UHI from MN to SN. But from SN to MN, there is no need to provide MN UHI back to SN since MN is aware of MN UHI and the MN provided by SN may be outdated.

Question 2-1: Do companies agree with the following proposals?

Proposal 2-1: Correlated MN and SN UHI using a nested structure is transferred from MN to SN.

Proposal 2-2: Only SN UHI is transferred from SN to MN.

| Company | Yes/No | Comment |
| --- | --- | --- |
| ZTE | Yes | Since come companies argue that it may be beneficial to provide MN UHI to SN and we already agreed to transfer correlated MN and SN UHI form MN to MN, we can agree to transfer correlated MN and SN UHI from MN to SN. However, we think that only SN UHI shall be sent from SN to MN since SN does not update MN UHI, and it may be more complicated to provide the outdated MN UHI back to MN. |
| Nokia | ? / Yes | Surely, from the SN only the SCG UHI is needed. The question arises about providing MCG UHI to the SN at the addition. Besides very general statements that “it can help SN”, there was no general acknowledgement that this is needed. So, we would prefer to send only the SCG UHI to the SN at the Addition. |
| Ericsson | Yes to both | While agreeing with this statement, I think that 1st proposal is not only about “correlated UHI is useful at SN”. We’ve already agreed that “Correlation of MN UHI and SN UHI is beneficial” and that it is “Realized via two-dimensional structure for UHI”. If correlation has to be done at some point, it is more effective to transmit the correlated UHI from source to target MN at HO. Otherwise correlation will have to be done again and again from the 1st entry at every HO. And if that’s the case, it is easier for the MN to send correlated UHI as received to SN at SN Addition, rather than extracting the SN UHI from the correlated UHI. |
| Qualcomm | P2-1: No strong view P2-2: OK | Proposal 2-1: OK with a correlated list. But considering that we have not exactly identified the value of MN UHI to SN (except that it helps in correlation and helps maintain a single list), we are OK to just send SN UHI as well from MN to SN to make progress. Proposal 2-2: OK |
| Lenovo and Motorola Mobility | Yes | Ok for Proposal 2-1 and Proposal 2-2 |
| CATT | Yes for Propsoal 2-1 | For Proposal 2-2, we think the question is on which node make the correlation ,MN or SN?Both of the solution could work.Hwoever,we have a preference to let SN make the correlation considering it is more effective. |
| Samsung | Proposal 2-1: No Proposal 2-2: Yes | Proposal 2-1: The value is not clear. It bring complexity for the SN. From SN point of view, it is simple to keep the information which is really needed. Proposal 2-2: There is no need to provide MN UHI back to SN since MN is aware of MN UHI and the MN UHI provided by SN may be outdated |
| Huawei | Yes | 2-1: If MN should anyway provide the aggregated UHI and will receive the aggregated UHI, it is most simple to just fwd this to SN. Otherwise MN has to reconstruct the information. 2-2 the only unknown info in the MN is the new SN part. So if SN also merge, it is not really useful. It is better to just send the new SN history collected in SN |

Moderator summary: For proposal 2-1, (5/8) companies say yes, (2/8) companies say no, (1/8) company say neutral. For proposal 2-2, (7/8) companies say yes, (1/8) companies say no.

For proposal 2-1, the opponents argue that SN does not need MN UHI, and this may bring complexity for the SN. From moderator’s point of view, one of MN and SN has to extract the SN UHI. Since source MN needs to provide correlated UHI to target MN, it is simple to just forward the correlated UHI to SN, and let SN further extract and keep the SN UHI. Moderator would suggest to follow majority’s view that correlated MN and SN UHI shall be transferred from MN to SN.

For proposal 2-2, only one company says no and prefers to let SN make the correlation. From moderator’s point of view, SN is not aware of PCell change which indicates SN may have outdated MN UHI. If SN performs the correlation and the outdated MN is provided to SN, MN may need to check the differences between up-to-date MN UHI and the outdated MN UHI, which will definitely complicate the situation since MN cannot perform correlation. Thus, it is more efficient to let MN perform the correlation and SN only provides SN UHI to MN.

Moderator would suggest the following proposals.

Proposal 3-1: Correlated MN and SN UHI using a nested structure is transferred from MN to SN.

Proposal 3-2: Only SN UHI is transferred from SN to MN.

The next important issue is about UHI IE design. We need to consider how to achieve correlated UHI IE design. Furthermore, we also need to design a SN UHI IE if we agree to only transfer SN UHI from SN to MN. According to the contributions, moderator would suggest the following enhancement.

- 9.2.3.64	UE History Information

The UE History Information IE contains information about cells that a UE has been served by in active state prior to the target cell. The overall mechanism is described in TS 36.300 [12].

NOTE:	The definition of this IE is aligned with the definition of the UE History Information IE in TS 38.413 [5].

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Last Visited Cell List |  | 1..<maxnoofCellsinUEHistoryInfo> |  | Most recent information is added to the top of this list | - |  |
| >Last Visited Cell Information | M |  | 9.2.3.65 |  | - |  |
| >Last Visited Secondary Cell List |  | 0..<maxnoofSecondaryCellsPerPrimaryCellinUEHistoryInfo> |  | List of cells configured as PSCells. Most recent PSCell related information is added to the top of the list. | YES | ignore |
| >>Last Visited Secondary Cell Information | M |  | 9.2.3.X | The PSCell related information. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinUEHistoryInfo | Maximum number of last visited cell information records that can be reported in the IE. Value is 16. |
| maxnoofSecondaryCellsPerPrimaryCellinUEHistoryInfo | Maximum number of last visited secondary cell information records that can be reported in the IE. Value is 16. |

- 9.2.3.X	Last Visited Secondary Cell Information

The Last Visited Secondary Cell Information may contain cell specific information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Last Visited Secondary Cell Information | M |  |  |  |
| >NG-RAN Cell |  |  |  |  |
| >>Last Visited Secondary NG-RAN Cell Information | M |  | 9.2.3.X1 |  |
| >E-UTRAN Cell |  |  |  |  |
| >>Last Visited Secondary E-UTRAN Cell Information | M |  | 9.2.3.X2 |  |

- 9.2.3.X1	Last Visited Secondary NG-RAN Cell Information

This IE contains information about a cell. In case of NR cell, this IE contains information about a set of NR cells with the same NR ARFCN for reference point A, and the Global Cell ID IE identifies one of the NR cells in the set. The information is to be used for RRM purposes.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Global Cell ID | M |  | Defined in TS 38.413 [5] |  |
| Time UE Stayed in Cell | M |  | INTEGER (0..4095) | The duration of time the UE stayed in the cell, or set of NR cells with the same NR ARFCN for reference point A, in seconds. If the duration is more than 4095s, this IE is set to 4095. |
| Time UE Stayed in Cell Enhanced Granularity | O |  | INTEGER (0..40950) | The duration of time the UE stayed in the cell, or set of NR cells with the same NR ARFCN for reference point A, in 1/10 seconds. If the duration is more than 4095s, this IE is set to 40950. |

- 9.2.3.X2	Last Visited Secondary E-UTRAN Cell Information

The Last Visited E-UTRAN Cell Information contains information about a cell that is to be used for RRM purposes

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Global Cell ID | M |  | Defined in TS 36.413 |  | - |  |
| Time UE stayed in Cell | M |  | INTEGER (0..4095) | The duration of the time the UE stayed in the cell in seconds. If the UE stays in a cell more than 4095s, this IE is set to 4095. | - |  |
| Time UE stayed in Cell Enhanced Granularity | O |  | INTEGER (0..40950) | The duration of the time the UE stayed in the cell in 1/10 seconds. If the UE stays in a cell more than 4095s, this IE is set to 40950. | YES | ignore |

- 9.2.3.Z SN UE History Information

The SN UE History Information IE contains information about the cells served by the secondary node in an active state.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Last Visited Secondary Cell List |  | 0..<maxnoofSecondaryCellsPerPrimaryCellinUEHistoryInfo> |  | List of cells configured as PSCells. Most recent PSCell related information is added to the top of the list. |
| >Last Visited Secondary Cell Information | M |  | 9.2.3.X | The PSCell related information. |

| Range bound | Explanation |
| --- | --- |
| maxnoofSecondaryCellsPerPrimaryCellinUEHistoryInfo | Maximum number of last visited secondary cell information records that can be reported in the IE. Value is 16. |

    - Parameters for SN UHI

In addition, majority companies propose to include “Time spent without SCG” or “Time stamp” in SN UHI to correctly judge SN Ping pong error and interpret the correlated information. According to the contributions, the following modifications are proposed to include “Time spent without SCG” or “Time stamp” respectively. From moderator’s point of view, these two parameters have the same function, and using “Time stamp” has less specification impact. Therefore, moderator would suggest to include “Time stamp” in SN UHI.

Time spent without SCG:

- 9.2.3.X	Last Visited Secondary Cell Information

The Last Visited Secondary Cell Information may contain cell specific information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE Last Visited Secondary Cell Information | M |  |  |  |  |  |
| >NG-RAN Cell |  |  |  |  |  |  |
| >>Last Visited Secondary NG-RAN Cell Information | M |  | 9.2.3.X1 |  |  |  |
| >E-UTRAN Cell |  |  |  |  |  |  |
| >>Last Visited Secondary E-UTRAN Cell Information | M |  | 9.2.3.X2 |  |  |  |
| >No Secondary Cell |  |  |  |  | YES | ignore |
| >>No Secondary Cell Information | M |  | OCTET STRING | FFS | YES | ignore |

Time stamp:

- 9.2.3.X1	Last Visited Secondary NG-RAN Cell Information

This IE contains information about a cell. In case of NR cell, this IE contains information about a set of NR cells with the same NR ARFCN for reference point A, and the Global Cell ID IE identifies one of the NR cells in the set. The information is to be used for RRM purposes.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Global Cell ID | M |  | Defined in TS 38.413 [5] |  |
| Time UE Stayed in Cell | M |  | INTEGER (0..4095) | The duration of time the UE stayed in the cell, or set of NR cells with the same NR ARFCN for reference point A, in seconds. If the duration is more than 4095s, this IE is set to 4095. |
| Time UE Stayed in Cell Enhanced Granularity | O |  | INTEGER (0..40950) | The duration of time the UE stayed in the cell, or set of NR cells with the same NR ARFCN for reference point A, in 1/10 seconds. If the duration is more than 4095s, this IE is set to 40950. |
| Time Stamp | O |  | OCTET STRING (SIZE(4)) | Defined as Time Stamp in TS 38.413[5] |

- 9.2.3.X2	Last Visited Secondary E-UTRAN Cell Information

The Last Visited E-UTRAN Cell Information contains information about a cell that is to be used for RRM purposes

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Global Cell ID | M |  | Defined in TS 36.413 |  | - |  |
| Time UE stayed in Cell | M |  | INTEGER (0..4095) | The duration of the time the UE stayed in the cell in seconds. If the UE stays in a cell more than 4095s, this IE is set to 4095. | - |  |
| Time UE stayed in Cell Enhanced Granularity | O |  | INTEGER (0..40950) | The duration of the time the UE stayed in the cell in 1/10 seconds. If the UE stays in a cell more than 4095s, this IE is set to 40950. | YES | ignore |
| Time Stamp | O |  | OCTET STRING (SIZE(4)) | Defined as Time Stamp in TS 38.413[5] | YES | ignore |

Question 2-2: Do companies agree with the following proposals?

Proposal 2-3: RAN3 shall use the IE design in section 3.1.1 for correlated MN and SN UHI.

Proposal 2-4: RAN3 shall use the IE design in section 3.1.1 for SN UHI if it is agreed to only transfer SN UHI from SN to MN.

Proposal 2-5: RAN3 shall include “Time stamp” in SN UHI.

Proposal 2-6: RAN3 shall use the IE design in section 3.1.2 to include “Time stamp” in SN UHI if it is agreed to include “Time stamp”.

Proposal 2-7: RAN3 shall use the IE design in section 3.1.2 to include “Time spent without SCG” in SN UHI if it is agreed to include “Time spent without SCG”.

| Company | Yes/No | Comment |
| --- | --- | --- |
| ZTE | Yes | Prefer using Time stamp since it has less specification impact. |
| Nokia | Partially yes | The SN UHI should rather be named “SCG UHI”, shouldn’t it? And it should be used also from the MN to the SN, as discussed above. Agree that a timestamp is better. |
| Ericsson | Partially ok | Looks ok for 2-3 and 2-4. Also, for Last Visited Secondary NG-RAN/E-UTRAN Cell Information IEs in 38.423, prefer to use OCTET STRING and references to 38.413 as for Last Visited Cell Information IE. If timestamp is replacing time spent without SCG, we need addition and release timestamps in order to perform correlation. It would also avoid having absolute timestamp, which adds complexity. |
| Qualcomm | P2-3 and P2-4: OK P2-5, P2-6 and P2-7: FFS | Proposal 2-3/2-4: I assume this proposal is showing an IE structure for MN-> SN (with both MN and SN UHI) and SN-> MN (just SN UHI) and not for MN->MN. Also, it seems that the moderator has used an independent list (not nested list) of MN and SN UHI, thereby not achieving “correlation”. If that’s the moderator’s intention, the IE structure looks OK. But this depends on conclusion from Proposal 2-1 and 2-2. Proposal 2-5/2-6/2-7: This can be discussed later. Also, what is the definition of Time stamp (timestamp when PSCell is released?) and how is it represented (UTC time?) |
| Lenovo and Motorola Mobility | Yes for Proposal 2-3 and Proposal 2-4 | If we agree to include “Time stamp” in SN UHI, the details need further discussion, e.g. timestamp is the absolute time when PSCell is added or released or both. |
| CATT | YES |  |
| Samsung | Partially ok | 2-3: in 9.2.3.X, choice with NR cell and E-UTRA cell may be enough. NG-RAN cell is really needed ? 2-4: SN UHI is also needed from the MN to the SN. The maximum number should not be restricted by PCell, right? maxnoofSecondaryCellsPerPrimaryCellinUEHistoryInfo => maxnoofSecondaryCellsinUEHistoryInfo 2-5/2-6/2-7: We don’t see they are useful. So far, the analysis is academic. Too many parameter may destroy with clear usage will destroy the function. |
| Huawei |  | Proposal 2-3: Yes Proposal 2-4: Yes Proposal 2-5/2-6/2-7: We prefer "stay time without SCG" in the correlated report. This is an intuitive way to represent the case that no SN is used and aligned with the normal UHI where we have stay time with SCG. Note that this information shall be possible to understand in all subsequent nodes. Note that the stay time is good because it is not so important to get the higher values correct (we can have a max time reporting) but it is important to be able to capture short times. Stay time is reset for each change and is therefore not sensitive to selecting the range. If we use time stamps, we encounter the problem about defining absolute time and also consider wrap-around (or creating a large ranges). If we use relative time we have to define the common starting point. We also have the problem of the receiving node having to reconstruct the sequence of events since the representation is different from the normal UHI. |

Moderator summary: Since companies still have diverging views on proposal 2-5/2-6/2-7, moderator would suggest leaving this issue to the next meeting.

Open issue 1: Whether/how to include Time spent without SCG or Time stamp in SN UHI.

For proposal 2-3 and 2-4, moderator thinks companies can reach a consensus on the skeleton of the designed IEs according to the comments, and some details may need further discussion. Thus, moderator would suggest the following IE design.

- 9.2.3.64	UE History Information

The UE History Information IE contains information about cells that a UE has been served by in active state prior to the target cell. The overall mechanism is described in TS 36.300 [12].

NOTE:	The definition of this IE is aligned with the definition of the UE History Information IE in TS 38.413 [5].

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Last Visited Cell List |  | 1..<maxnoofCellsinUEHistoryInfo> |  | Most recent information is added to the top of this list | - |  |
| >Last Visited Cell Information | M |  | 9.2.3.65 |  | - |  |
| >Last Visited Secondary Cell List |  | 0..<maxnoofSecondaryCellsPerPrimaryCellinUEHistoryInfo> |  | List of cells configured as PSCells. Most recent PSCell related information is added to the top of the list. | YES | ignore |
| >>Last Visited Secondary Cell Information | M |  | 9.2.3.X | The PSCell related information. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinUEHistoryInfo | Maximum number of last visited cell information records that can be reported in the IE. Value is 16. |
| maxnoofSecondaryCellsPerPrimaryCellinUEHistoryInfo | Maximum number of last visited secondary cell information records that can be reported in the IE. Value is FFS. |

- 9.2.3.X	Last Visited Secondary Cell Information

The Last Visited Secondary Cell Information may contain cell specific information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Last Visited Secondary Cell Information | M |  |  |  |
| >NG-RAN Cell |  |  |  |  |
| >>Last Visited Secondary NG-RAN Cell Information | M |  | FFS | FFS |
| >E-UTRAN Cell |  |  |  |  |
| >>Last Visited Secondary E-UTRAN Cell Information | M |  | FFS | FFS |

- 9.2.3.Y SN UE History Information

The SN UE History Information IE contains information about the cells served by the secondary node in an active state.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Last Visited Secondary Cell List |  | 0..<maxnoofSecondaryCellsPerPrimaryCellinUEHistoryInfo> |  | List of cells configured as PSCells. Most recent PSCell related information is added to the top of the list. |
| >Last Visited Secondary Cell Information | M |  | 9.2.3.X | The PSCell related information. |

| Range bound | Explanation |
| --- | --- |
| maxnoofSecondaryCellsPerPrimaryCellinUEHistoryInfo | Maximum number of last visited secondary cell information records that can be reported in the IE. Value is FFS. |

## Messages for UHI transfer

### SN addition request message

If we agree that correlated MN and SN UHI using a nested structure is transferred from MN to SN in Question 1, moderator would suggest including the UE History Information IE in the S-NODE ADDITION REQUEST and SGNB ADDITION REQUEST messages.

Furthermore, [9] believes that a new IE, e.g. PSCell change subscription, shall also be added in the SN addition request messages to indicate that MN subscribes to PSCell changes from SN.

Question 2-3: Do companies agree with the following proposals?

Proposal 2-8: UE History Information IE shall be included in the S-NODE ADDITION REQUEST and SGNB ADDITION REQUEST messages if RAN3 agrees that correlated MN and SN UHI using a nested structure is transferred from MN to SN.

Proposal 2-9: A new IE, e.g. PSCell change subscription, shall be also be added in the S-NODE ADDITION REQUEST and SGNB ADDITION REQUEST messages to indicate that MN subscribes to PSCell changes from SN.

| Company | Yes/No | Comment |
| --- | --- | --- |
| ZTE | Yes | Adding a new indicator for subscription is more straightforward, and it is not a big cost. |
| Nokia | No | In the ADD REQ, only the SCG UHI is needed at this stage. There is already existing mechanism related to subscribing to PSCell changes: the Location Information at S-NODE reporting IE is used to start it. Once the “subscription” starts, the MN can derive information like time stayed in the cell and the timestamp when the PSCell was configured for the UE. No need to duplicate this functionality. |
| Ericsson | Yes | Location Information reporting is a sensitive security IE and it is preferable to avoid mixing the 2 purposes, at least without SA3 agreement. This IE usage might need authorization and might be monitored because of security reasons. If the mechanisms are similar, it can be reused, but triggered by a new IE. |
| Qualcomm | P2-8: Depends on P2-1 P2-9: Alternative solution provided | Proposal 2-8: OK, but depends on Proposal 2-1 Proposal 2-9: Location Information Reporting could be reused, but this is a procedure for different purposes. Location Information Reporting is initiated by the AMF and enables NG-RAN to report the UE's current location, or the UE's last known location with time stamp, or the UE's presence in the area of interest, optionally along with the PSCell information. We think there are benefits to potentially define a new “subscription” procedure over Xn, which is initiated by NG-RAN (no AMF involvement) and can be used to inform MN of any intra-SN PSCell changes without MN involvement Infact, a similar topic is being discussed in CB # SONMDT4_SNChangeFailure where MN can potentially use a “class-1” message to check with SN if there is an intra-SN PSCell change without MN involvement. We think we could also reuse the above “class-1” message to also notify PSCell ID upon intra-SN PSCell change along with the indication. This can alleviate the concerns of some companies on too much Xn signaling impacts. |
| Lenovo and Motorola Mobility | Yes for Proposal 2-8 | For Proposal 2-9, further consider whether to reuse Location Information Reporting procedure or define a new “subscription” procedure. |
| CATT | Yes |  |
| Samsung | No | As we indicated for Proposal 2-1, only SN UHI is needed from MN to the SN. For the subscription procedure, the existing location reporting procedure is enough. Regarding the security concerns, using a new procedure doesn’t solve the problem. Anyway, the MN has get the UE location in the SN. Why the security issue has different with the procedure to be used. |
| Huwaei |  | Proposal 2-8: Yes Proposal 2-9: we can discuss this more in next meeting |

Moderator summary: For proposal 2-8, (6/8) companies say yes, (2/8) companies say no. For proposal 2-9, (3/8) companies say yes, (3/8) companies say no, (2/8) companies say FFS.

Moderator would suggest to agree with including UE History Information IE shall be included in the S-NODE ADDITION REQUEST and SGNB ADDITION REQUEST messages to follow majority’s view and keep align with proposal 3-1.

Since companies still have diverging views on proposal 2-9, moderator would suggest leaving this issue to the next meeting.

Proposal 3-3: UE History Information IE shall be included in the S-NODE ADDITION REQUEST and SGNB ADDITION REQUEST messages.

Open issue 2: FFS how MN subscribes to PSCell changes from SN.

### SN modification request message

To retrieve SN UHI, [2][10] believe that a new IE, e.g. UE history information query, shall be added in the SN modification request messages to indicate the retrieving of SN UHI. [9] believes that a new IE, e.g. PSCell change subscription, shall be added in the SN modification request messages to indicate that MN subscribes to PSCell changes from SN. Since the query and subscription function can not be used at the same time, moderator would suggest using a Choice IE or using a two-codepoint IE as follows.

Choice IE:

| CHOICE PSCell History Information Retrieve | O |  |  |  | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |
| >Query |  |  |  |  |  |  |
| >>PSCell History Information Query | M |  | ENUMERATED (true, …) |  | – |  |
| >Subscription |  |  |  |  |  |  |
| >>PSCell Change Subscription | M |  | ENUMERATED (start, ...) |  | – |  |

Two-codepoint IE:

| PSCell History Information Retrieve | O |  | ENUMERATED (query, subscription, ...) | Indicates how to retrieve SN UE history information. | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |

Question 2-4: Companies are kindly asked which option is preferred, and it is also encouraged to propose other options.

Option 1: Add a new Choice IE in the S-NODE MODIFICATION REQUEST and SGNB MODIFICATION REQUEST messages to indicate whether MN needs SN UHI or MN subscribes to PSCell changes.

Option 2: Add a new IE with two codepoints in the S-NODE MODIFICATION REQUEST and SGNB MODIFICATION REQUEST messages to indicate whether MN needs SN UHI or MN subscribes to PSCell changes.

| Company | Option | Comment |
| --- | --- | --- |
| ZTE | 1 or 2 | Both ways are fine, but we slightly prefer option 2 since it is simpler and easier to expand. |
| Nokia | Neither | For the subscription, we shall reuse the existing Location Information at S-NODE reporting IE. The request for one-time report on the complete SCG UHI shall rather be a separate IE, because the result is completely different. |
| Ericsson | 1bis | Prefer 2 IEs, but separated (not a choice). Because the 2 mechanisms are different. And the subscription IE will be similar to SN Addition message. Also do we need a stop codepoint for the subscription? |
| Qualcomm | Postpone | Stage-3 details can be discussed once we agree on introducing this “subscription” mechanism and after discussing whether we can define a common message for SN UHI retrieval in case of intra-SN PSCell changes and SCG failure information detection for pre-Rel-17 UEs. |
| Lenovo and Motorola Mobility | Postpone | Discuss the details later |
| CATT |  | We prefer to use separate IE since it seems the intention is different |
| Samsung | Neither | Agree with Nokia. |
| Huawei | Postpone | Prefer to discuss next meeting |

Moderator summary: Since companies still have diverging views on question 2-4, moderator would suggest leaving this issue to the next meeting. This part of discussion is related to open issue 2.

### SN modification required message

[2][6] believe that SN UHI, e.g. SN UE History Information IE, shall be included in the SN modification required messages if MN subscribes to PSCell changes. [4] believes that a new procedure shall be used. [9] believes that both ways can work. Moderator would suggest including SN UHI in the SN modification required messages since it has less specification impact.

Question 2-5: Do companies agree to include SN UHI, e.g. SN UE History Information IE, in the S-NODE MODIFICATION REQUIRED and SGNB MODIFICATION REQUIRED messages?

| Company | Yes/No | Comment |
| --- | --- | --- |
| ZTE | Yes | Prefer the way with less specification impact if both way works. |
| Nokia | Yes |  |
| Ericsson | Yes |  |
| Qualcomm | Yes |  |
| Lenovo and Motorola Mobility | Yes |  |
| CATT | Yes |  |
| Samsung | Yes |  |
| Huawei | Yes |  |

Moderator summary: All the companies agree to include SN UHI, e.g. SN UE History Information IE, in the SN modification required messages if MN subscribes to PSCell changes. Thus, moderator would suggest the following proposal.

Proposal 3-4: SN UHI, e.g. SCG UE History Information IE, shall be included in the S-NODE MODIFICATION REQUIRED and SGNB MODIFICATION REQUIRED if MN subscribes to PSCell changes.

### Other messages

Since we have agreed that correlated MN and SN UHI using a nested structure is transferred from MN to MN in phase 1 discussion, moderator would suggest including the UE History Information IE in the HANDOVER REQUEST message.

Moderator would also suggest including SN UHI, e.g. SN UE History Information IE, in the SN change required, SN modification request acknowledge, SN release required, and SN release request acknowledge messages if we agree that only SN UHI is sent from SN to MN in Question 1.

Question 2-6: Do companies agree with the following proposals?

Proposal 2-10: Include the UE History Information IE in the HANDOVER REQUEST message.

Proposal 2-11: Include SN UHI, e.g. SN UE History Information IE, in the following messages if RAN3 agrees that only SN UHI is sent from SN to MN.

- S-NODE CHANGE REQUIRED and SGNB CHANGE REQUIRED messages

- S-NODE MODIFICATION REQUEST ACKNOWLEDGE and SGNB MODIFICATION REQUEST ACKNOWLEDGE messages

- S-NODE RELEASE REQUEST ACKNOWLEDGE and SGNB RELEASE REQUEST ACKNOWLEDGE messages

- S-NODE RELEASE REQUIRED and SGNB RELEASE REQUIRED messages

| Company | Yes/No | Comment |
| --- | --- | --- |
| ZTE | Yes |  |
| Nokia | Yes mostly | P 2-10 is not needed, is it? The UE Hist Info is already included in the HO REQ… |
| Ericsson | Yes | Agree with Nokia that if we extend the existing IE no need to add anything in HO Request |
| Qualcomm | Yes | Does P 2-10 refer to SN UHI? MN UHI is already included as Nokia pointed out. |
| Lenovo and Motorola Mobility | Yes |  |
| CATT | Yes but | If only SN UHI is sent from SN to MN in Question 1 is agreed, we agree the above proposals. |
| Samsung | Yes | Proposal 2-10 should be reworded. The intention is to add PScell information to UE history information in Handover Request message. |
| Huawei | Yes | Agree with Samsung |

Moderator summary: The intention of proposal 2-10 is to indicate that both SN and MN UHI shall be included in the Handover Request message as one IE. Since the existing UE History Information IE is already included in the Handover Request message, proposal 2-10 is not needed.

Almost all the companies agree with proposal 2-11, thus moderator would suggest the following proposal to keep align with proposal 3-2.

Proposal 3-5: Include SN UHI, e.g. SCG UE History Information IE, in the following messages.

- S-NODE CHANGE REQUIRED and SGNB CHANGE REQUIRED messages

- S-NODE MODIFICATION REQUEST ACKNOWLEDGE and SGNB MODIFICATION REQUEST ACKNOWLEDGE messages

- S-NODE RELEASE REQUEST ACKNOWLEDGE and SGNB RELEASE REQUEST ACKNOWLEDGE messages

- S-NODE RELEASE REQUIRED and SGNB RELEASE REQUIRED messages

## Stage 2

Since introducing SN UHI will impact stage 2 specifications, it would be beneficial to add some description for SN UHI in TS 37.340 and TS 38.300.

- proposes to add the Notes in section 10.5.1, 10.5.2, 10.7.1, 10.7.2, 10.8.1, and 10.8.2 to indicate that MN wants to retrieve SN UHI. Thus, moderator would suggest using the following content for the notes since we have agreed with option 4 to retrieve SN UHI.

NOTE X: The MN may trigger the MN-initiated SN Modification procedure (to the source SN) to retrieve the SN UE history information before step 1. The MN may subscribe to PSCell changes if the MN-initiated SN Modification procedure is not triggered to retrieve the SN UE history information.

[5][11] also propose to add some specific description for SN UHI in 13.x in TS 37.340. Moderator has consulted the Rapporteur of TS 37.340, and Rapporteur thinks it is fine to put the specific description under 13.x. Thus, moderator would suggest adding the following contents in TS 37.340.

- 13.x SN UE history information

The MN collects and stores the UE History Information from MN and SN(s) as long as the UE stays in MR-DC, forwarding UE History Information and optional UE History Information from the UE to the connected SNs or target MN. The resulting information is then used by SN in subsequent handover preparation.

[5] proposes to add some description for SN UHI in section 15.5.4 in TS 38.300. Thus, moderator would suggest adding the following contents in TS 38.300.

In MR-DC, the SN can collect the SN UE History Information. Further details can be found in TS 37.340 [21].

Question 2-7: Do companies agree to add the above contents in TS 37.340 and TS 38.300?

| Company | Yes/No | Comment |
| --- | --- | --- |
| ZTE | Yes | It is beneficial to add some stage 2 descriptions for SN UHI. |
| Nokia | No | The “note X” is all right. But the two statements under “13.x” are contradictory, because they point to different nodes for the collecting of SCG UHI. This has to be resolved before agreeing TPs. Preferably, 38.300 indicates that the SN collects SCG UHI as long as the UE is connected with a PSCell of this SN. The MN is responsible to handle the SCG UHI when the SN changes. |
| Ericsson | Yes for stage-2 | Maybe 13.X needs more thinking. MN is in charge of UHI in general, with or without MR-DC. SN is in charge of collecting SN UHI and to keep the MN up to date (on request, at release/SN change or by subscription) |
| Qualcomm | Yes for stage-2 | Same view as Nokia and Ericsson |
| Lenovo and Motorola Mobility | Yes | Agree with Nokia |
| CATT |  | Maybe we could discuss the stage 2 after have a clear conclusion on the previous question. |
| Samsung |  | After the overall mechanism is clear, we can discuss the text for stage 2. |
| Huawei |  | Also prefer to wait with stage2 |

Moderator summary: Majority companies prefer to postpone the discussion on stage 2 description. Thus moderator would suggest leaving this issue to the next meeting.

Open issue 3: FFS on the stage 2 description for SN UHI.

## How to correlate MN and SN UHI

In the phase 1 discussion, some companies raised questions about how to correlate MN and SN UHI and which node performs the correlation. From moderator’s point of view, we have agreed to transfer correlated MN and SN UHI from MN to MN, and if we also agrees to transfer correlated MN and SN UHI from MN to SN, it shall be MN that correlates MN and SN UHI. How to correlate MN and SN UHI is up to implementation.

Question 2-8: Do companies agree with the following proposal?

Proposal 2-12: MN shall correlate MN and SN UHI, and how to correlate MN and SN UHI is up to implementation.

| Company | Yes/No | Comment |
| --- | --- | --- |
| ZTE | Yes | We think MN shall perform the correlation of MN and SN UHI instead of SN. If MN chooses to use option 2 to retrieve SN UHI, it can perform the correlation with the up-to-date SN UHI. If option 1 is used, MN may not know the up-to-date SN UHI. However, two ways can be used to perform the correlation without up-to-date SN UHI according to [5] and [10]. Furthermore, how to achieve the correlation is up to implementation, which can not be reflected in the specifications. |
| Nokia | Yes |  |
| Ericsson | Yes | Agree. However, we need to make sure that the MN has all the information it needs to perform the correlation (depends on results of question 2-2 (IE design) |
| Qualcomm | No | In the online session, we agreed “Correlated MN and SN UHI using a nested structure shall be included in the handover request message from MN.” Regarding Proposal 2-12, we agree that MN shall do the correlation, but it is NOT up to implementation and can be achieved via the nested structure of MN and SN UHI, |
| Lenovo and Motorola Mobility | Yes | For the case that MN transfers correlated MN and SN UHI to the peer node, obviously it is MN that correlates MN and SN UHI. |
| CATT |  | It depends on the results of question in 3.1.1 and we have a slight preference to let SN do the correlation. Nevertherless, it is also acceptable for us to let MN make the decision. |
| Samsung |  | It’s not purely implementation issue. Whether to have this in the specification is one thing. The important is that the node who performs correlation should have enough information. The right logic should be that we have common understanding on which node perform the correlation and this node has enough information, then to discuss whether nested structure is used. If MN chooses to use option 1 to retrieve SN UHI without option 2 (option 2 is not mandatory, that’s why option 1 can also be agreed), then the MN has no enough information. That’s why we think separate IE for MN UHI and SHI is better. This give flexibility for the MN. |
| Huawei |  | Agree that MN is responsible and that we need to make sure MN has enough information |

Moderator summary: Almost all the companies agree that MN shall correlate MN and SN UHI. Moderator would suggest the following proposal.

Proposal 3-5: MN shall correlate MN and SN UHI, FFS on how to correlate MN and SN UHI.

- For the Chairman’s Notes (Phase 1)

Based on the contributions, we have 5 candidate options as follows.

Option 1: MN initiates SN modification procedures to retrieve SN UHI before handover

Option 2: MN is always aware of the latest SN UHI by subscribing to PSCell changes

Option 3: Use new Xn message to transfer SN UHI after source SN release

Option 4: Hybrid option 1&2

Option 5: Hybrid option 1&3

According to the discussion, moderator proposes the following agreements:

Proposal 1: MN can initiate SN modification procedures to retrieve SN UHI before handover, MN can also subscribe to PSCell changes from SN. (Option 4)

Proposal 2: Correlated MN and SN UHI using a nested structure shall be included in the handover request message.

Note:The conclusion from MN to MN does not impact the IE design for MN to SN and vice versa.

- Phase 1 discussion

## How to retrieve SN UHI

In the last meeting, we have discussed seriously on the solutions to retrieve SN UHI. From moderator’s point of view, the detailed signaling design shall be discussed after we reach a consensus on the basic solution. The main controversial point is whether up-to-date SN UHI is needed and how to transfer up-to-date SN UHI. Based on the contributions, now we have 5 options as follows.

Option 1: MN initiates SN modification procedures to retrieve SN UHI before handover[3][10].

If SRB3 is allowed and delta configuration enabled, MN needs to trigger MN-initiated modification procedure before handover. Thus, no extra delay may be introduced to retrieve SN UHI using the same procedure. Some companies argue that if SRB3 is not allowed or delta config is not enabled, the latest SN UHI cannot be provided to target MN after source SN release using this option. However, it is noted that in the RAN2 #115 e-meeting, the following agreements were made. [10] believes that according to the agreements, UE can report the latest SN UHI to source MN if up-to-date SN UHI is need, and thus it is sufficient to use option 1 to retrieve SN UHI.

SN Related MHI Information: 2 RAN2 to confirm that the PSCell transition is part of MHI. 3 PSCell MHI is reported only to PCell. 4 UEInformationResponse message is used to convey the PSCell MHI to the MN. 5 Take Option 1 ‎(PSCell MHI nested within the PCell MHI) as baseline.

Option 2: MN is always aware of the latest SN UHI by subscribing to PSCell changes [9]

[9] believes that supporting MN subscription to PSCell changes has the advantages including: 1) help target MN to make informed decisions about DC and target SN to optimize PSCell selection, and 2) simplify CHO by ensuring up-to-date PSCell information at candidate NG-RAN nodes. However, this option may cause too much X2/Xn signalling, and the latest SN UHI is not always needed.

Option 3: Use new Xn message to transfer SN UHI after source SN release [16]

[16] proposes to transfer SN UHI via a new Xn message after handover preparation procedure, which is simple and clean. However, the main drawback of this option is latency which may delay the Ping pong decision made by target NG-RAN nodes.

Option 4: Hybrid option 1&2 [2][4][6]

[2][4][6] proposes a hybrid option combing option 1 and option 2. If SRB3 is allowed and delta config enabled, SN UHI can be retrieved by the MN-initiated modification procedure. If SRB3 is not allowed or delta config is not enabled, SN UHI shall be included in the SN Modification Required message, so that it can be updated in the MN when the MN requests PSCell change notifications. This solution brings more Xn signalling in exchange for the decrease of handover delay.

Option 5: Hybrid option 1&3 [5]

[5] proposes to use option 1 as baseline, and also use a new Xn message to transfer latest UHI to target NG-RAN nodes. The drawback of this option is the same as option 3, e.g. latency.

Question 1: Companies are kindly asked which option above is preferred to retrieve SN UHI.

| Company | Option | Comment |
| --- | --- | --- |
| ZTE | Option 1, 4 | Option 2 may introduce unnecessary signalling, option 3 and 5 have more specification impacts compared with the other options. We think option 1 is sufficient, but we can accept option 4 as a compromise. |
| Lenovo and Motorola Mobility | Option 1 | For Option 2, it may cause much signalling and resource to transfer latest SN UHI from SN to MN if PSCell changes frequently. For Option 3, new Xn message(s) are introduced to transfer SN UHI after source SN release, which has more spec impact. |
| CATT | Option 4 | Option 2 can be used as a supplementation by supporting MN subscription to PSCell changes. |
| Nokia | 1, possibly 2 | Option 1 is the primary solution that shall work 99% of cases. However, existing signalling enables the MN to be updated concerning PSCell changes, so, epending on the implementation, this mechanism can be reused. |
| Qualcomm | Option 1, 4 | Option 1 is the baseline. In addition to Option 1, we are also OK to support Option 2 i.e., MN to be aware of intra-SN PSCell changes without MN involvement by subscribing to PSCell Changes. SN can then provide a PSCell Change notification to MN via SN Modification Required or a new class-2 message. Maybe we can restrict this to only intra-SN PSCell changes without MN involvement to limit Xn signaling overhead. |
| Huawei | 1 or 5 | We think the usage of SN UHI does not need the real time SN UHI. - For the ping pong issue, in our understanding, the ping pong detection is based on the statistics over a long time. Therefore we think the target node does not need the real time SN UHI, i.e. the target node can get the SN UHI after handover. For the assisting target MN in selecting the appropriate SN and in determining whether DC needs to be supported, we think the motivation is to avoid the unnecessary handover. In our understanding, it does not Therefore Option 1 should be the baseline. In case we need statistics also from cases with full reconfiguration, we prefer to combine 1 with 3 which is option 5. We would also like to highlight that 2 and 3 should not be used standalone but always combined with 1. There is no point in using these if the information is already available at the right time. |
| China Telecom | Option 1/4 | We think option1 is the baseline, and we can accept option4 as the supplementary solution if option1 cannot be used, other solutions have more impact on the specification and may cause extra delay. |
| Samsung | 5 | The current description on 1 may bring confusion. At last RAN3#113-e meeting it was agreed that: RAN3 should consider solutions which would not delay HO more than it would have been delayed without UHI. So we suggest to update Option 1 to “In case SN modification procedure is needed without UHI e.g. for delta configuration, the source MN retrieve SN UHI from the SN via SN modification procedure. With this change, we are fine to take Option 1 as baseline. For Option 2, we agree with the analysis from the moderator, too much X2/Xn signaling. Option 3 can be used in case the MN receives the updated SN UHI after source SN release. This is also in line with what we have agreed at last RAN3#114-e meeting: Option 4 has similar issue as Option 2. Based on above, we are fine for Option 5. |
| Ericsson | 4 | Option 1 should be the baseline when the MN has to fetch the SCG configuration anyway (e.g. SRB3). Option 2 will therefore be optional. Implementation that does not want to support the additional signaling load brought by solution 2 can always use option 1. Option 1 alone does not solve the HO delay issue, which has been acknowledged For option 3 and 5, updating the SN UHI in the target MN after HO REQUEST and in the target SN after SN ADDITION will bring even more complexity than option 2, as explained in [9] |
| CMCC | 1 or 4 | 1 is thebaseline |

Moderator summary: All the company agree to use option 1 as baseline. (7/10) companies supports using option 2 as the supplementary solution which is option 4, (2/10) companies supports using option 3 as the supplementary solution which is option 5.

Question 2: Companies are kindly asked to indicate your least favorite option.

| Company | Option | Comment |
| --- | --- | --- |
| ZTE | Option 2, 3, 5 | Same comment to Q1 |
| Lenovo and Motorola Mobility | Option 2/3/5 |  |
| CATT | Option 3,5 | It is too complex for target MN to update UHI when receiving the new updated message. |
| Nokia | 3 (and 5) | Post-release and after HO signalling complicates the solution beyond reasonable limits. Option 2 is acceptable only if the existing signalling is reused to a great extend. |
| Qualcomm | Option 3 and 5 | Similar view as CATT and Nokia. Also, this will mean latest SN UHI might not be available at target MN right after HO. If we are targeting that use case, we might as well come up with a complete solution (via a subscribe mechanism) |
| Huawei | 2,3 4 | (2,3) There is no point in using these if the information is already available at the right time. So we should pick a hybrid solution (4) This will increase signaling if we need to notify at every cell change |
| China Telecom | Option 2/3/5 |  |
| Samsung | Option 2, 4 | The ping pong detection is based on the statistics over a long time. Therefore the target node does not need the real time SN UHI. 2 and 4 bring too much signaling. Usually SN is used for NW capacity issue, so SN could consist of small cells with high frequency. It could be very often that Intra-SN PSCell change occurs. |
| Ericsson | 1 alone 3 alone | 1 alone will delay HO, which contradict our agreement 3 alone may always lead to wrong DC decisions, and brings too much complexity on the target nodes |

Moderator summary: (8/9) companies choose option 3, (6/9) companies choose option 5, (5/9) companies choose option 2, (2/9) companies choose option 4, (1/9) companies choose option 1.

According to the comments, most companies support option 4, which uses option 1 as baseline and uses option 2 as supplementary. However, there are still two companies that do not support option 4. The main concern is that using option 2 will bring too much signalling. From moderator’s point of view, option 1 and option 2 have overlapping in signalling which indicates that the signalling design of option 4 is simpler than that of option 5. Furthermore, if option 4 is used, a smart MN can choose whether to use option 1 or option 2 depending on the actual situation. If MN does not want to bring additional signalling overhead, MN can choose to use option 1. Option 5 has more specification impacts compared with option 4. The main drawback of option 5 is 1) latency which may delay the Ping pong decision made by target NG-RAN nodes, and 2) too complex for target MN to update UHI when receiving the new updated message. Thus, moderator would suggest to adopt option 4 to retrieve SN UHI.

Proposal 1: MN can initiate SN modification procedures to retrieve SN UHI before handover, MN can also subscribe to PSCell changes from SN.

## Correlation of MN and SN UHI

We have already agreed to include both MN and SN UHI in the handover request message during inter-MN handover. The remaining issue is whether to transfer correlated MN UHI and SN UHI or separate MN UHI and SN UHI. It seems to be majority view that correlated MN UHI and SN UHI shall be achieved by using a two-dimension structure, where PSCell information is listed within the related PCell information.

MN and SN UHI shall be included in inter-MN handover message i.e. Handover Request message. It is FFS whether MN UHI and SN UHI will be separated Ies or a list of MN UHI containing a list of SN UHI.

According to the contributions, two options are proposed as follows. It is noted in the last meeting, RAN2 has agreed that the PSCell MHI is nested within the PCell MHI in the UE history from the UE. From moderator’s view, it could be better to choose option 1 to keep align with RAN2. Furthermore, additional information needs to be transferred to help target MN correlate the information if option 2 is used.

Option 1: Correlated MN and SN UHI using a nested structure shall be included in the handover request message.

Option 2: Separate MN and SN UHI shall be included in the handover request message. Target MN will correlate these two information.

Question 3: Companies are kindly asked which option above is preferred.

| Company | Option | Comment |
| --- | --- | --- |
| ZTE | Option 1 | Option 1 is simpler since correlation only needs to be performed once, and no additional information is required. |
| Lenovo and Motorola Mobility | Option 1 | It is better for source MN to make the correlation for MN UHI and SN UHI. |
| CATT | Option 1 | We already have a WA to use a two-dimensional structure which is captured in the chairman notes: WA: Correlation of MN UHI and SN UHI could be realized via two-dimensional structure for UHI (PSCells history information are listed within each Pcell in the UHI); it may not be feasible on all interfaces. Besides, we should also align with RAN2 |
| Nokia | Option 2 | SCG UHI will have to be delivered from the MN to the SN and vice versa. So far there is no reason to have them combined (why the SN would send outdated MCG UHI back to the MN???). So, having such “combined” or “embedded” list just for the HO purpose does not seem reasonable. There will also be the question how that coexists with the legacy UHI… So, adding a new SCG UHI IE (with all the information needed to correlate it with the MCG UHI!) and keeping the existing MCG UHI as is seems the best compromise. No idea what are we supposed to “align with RAN2”, if RAN2 does not discuss HO signalling (as far as I know)? The UE history is a totally different story. |
| Qualcomm | See comments | Regarding Nokia’s comment that “SN might end up sending an outdated MCG UHI back to the MN”, can you clarify why/when does this happen? In my understanding, source MN can just send the correlated list of MN and SN UHI which it is maintaining to the target MN in HANDOVER REQUEST. No need of using any outdated MN UHI from the SN, right? Or is there any other scenario you are referring to where SN will have the outdated MN UHI? Also, I think this comes down to the fundamental question: Whether MN UHI should be forwarded from MN to SN and vice versa? But we do understand that the IE structure can be simpler to coexist with legacy UHI if we use an independent list, but nested list just makes correlation easier at the cost of signaling overhead. We are OK to go with an independent list as well if this can help make some progress. |
| Huawei | Option 1 | Agree the comments from ZTE |
| China Telecom | Option 1 | Agree with ZTE. |
| Samsung | Option 2 | Agree with Nokia. Besides that, we still doubt how the correlation is done and whether it is in the MN or in the SN. - If the correlation is done in the SN, currently the PCell ID in the message from MN to the SN is optional. If the MN doesn’t send PCell ID to the SN. This means the SN cannot get the relation of PScell and PCell. If we mandate the MN to send the PCell ID to the SN, this is a non-backward compatible change. - If the correlation is done in the MN, the MN may not know the up-to-date PScell as well in case of intra-SN PScell change via SRB3. |
| Ericsson | Option 1 | Correlation will be needed at some point, for source and target MNs to understand mobility patterns. It is therefore easier to do it at the node which has all the needed information i.e. source MN. WA for option 1 can be taken, and how correlation is done can be further discussed |
| CMCC | Option 1 | Same view as ZTE and CATT |

Moderator summary: (7/10) companies choose option 1, (2/10) companies choose option 2, (1/10) company chooses neutral.

According to the comments, majority companies support option 1. However, there are still two companies that support option 2. Moderator would like to clarify that we are discussing how to transfer MN and SN UHI from source MN and target MN which seems to be the easiest point that we may reach a consensus, while whether SN and/or MN UHI shall be transferred from MN to SN and vice versa is still FFS. Since we have agreed to add both SN and MN UHI in the handover request, the correlation of MN and SN UHI has to be performed by source MN and/or target MN. If we choose option 1, correlation only needs to be performed once, and no additional information is required for target MN to perform correlation. Thus, moderator would suggest to adopt option 1.

Proposal 2: Correlated MN and SN UHI using a nested structure shall be included in the handover request message.

Note:The conclusion from MN to MN does not impact the IE design for MN to SN and vice versa.

- Phase 2 discussion

Based on outcome of phase 1 discussion and on-line discussion, the following open issues may be discussed in the phase 2 discussion.

- Correlated MN and SN UHI from MN to SN is benefit?

- Messages for UHI transfer

- SN UHI information content

- Interface impact of SN UHI from UE

- Stage 2/3 update

- Conclusion, Recommendations [if needed]

If needed

- References

- R3-214469	Summary of Offline Discussion on CB: # SONMDT3_UEHistoryInfor

- R3-214811	[SON BL CRs, TP to TS 38.423, TP to TS 36.423]  HO delay and non-up-to-date SCG UHI information (Nokia, Nokia Shanghai Bell)	other

- R3-214840	Consideration on UE history information (China Telecommunication)	discussion

- R3-214915	UE History Information in MR-DC (Qualcomm Incorporated)	discussion

- R3-214956	(TP for SON BLCR for 38.423,38.413, 36.423, 36.413 , 37.340, 38.300) UE History Information in MR-DC (Huawei)	other

- R3-215068	Enhancement of UE history information in MR-DC scenario (CATT,CMCC)	discussion

- R3-215069	(TP on UE history information for 36.413) Addition of UE history information for SN (CATT,CMCC)	other

- R3-215070	(TP on UE history information for 36.423) Addition of UE history information for SN (CATT,CMCC)	other

- R3-215444	(TP for SON BL CR for TS 38.423, TS 38.413, TS 36.413): UE History Information for Secondary Node (Ericsson)	other

- R3-215456	UE History Information in MR-DC (ZTE, Lenovo, Motorola Mobility, China Unicom)	discussion

- R3-215457	(TP for SON BL CR for TS 37.340) Introduce UHI in MR-DC (ZTE, Lenovo, Motorola Mobility, China Unicom)	other

- R3-215458	(TP for SON BL CR for TS 38.423) Introduce UHI in MR-DC (ZTE, Lenovo, Motorola Mobility, China Unicom)	other

- R3-215459	(TP for SON BL CR for TS 36.423) Introduce UHI in MR-DC (ZTE, Lenovo, Motorola Mobility, China Unicom)	other

- R3-215460	(TP for SON BL CR for TS 38.413) Introduce UHI in MR-DC (ZTE, Lenovo, Motorola Mobility, China Unicom)	other

- R3-215461	(TP for SON BL CR for TS 36.413) Introduce UHI in MR-DC (ZTE, Lenovo, Motorola Mobility, China Unicom)	other

- R3-215519	UE History Information in MR-DC (Samsung)	discussion

- R3-215520	TP for SON BLCR for 38.423: UE History Information in MR-DC (Samsung)	other

- R3-215521	TP for SON BLCR for 36.423: UE History Information in EN-DC (Samsung)	other

- R3-215695	(TP to TS 38.413)UE history information in MR-DC (CMCC, CATT)	other

- R3-215696	(TP to TS 38.423)UE history information in MR-DC (CMCC, CATT)	other