3GPP TSG-RAN WG3 #114bis-e							                                             R3-22xxxx

17-26 Jan 2022

Online

Title: 	(TP for NR_IAB BL CR for TS 38.473):  F1-C transfer path configuration

Source: 	ZTE, CATT, Qualcomm, Samsung

Agenda item:	13.2.3

Document for:	Other

# Introduction

In RAN2#116-e meeting, it was agreed that

The configuration of F1-C traffic on the indication of the the leg(s) used for transferring the F1-C traffic is configured to IAB-MT by a new field , e.g., f1c-TransferPath-r17  ENUMERATED {MCG, SCG, both}.

According to the agreement, donor-CU configures the F1-C transfer path, which is indicated by f1c-TransferPath-r17,  to IAB-MT via RRC message. Besides, there was a post email discussion in RAN 2 to discuss the 38.331 running CR. Within the agreed CR R2-2111604 [1], the f1c-TransferPath-r17 IE is included in the CellGroupConfig IE. The details are shown below.

– CellGroupConfig The CellGroupConfig IE is used to configure a master cell group (MCG) or secondary cell group (SCG). A cell group comprises of one MAC entity, a set of logical channels with associated RLC entities and of a primary cell (SpCell) and one or more secondary cells (SCells). CellGroupConfig information element -- ASN1START -- TAG-CELLGROUPCONFIG-START -- Configuration of one Cell-Group: CellGroupConfig ::= SEQUENCE { ......<non-related part omitted>...... bap-Address-r16 BIT STRING (SIZE (10)) OPTIONAL, -- Need M bh-RLC-ChannelToAddModList-r16 SEQUENCE (SIZE(1..maxBH-RLC-ChannelID-r16)) OF BH-RLC-ChannelConfig-r16 OPTIONAL, -- Need N bh-RLC-ChannelToReleaseList-r16 SEQUENCE (SIZE(1..maxBH-RLC-ChannelID-r16)) OF BH-RLC-ChannelID-r16 OPTIONAL, -- Need N f1c-TransferPath-r16 ENUMERATED {lte, nr, both} OPTIONAL, -- Need M simultaneousTCI-UpdateList1-r16 SEQUENCE (SIZE (1..maxNrofServingCellsTCI-r16)) OF ServCellIndex OPTIONAL, -- Need R simultaneousTCI-UpdateList2-r16 SEQUENCE (SIZE (1..maxNrofServingCellsTCI-r16)) OF ServCellIndex OPTIONAL, -- Need R simultaneousSpatial-UpdatedList1-r16 SEQUENCE (SIZE (1..maxNrofServingCellsTCI-r16)) OF ServCellIndex OPTIONAL, -- Need R simultaneousSpatial-UpdatedList2-r16 SEQUENCE (SIZE (1..maxNrofServingCellsTCI-r16)) OF ServCellIndex OPTIONAL, -- Need R uplinkTxSwitchingOption-r16 ENUMERATED {switchedUL, dualUL} OPTIONAL, -- Need R uplinkTxSwitchingPowerBoosting-r16 ENUMERATED {enabled} OPTIONAL -- Need R ]], [[ reportUplinkTxDirectCurrentTwoCarrier-r16 ENUMERATED {true} OPTIONAL -- Need N ]], [[ f1c-TransferPathNRDC-r17 ENUMERATED {mcg, scg, both} OPTIONAL -- Need M ]] }

From the CR, it seems that parent IAB-DU determines the F1-C transfer path because the CellGroupConfig IE is generated by the parent IAB-DU. As we know, R16 IAB considered who decided the F1-C transfer path in EN-DC case. It was agreed that donor-CU determined the F1-C transfer path and sent such configuration to parent IAB-DU. After receiving the configuration, parent IAB-DU included the F1-C transfer path configuration into the CellGroupConfig IE. Similarly, donor-CU should decide the F1-C transfer path in CP/UP separation scenarios, as it has the whole picture of the topology. Furthermore, to our understanding, it is also donor-CU to determine the F1-C transfer path according to RAN2’s intention. So, we propose that donor-CU needs to send the F1-C transfer path configuration to parent IAB-DU in CP/UP separation scenarios. A TP for TS 38.473 is provided in the Annex.

Proposal 1: Donor-CU needs to send the F1-C transfer path configuration to parent IAB-DU in CP/UP separation scenarios.

Proposal 2: It is suggested to agree the TP in Annex.

# Conclusion

In this contribution, we discuss RAN3 enhancement to support CP/UP separation scenarios, and have the following proposals:

Proposal 1: Donor-CU needs to send the F1-C transfer path configuration to parent IAB-DU in CP/UP separation scenarios.

Proposal 2: It is suggested to agree the TP in Annex.

# Reference

- R2-2111604, Running CR to 38.331 on NR IAB enhancements, Ericsson.

# Annex   TP for IAB BL CR for TS 38.473

-------------------------------------------Change 1-------------------------------------------

### 8.3.1	UE Context Setup

#### 8.3.1.1	General

The purpose of the UE Context Setup procedure is to establish the UE Context including, among others, SRB,DRB, BH RLC channel, and SL DRB configuration. The procedure uses UE-associated signalling.

#### 8.3.1.2	Successful Operation

Figure 8.3.1.2-1: UE Context Setup Request procedure: Successful Operation

The gNB-CU initiates the procedure by sending UE CONTEXT SETUP REQUEST message to the gNB-DU. If the gNB-DU succeeds to establish the UE context, it replies to the gNB-CU with UE CONTEXT SETUP RESPONSE. If no UE-associated logical F1-connection exists, the UE-associated logical F1-connection shall be established as part of the procedure.

<unrelated part is omitted>

If the F1-C Transfer Path IE is included in UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, take it into account.

If the F1-C Transfer Path NRDC IE is included in UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, take it into account.

-------------------------------------------Next Change-------------------------------------------

### 8.3.4	UE Context Modification (gNB-CU initiated)

#### 8.3.4.1	General

The purpose of the UE Context Modification procedure is to modify the established UE Context, e.g., establishing, modifying and releasing radio resources or sidelink resources. This procedure is also used to command the gNB-DU to stop data transmission for the UE for mobility (see TS 38.401 [4]). The procedure uses UE-associated signalling.

#### 8.3.4.2	Successful Operation

Figure 8.3.4.2-1: UE Context Modification procedure. Successful operation

The UE CONTEXT MODIFICATION REQUEST message is initiated by the gNB-CU.

<unrelated part is omitted>

If the F1-C Transfer Path IE is included in UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, take it into account.

If the F1-C Transfer Path NRDC IE is included in UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, take it into account.

-------------------------------------------Next Change-------------------------------------------

#### 9.2.2.1	UE CONTEXT SETUP REQUEST

This message is sent by the gNB-CU to request the setup of a UE context.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| <unrelated part is omitted> |  |  |  |  |  |  |
| F1-C Transfer Path | O |  | 9.3.1.207 |  | YES | reject |
| F1-C Transfer Path NRDC | O |  | 9.3.1.x |  | YES | reject |

-------------------------------------------Next Change-------------------------------------------

#### 9.2.2.7	UE CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU to provide UE Context information changes to the gNB-DU.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| <unrelated part is omitted> |  |  |  |  |  |  |
| SCG Indicator | O |  | ENUMERATED(released,...) | This IE is used at the MN in NR-DC and NE-DC and it indicates the release of an SCG | YES | ignore |
| F1-C Transfer Path NRDC | O |  | 9.3.1.x |  | YES | reject |

-------------------------------------------Next Change-------------------------------------------

#### 9.3.1.x	F1-C Transfer Path NRDC

This IE indicates the transmission path of the F1-C traffic in NR-DC.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| F1-C Path NRDC | M |  | ENUMERATED (mcg, scg, both) | This IE indicates the transmission path of the F1-C traffic in NR-DC. |

-------------------------------------------Next Change-------------------------------------------

### 9.4.4	PDU Definitions

<unrelated part is omitted>

F1CTransferPath,

SCGIndicator,

SpatialRelationPerSRSResource,

F1CTransferPathNRDC

<unrelated part is omitted>

id-F1CTransferPath,

id-SCGIndicator,

id-SRSSpatialRelationPerSRSResource,

id-F1CTransferPathNRDC,

<unrelated part is omitted>

-- **************************************************************

--

- -- UE Context Setup ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

- -- UE CONTEXT SETUP REQUEST

--

-- **************************************************************

UEContextSetupRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { UEContextSetupRequestIEs} },

...

}

UEContextSetupRequestIEs F1AP-PROTOCOL-IES ::= {

<unrelated part is omitted>

{ ID id-F1CTransferPath							CRITICALITY reject	TYPE F1CTransferPath									PRESENCE optional }|

{ ID id-F1CTransferPathNRDC						CRITICALITY reject	TYPE F1CTransferPathNRDC								PRESENCE optional },

...

}

<unrelated part is omitted>

-- **************************************************************

--

- -- UE Context Modification ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

- -- UE CONTEXT MODIFICATION REQUEST

--

-- **************************************************************

UEContextModificationRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { UEContextModificationRequestIEs} },

...

}

UEContextModificationRequestIEs F1AP-PROTOCOL-IES ::= {

<unrelated part is omitted>

{ ID id-F1CTransferPath							CRITICALITY reject	TYPE F1CTransferPath									PRESENCE optional }|

{ ID id-SCGIndicator							CRITICALITY ignore	TYPE SCGIndicator										PRESENCE optional	}|

{ ID id-F1CTransferPathNRDC						CRITICALITY reject	TYPE F1CTransferPathNRDC								PRESENCE optional },

...

}

-------------------------------------------Next Change-------------------------------------------

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

<unrelated part is omitted>

- -- F

<unrelated part is omitted>

F1CPathNRDC ::= ENUMERATED {mcg, scg, both}

F1CTransferPathNRDC ::= SEQUENCE {

f1CPathNRDC						F1CPathNRDC,

iE-Extensions					ProtocolExtensionContainer { { F1CTransferPathNRDC-ExtIEs} } OPTIONAL,

...

}

F1CTransferPathNRDC-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

-------------------------------------------Next Change-------------------------------------------

### 9.4.7	Constant Definitions

<unrelated part is omitted>

id-SCGIndicator										ProtocolIE-ID ::= 432

id-EstimatedArrivalProbability						ProtocolIE-ID ::= 433

id-TRPType											ProtocolIE-ID ::= 434

id-SRSSpatialRelationPerSRSResource					ProtocolIE-ID ::= 435

id-F1CTransferPathNRDC								ProtocolIE-ID ::= xxx

END

<<<<<<<<<<<<<<<<<<<<<<<<<<<< End of the TP >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>